Methods

The second part of the handbook explains the basic concepts, inputs and methods used to develop the models presented in Part III. The goal is to provide all information required for the development of new models and for updating existing ones.

Intended audience:

Users who want to understand the modelling background, and assess the assumptions and limitations of the models presented in this handbook.

Users with a need to develop new models in consistency with the model development philosophy of this handbook, in order to close gaps in Part III.

Users who wish to update the presented models with new data, e.g. from reliability testing or in-orbit feedback.

Overview on Part II:

Chapter 6 describes the foundations for model development: Basic mathematical concepts, data sources and inputs, as well as methods and modelling approaches.
  Section 6.7 explains how to use these foundations for the development of new models

Chapter 7 introduces Bayesian methods for model development and updating
  The general process for applying these methods is outlined in Section 7.2.

Chapter 8 discusses various uncertainties associated with reliability models and their usage
  Section 8.3 explains how to identify relevant uncertainties associated with a prediction.

Part II Table of Contents

6	Model development basic concepts	II-7

6.1	Basic mathematical concepts and definitions	II-7

6.1.1	Reliability metrics related to time	II-7

6.1.2	Reliability metrics related to stress or demand	II-9

6.2	Data sources and inputs	II-10

6.2.1	In-orbit return	II-10

6.2.2	Manufacturer data	II-11

6.2.3	Test results	II-12

6.2.4	Handbook data sources	II-13

6.2.5	Failure mechanisms analysis	II-13

6.2.6	Failure mode repartition data	II-13

6.3	Methods and modelling approaches	II-15

6.4	Statistical approach and classical reliability methods	II-18

6.4.1	Sampling characterisation	II-18

6.4.2	Choice of distribution function	II-18

6.4.2.1	The Exponential distribution for constant failure rates	II-20

6.4.2.2	The Weibull distribution for non-constant failure rates	II-21

6.4.2.3	Models for failures of standby components and one-shot items	II-21

6.4.3	Parameter estimation	II-22

6.4.3.1	Failure rate estimation for the Exponential model	II-24

6.4.3.2	Parameter estimation for the Weibull distribution and other failure time distributions	II-25

6.4.3.3	Parameter estimation for the Binomial distribution	II-25

6.5	Physics of failure and structural reliability methods	II-27

6.5.1	Failure mechanisms identification and modelling	II-28

6.5.2	Quantification of variable and model uncertainties	II-28

6.5.3	Uncertainty propagation and reliability estimation	II-30

6.5.4	Overview of structural reliability methods	II-30

6.5.4.1	Analytic methods strengths and limitations	II-32

6.5.4.2	FORM / SORM methods strengths and limitations	II-33

6.5.4.3	Monte Carlo methods strengths and limitations	II-34

6.6	Combined approach	II-36

6.7	Process for the development of new models	II-37

6.7.1	General process for all technical domains	II-37

6.7.2	Development of new models by technical domain	II-38

7	Bayesian methods for reliability prediction modelling	II-41

7.1	The Bayes rule and its application for estimation and model updating	II-41

7.1.1	Bayesian methods for model updating	II-42

7.1.1	Bayes rule as estimation method	II-43

7.2	Applying Bayesian methods in practice	II-44

7.3	Likelihood formulation	II-45

7.3.1	Distributional model for the sampling distribution	II-45

7.3.2	Censoring and truncation	II-45

7.4	Guidance on the prior selection	II-48

7.4.1	Informative prior definition	II-49

7.4.2	Non-informative prior selection	II-49

7.5	Methods for Bayesian updating with new data	II-51

7.5.1	Analytical approach with conjugate prior	II-51

7.5.2	Numerical approach using Markov Chain Monte Carlo Methods	II-57

7.6	Practical guidance for Bayesian updating in specific applications	II-60

7.6.1	Updating of estimates from previous statistical analysis	II-60

7.6.2	Updating of reliability estimates derived from reliability handbooks	II-60

7.6.3	Prior definition based on expert elicitation	II-61

7.6.4	Bayesian updating with Physics of Failure methods	II-62

8	Uncertainties in the prediction results	II-65

8.1	Uncertainties of reliability estimates based on Part III models	II-65

8.2	Classification and relevance of different prediction uncertainties	II-66

8.2.1	Epistemic uncertainties classification	II-66

8.2.2	The role of statistical uncertainties in system level reliability prediction	II-67

8.2.3	A more general view on epistemic uncertainties	II-68

8.3	Identification of relevant uncertainties associated with a prediction	II-70

8.3.1	Model development uncertainties	II-70

8.3.2	Model usage uncertainties	II-72

II.A	Probability distributions	II-75

II.B	Prediction uncertainties for Part III models	II-77

II.C	Part II Bibliography	II-85

# Model development basic concepts

Katharina Fischer, Matrisk GmbH

Goal of this chapter: To introduce the basic mathematical concepts of reliability modelling, the available data sources and methods, and their application for the development of new reliability prediction models.

## Basic mathematical concepts and definitions

In the following, the most important metrics used in the context of reliability prediction are briefly defined, using a generic notation. It should be noted that the term “failure” in the most general sense can be understood to represent any (adverse) event that is of interest for the reliability modelling.

### Reliability metrics related to time

Modelling reliability as a function of time requires a quantification of the probability distribution for the random time to failure T. In the following, no specific assumption is made regarding the form of this distribution, which may take any parametric or non-parametric form. Definitions for the well-known Exponential model (constant failure rate assumption) and the Weibull distribution are given in Section 6.4.2.

Reliability R(t)

The probability that the system or component does not fail and performs its intended function at the time t

| RTt=PT>t=1-0tfTtdt=1-FTt=exp-0tλxdx | Equation 6-1 |
| --- | --- |

The reliability function  Rt=RTt is thus defined as the probability of survival until time t.

Equation 6-1 also defines the relation to the probability of failure FTt, the failure density fTt, and to the failure rate λt, which are defined hereafter.

Failure Probability F(t)

The probability that a failure occurs and the system or component cannot perform its intended function anymore at the time t.

| FTt=PT≤t=0tfTtdt=1-RTt=1-exp-0tλxdx | Equation 6-2 |
| --- | --- |

In statistical terminology, the probability of failure during a time interval t is equivalent to the cumulative distribution function of the failure time distribution. Note that the subscript T, referring to the random time to failure, can be dropped throughout the handbook for the ease of notation, i.e. Ft=FTt.

Following Boolean logic, each system and its components can have two possible states, either operating error-free with the intended function or failing to do this. Therefore, there is a mathematical relation between the probability of failure (unreliability) Ft and reliability Rt:

| Rt+Ft=1 .0 | Equation 6-3 |
| --- | --- |

Failure density fT(t)

The first derivative of the failure probability at time t.

| fTt=dFTtdt | Equation 6-4 |
| --- | --- |

The failure density corresponds to the probability density function of the failure time distribution.

Failure rate λ(t)

The failure rate λt describes the probability that a failure occurs in a small time interval between t and t+Δt, assuming that the system or component was operating error-free at the time t.

| λt=limΔt→0FTt+Δt-FT(t)Δt*1RT(t)=fT(t)RT(t) | Equation 6-5 |
| --- | --- |

Considering the limit Δt→0, the failure rate λt is sometimes also referred to as “hazard rate”.

Mean Time To Failure MTTF

The average time until failure occurs, defined as the expected value of the failure time distribution:

| MTTF=ET=0∞t∙fTt dt=0∞RTt dt | Equation 6-6 |
| --- | --- |

The MTTF equivalent for repairable systems is the Mean Time Between Failure (MTBF). In practice, both terms are often defined as the inverse of the failure rate, i.e. MTTF=1/λ, assuming a constant failure rate λ during the steady-state phase of the bath-tub curve. It must not be confused with the useful life of a component, which determines the length of this steady-state phase.

### Reliability metrics related to stress or demand

Reliability as a function of time is certainly the most common notion in reliability modelling. It is appropriate for items whose failure process can be best described by a time-to-failure distribution. Stresses due to the environment and operating conditions can be considered as stress contributors in the modelling.

A simple generalization of the metrics defined in Section 6.1.1 above can be made for items whose reliability depends mainly on use, e.g. on the number of cycles. In this case, the time-to-failure distribution is simply replaced by a cycles-to-failure distribution, and the same definitions apply. With the aid of a use profile, the reliability modelling can be transformed to the time domain. The same applies for failures due to environmental stresses (e.g. radiation), whose occurrence or accumulation can be modelled as a function of time.

A special case of use-related failures are failures of items which are actuated only once (i.e. one-shot items) or a few times during the mission. In this case, a modelling based on the probability of failure on demand is better suited for reliability modelling. Note that a “demand” in this context must not necessarily be use-related, but can also represent an environmental stress acting at a discrete point in time (e.g. shock due to pyro activation).

Probability of failure on demand

The probability of failure on demand is the probability that a failure occurs, conditional on a specific event that takes place at a discrete (possibly random) point in time.

## Data sources and inputs

Different kinds of inputs, as defined in the following subsections, can be used for the development or updating of reliability prediction models.

The lists given hereafter provide the order of preference for the use of the different inputs, which was used as a guidance for the development of this handbook. For the preference ordering, it is assumed that a given data source is readily available, and in sufficient quantity to be used as a standalone input. For practical applications it needs to be emphasized that the different inputs must not be seen as mutually exclusive, and combinations of data sources can be considered as appropriate.

Irrespective of the preference ordering, a specific input should only be used under the conditions specified in the following subsections.

For EEE reliability prediction, the following preference ordering applies:

- IOR data

- Manufacturer data

- Test data

- Handbook data

- Failure mechanism analysis

The preference ordering changes in case of doubts regarding the applicability of existing handbooks for space application, e.g. in the context of mechanical reliability prediction:

- IOR data

- Manufacturer data

- Test data

- Failure mechanism analysis

- Handbook data

Which data has been used for the development of this handbook is specified at the beginning of each chapter in Part III.

The models provided in Part III generally consider all potential failure modes that are relevant for the overall reliability figure. A brief discussion of failure mode repartition data is provided in Section 6.2.6.

### In-orbit return

Satellite operators, prime contractors and lower level manufacturers can obtain IOR at spacecraft level, subsystem level (e.g. platform), equipment level (e.g. on-board computers) or in some cases at part level (e.g. passive RF parts or some families of FPGA by technology nodes).

With the analysis of the elements contained in the spacecraft, it is then possible to interpret the data through statistical methods, provided the total number of elements (or cumulated hours and observed failures) is sufficient.

In-orbit return data can be used for the development of new reliability prediction models, provided that the following conditions are fulfilled:

the item for which the In-Orbit Return data has been compiled is comparable to the item under consideration, technology and functions wise (e.g. for mechanisms or structures, same material, same shape),

the conditions of use are comparable (e.g. same or comparable orbit, as the use of data obtained for a GEO satellite cannot be used for LEO satellites without technical justification),

the thermal conditions (ambient temperature and thermal cycling) are in the same range.

In case only the first condition is not fulfilled, i.e. the IOR data is relevant for the application, but the sample size is not sufficient for a pure statistical approach, IOR data can also be used in combination with another input. The preferred approach to perform such a combination is called the Bayes inference, see Chapter 7 for details.

### Manufacturer data

There are two types of manufacturer data:

Manufacturer general data
It corresponds to the information that can be used to perform reliability calculations. The data can be related, for example, to the quality level or the qualification of a subsystem, a unit or a part, to the package information or to the complexity of a die for a EEE part, to the physical characteristics of the material for mechanical parts, to the constituting components for miscellaneous items, or the FMEA or RBDs at system or subsystem level.
These data are either requested by the project as part of the deliverable documentation (e.g. in the form of a technical note or a datasheet), provided within the elements of the data packages or available publicly (e.g. on the manufacturer’s website).

Manufacturer reliability data 
It corresponds to reliability or reliability oriented data provided by the manufacturer of the part, unit or subassembly, such as FIT rates, success probabilities or MTBF. 
It can be part of catalogues, reliability reports or datasheets, provided on request or as part of data packages, and take the form of analyses, curves, figures or tables. 
In some cases, it does not only apply to the specific item under consideration, but it can also be generic information, e.g. for a given technology or for a family of subassemblies designed and manufactured by the same entity. In those cases, it is necessary to define whether these data can apply for the specific cases.

Manufacturer general data are needed as a general input for the prediction, independent of the data sources used to develop the models. Manufacturer reliability data, in turn, can be used directly as an input for reliability modelling, provided that the following conditions are fulfilled:

if the data corresponds to manufacturer’s in-orbit return, it is considered in Section 6.2.1

if the data comes from a test, the conditions listed in Section 6.2.3 apply.

If the data is taken from another handbook data source, it should be consistent with the recommendations made in Part III of this handbook.

### Test results

For space applications, the following tests are considered, all of them performed on ground, through simulations of the use conditions for the specified lifetime:

- Screening/Burn-in:  to demonstrate that youth defects have been eliminated

- Reliability:  to demonstrate a failure rate or a probability of success at elementary level

- Functional:  to demonstrate the ability of the product to perform the specified functions

- Lifetime:  to demonstrate the appropriate safety margin versus wear-out phenomena

- Radiation: to demonstrate a SEP rate or a safety margin towards cumulated radiation dose

- Characterisation: to assess the margins of the product characteristics

For reliability assessments, the test results that can be taken into account for reliability predictions are the reliability tests and the lifetime tests, and radiation testing to investigate the impacts of radiations.

A certain number of hypotheses are taken for tests and it is important to keep some control on the assumptions to have a better understanding of the data obtained. For this reason, test results need to be carefully handled as far as the following elements are concerned:

The reference versioning (reliability tests),

The test conditions (e.g. the data measured, the detail of the test settings)

The estimation method used (e.g. Chi Square estimator),

The environment of the test, which needs to be known and managed in order not to introduce additional stresses or root causes inherent to fore mentioned environment,

The use conditions, which need to be either representative of the actual operational conditions that will be met or considered with a predefined margin,

The technology of the elements tested, which needs to be close to the technology of the part under consideration.

It is also possible to combine test data with other reliability data, see Chapter 7 for details.

In case there are different types of tests that could be performed to obtain reliability data, a priority can be defined based on the objective of the test result.

In case the accuracy of the test result is important, the most relevant aspects to consider are:

The adequacy of the test – the set of data and conditions tested has to be representative of the actual parts, environments and use conditions under consideration
e.g. a valve tested at 10 bar during 100 000 cycles might not be representative if the actual valve is used at 200 bar during 3 000 000 cycles in flight.

The test duration – the test has to be as long as possible for accuracy purpose.

If the test is used only for an order of magnitude estimation, approximations, if clearly stated, can be acceptable. If the main priority is the cost, a tradeoff should take into consideration the overall cost and overall duration of the test, and approximations can be made, knowing that it will impact the final result.

### Handbook data sources

There are a number of existing handbook data sources available that either present models (e.g. FIDES [RD10] ) or reliability data (e.g. NPRD [RD14] ) at parts level. The Chapters for each domain (EEE, Mechanical, Miscellaneous) in Part III of this handbook specify whether and for which conditions some of the existing data sources can be used for reliability prediction in space applications.

### Failure mechanisms analysis

Knowing and managing the failure mechanisms within parts or systems is generally a good starting point to perform a reliability prediction. From a practical point of view, it becomes even more interesting when no other data are available to perform the reliability prediction of a part, a unit or a subsystem, and the last resort for the reliability engineer is to analyse the failure mechanisms of the item under consideration.

The first step is to identify potential failure modes and associated failure mechanisms, e.g. by performing a Design or Process FMEA, or through engineering judgment and/or lessons learnt. The following steps will then consist in determining reliability data linked to the phenomenon identified as most likely responsible for the undesired failure mode. The approach used for analysing in depth the failure mechanisms is the Physics of Failures, see Section 6.5 for details.

### Failure mode repartition data

The present handbook generally takes into account all potential failure modes without providing information on the potential failure mode distribution for each part or component.

However, if needed for specific purposes (e.g. FMECA, safety analysis, fault tree analysis, common modes analysis, specific analyses for single point failures or Integrated Logistic Support) there are several ways to proceed in order to define the most appropriate failure mode repartition, making use of the sources discussed in the following.

For EEE mainly:

ECSS-Q-ST-30-02C FMECA [RD2]  Annex G

MIL HDBK 338B [RD13]  Section 7,

The Reliability engineer’s toolkit topic D9 by Rome Laboratory [BR5]

For both EEE and Mechanical parts:

Failure mode Mechanism Distribution FMD by Quanterion [BR6],

Company internal catalogues,

Any other source, with justification.

It should be noted that the failure modes are believed to be industry and application dependent, so it is recommended whenever possible to build an own catalogue.

## Methods and modelling approaches

Different approaches can be followed to develop reliability models. For elementary (e.g. part level) reliability modelling, one of the following approaches may be used:

Statistical approach: 
Starting with observations of an item’s behaviour under certain conditions (data from on-ground testing or operational feedback), a reliability model is derived using statistical methods. The mathematical tools and methods from classical reliability theory can be used for this purpose.

Physics of Failure: 
A reliability model is derived from a mathematical representation of the dominating failure mechanisms with due consideration of the relevant contributors (environmental and use conditions) and the associated uncertainties. Mathematical tools and methods may be taken e.g. from structural reliability theory, encompassing stress-strength interference methods but also methods to account for more complex failure mechanisms.

- Combined approach: By combination of statistical and Physics of Failure approaches, the strengths of both approaches are combined, while mitigating their weaknesses. Mathematical methods for combining different sources of information are available e.g. from the field of Bayesian statistics, allowing also the consideration of limited data samples by making use of additional information from Physics of Failure modelling.

To a certain extent, the same methods can also be applied at higher levels of assembly, in particular the statistical approach which is in principle applicable at any level from part to system, provided that suitable and sufficient data is available for the item under consideration. Physics of Failure models, on the other hand, are generally developed bottom-up, focussing on the failure mechanisms of parts or relatively small assemblies. Methods for combining elementary reliability models in system level reliability prediction are described in Chapter 12 (Part III).

Each approach makes use of different pieces of information to estimate the reliability of the considered item, see Figure 6-1 for illustration. The statistical approach uses relevant failure data to estimate the reliability of an item, whereas the Physics of Failure approach combines the analysis of the relevant failure mechanisms with information on the variability of the physical variables driving the process. Both approaches can be combined, e.g. with the aid of Bayesian updating, making best use of all available information.

Figure 6-1: Relation between different types of reliability prediction methods and the available inputs.

The following sections provide a short discussion of the three approaches listed above. Which approach is most suitable depends on the application. A simple flow chart for the selection of reliability prediction methods is given in Figure 6-2, naming only Bayesian updating as a possible combined method to use both knowledge about the physical process leading to failure and relevant failure data.

In practice, making a choice between Statistics and Physics of Failure may be less black and white than depicted in the flow chart. For example, understanding the physics can help judging the relevance of available statistical data, and vice versa. Also, it should be noted that a combination of both approaches may also be achieved in a less sequential manner than with the Bayesian approach, where the Physics of Failure is used to derive a prior model that can then be updated using statistical data on observed failures. To give an example, the FIDES approach for reliability prediction, underlying the approach for EEE reliability prediction in Part III of this handbook, makes use of statistical base failure rates that are then tuned to give relevant results for a specific application with the aid of scaling factors that have been derived from the Physics of Failure [RD10] .

Figure 6-2: Selection of reliability prediction methods depending on the available information.

## Statistical approach and classical reliability methods

Statistical methods, sometimes also called “classical reliability methods”, focus on the statistical distribution of the time to failure or other relevant characteristics related to the reliability of technical system and components, and on the estimation of the related reliability metrics from statistical data. Different data sources that are relevant for reliability prediction in space applications have been discussed in Section 6.2 above.

A correct implementation of the statistical approach requires the following three steps:

Sampling characterisation

Choice of distribution function

Parameter estimation

Once a distribution model has been selected and fitted to the data, it is generally straightforward to derive the probability of failure or survival, the failure rate, the MTTF or other reliability related metrics of interest, see Section 6.1 for details.

Each of the three modelling steps listed above will be briefly discussed below.

### Sampling characterisation

Before starting a statistical analysis, the relevant characteristics of the available data sample need to be understood. As a minimum, the following aspects and questions have to be considered:

Data representativeness: Is the sample representative for the technology and application?

Failure characteristics: Which failures (root causes, failure mechanisms/modes) are included?

Acceleration: What was the rationale behind acceleration factors applied in testing (if any)?

Sample homogeneity: Is the sample homogeneous, or merged from different populations?

Time related aspects: Which phases in an item’s life are considered by the data?

Time-to-failure data: Does the sample include individual time-to-failure data for each item?

Censoring: Is the data censored at a certain time, or at a certain number of failures?

Replacement strategy: Were failed items replaced during the observation?

Sample size: How many failures, items, and operating hours were considered?

### Choice of distribution function

The appropriate choice of distribution function depends on the physical characteristics of the failure process. Provided that sufficient time-to-failure data is available, competing models can be compared with the aid of probability plots (e.g. using probability paper, quantile-quantile plots or probability-probability plots) or based on statistical tests for goodness of fit.

Probability paper can be constructed for different one- and two parameter distributions by plotting the empirical cumulative distribution function derived from the data with a transformation of the y-axis that ensures a linear plot if the data follows the underlying distribution. The most well-known examples in a reliability context are certainly the Exponential, Weibull, Normal and Lognormal probability paper.

Alternative techniques like quantile-quantile plots or probability-probability plots are applicable also to distribution types for which it is not possible to construct a probability paper, making it necessary to estimate the distribution parameters for plotting. The same condition applies to goodness-of-fit tests, which refer to the distribution fitted to the data.

Expert or engineering knowledge about the failure process is another valuable source of information for selecting a suitable distribution model, alone or in combination with an assessment of the available statistical data. It becomes relevant especially when the data is not sufficient to decide on the appropriate distribution.

Table 6-1 provides an overview over some standard distribution models commonly used in the context of reliability prediction. The column “Derivation” provides information on the theoretical background for each model, which can be used to support the choice of distribution function. This is especially helpful if a selection purely based on data is not possible.

The probability distribution and density functions for the different models are defined in Annex II.A at the end of Part II. The two most well-known time-to-failure distribution, the Exponential and the Weibull model will be discussed in more detail in the following subsections. The modelling of standby components and one-shot items is discussed separately in Section 6.4.2.3.

Table 6-1: Failure rate characteristics of standard time to failure distribution models.

| Distribution | Failure rate | Derivation |
| --- | --- | --- |
| Exponential | Constant | Time between failures in a homogeneous Poisson process |
| Weibull | Increasing or decreasing | Limiting distribution for the minimum of a large number of i.i.d positive random variables (“weakest link”) |
| Gamma | Increasing or decreasing | Sum of Exponential distributed failure times |
| Lognormal | Increasing or decreasing | Product of independent random variables; degradation modelling with random increments proportional to the current degradation Sum of independent positive failure times (Central Limit Theorem for the sum of positive independent random variables) |
| Normal | Increasing | Sum of independent failure times (Central Limit Theorem) |
| Truncated Normal | Increasing | Sum of independent failure times (truncation ensures t>0) |
| Birnbaum-Saunders | Increasing or decreasing | Number of cycles until the length of a crack with random i.i.d. crack growth per cycle exceeds a fixed threshold (Fatigue life) |

#### The Exponential distribution for constant failure rates

The classical way to model the reliability especially of EEE components is to assume a constant failure rate λt=λ during the component’s useful life. This implies that the time to failure follows an Exponential distribution, which strongly simplifies the analysis. In particular, for a data-based estimation of constant failure rates it is sufficient to know the total number of failures and the total number of component hours or cycles in which the failures have been observed.

In the Exponential model, the probability density function fT(t) and the cumulative distribution function FT(t) of the time to failure are defined as follows:

| fT(t)= λ∙exp(-λt) | Equation 6-7 |
| --- | --- |
| FTt= 1-exp-λt=1-RTt | Equation 6-8 |

The expected time to failure is derived as ET=1/λ, which allows for a simple conversion between the failure rate and MTTF/MTBF. The standard deviation of T is equal to its mean value.

The Exponential time-to-failure model can also be derived from a homogeneous Poisson process for failure occurrence, assuming a constant failure rate. The constant failure rate assumption can be dropped by referring to a non-homogeneous Poisson process to model the occurrence of failures in time, which is not consistent with the Exponential distribution anymore, see e.g. [BR7]. Another important assumption of the Poisson process (homogeneous or non-homogeneous) is the implied independence between individual events. This assumption must be questioned if multiple failures during a relatively short time interval may be triggered by potential common-cause effects.

#### The Weibull distribution for non-constant failure rates

For some components, e.g. from the Mechanical domain, the failure rate is not constant but rather (slowly) increasing already during the part’s useful life, as a result of deterioration effects. In this case, other distributions than the Exponential model have to be used to model the time to failure. The same is obviously true when considering the wear-out phase of the bath-tub curve, e.g. in the context of life time extensions, or when modelling systematic failures that are likely to show decreasing failure rates (similar to “infant mortality” effects). The combination of a distribution accounting for deterioration effects with a constant failure rate is discussed in Chapter 12 (Part III).

The most common time-to-failure distribution model with non-constant failure rate is certainly the Weibull distribution. The probability density function fT(t) and the cumulative distribution function FT(t) of the Weibull distribution are defined as follows:

| fT(t)= βα∙tαβ-1exp-tαβ | Equation 6-9 |
| --- | --- |
| FTt= 1-exp-tαβ | Equation 6-10 |

Here, β is called the shape parameter and α the scale parameter or Weibull characteristic life. The shape parameter determines whether the failure rate is decreasing, constant or growing with time:

β<1 decreasing failure rate

β=1 constant failure rate (Exponential distribution with λ=1α)

β>1 increasing failure rate

The expected time to failure is a function of both distribution parameters and may be determined as ET=α∙Γ1+1/β, where Γ(x) is the gamma function:

| Γx=0∞tx-1exp-t dt | Equation 6-11 |
| --- | --- |

#### Models for failures of standby components and one-shot items

A special case is the consideration of components that are parts of standby systems, which are only operating when required, possibly at a random point in time. There are two different approaches to model failures of standby components:

Standby failures: Failure is assumed to occur at a random point in time during standby

Failure on demand: Failure is assumed to be triggered by the operation (demand)

The first approach assumes that failure occurs at a random point in time (i.e. during standby) and is only revealed when the component is operated. The reliability of the component is determined using the time-related failure models already discussed (e.g. Exponential distribution) to estimate the probability that a failure has already occurred during the time before operation.

The second approach assumes that failure occurs as a direct result of the operation. The probability that the component fails on demand is then modelled using statistical models for repeated experiment such as e.g. the Binomial model for a Bernoulli sequence of experiments.

A Bernoulli trial is an experiment with only two possible mutually exclusive outcomes, i.e. "success" or "failure", and a constant probability of failure p each time the experiment is conducted. The simplest example is the reliability of a one-shot item, which may be considered by a single Bernoulli experiment. The probability of y failures in n trials (to model failure on demand for items that are executed more than once) is determined using the Binomial distribution:

| pYY=y= nypy1-pn-y=n!y!n-y!py1-pn-y | Equation 6-12 |
| --- | --- |

Assuming constant p in each demand, the reliability for n demands is calculated as the probability of y=0 failures in n trials, i.e. pYy=0=(1-p)n. The corresponding distribution for the „waiting time“ (number of demands until the first failure occurs) is the Geometric distribution. More complex models can be built by modelling the failure probability p as a function of time or the number of operations.

The probability of failure on demand can also depend on certain characteristics of the item under analysis or its use (e.g. the current for a pyrotechnical device, where a Bruceton test is used to determine the probability of success or failure, see Chapter 11 Part III for details).

Which of the two models, i.e. standby failures versus failure on demand, is appropriate depends on the physical processes causing failure. It is also possible to combine both models by adding a failure on demand probability p to the time-dependent standby failure probability λt (when assuming an Exponential model for standby failures), see [BR7].

### Parameter estimation

Once a suitable distribution law has been selected, the data sample can be used to estimate the distribution parameters, i.e. to fit the distribution to the data. Various standard statistical methods can be used for this task, the preferred ones being the Maximum Likelihood Method or Bayesian parameter estimation:

The Maximum Likelihood Method or Bayesian methods with non-informative prior can be applied if the considered data sample is the only information used for modelling.

Bayesian methods with informative prior become relevant to combine different data samples or sources of information (Chapter 7).

Other methods, e.g. the Method of Moments, can be equivalent in terms of the point estimate derived for the parameters of usual time-to-failure distributions, but require additional considerations to quantify statistical uncertainties.

Maximum Likelihood estimators have desirable asymptotic (large sample) properties such as consistency and efficiency. For small data samples, the point estimate may be biased, highlighting the importance of statistical uncertainties which should be quantified (e.g. estimation by interval) rather than just providing a point estimate. Without prior information (i.e. in a non-informative analysis), Bayesian methods and Maximum Likelihood Estimation give equivalent results with respect to the point estimates of the estimated distribution parameters. However, to quantify the uncertainty associated with these estimates, Maximum Likelihood Methods rely on large sample theory, assuming an asymptotic multivariate Normal distribution for the distribution parameters. Bayesian methods do not require such an assumption, providing a more consistent uncertainty quantification especially for parameters that cannot be negative (e.g. variance parameters).

Two problems are commonly encountered during the analysis of failure data: Censoring and lack of failures. Both problems are closely related, as will be discussed in the following.

Censoring relates to observations of a random variable that are available only within a certain range, which may be fixed or random. To give an example, data from a reliability test programme with fixed duration  provides exact time-to-failure information only for a random number of items that fail during the test; for the items that survived the test it is only known that the failure time is larger than . This situation is sometimes termed Type I Censoring. An alternative test design would require a fixed number of failures, which leads to a random test duration (Type II Censoring). Various methods for dealing with censored data samples have been discussed in the literature, see e.g. [BR8]. Both the Maximum Likelihood Method and Bayesian parameter estimation allow the explicit consideration of censored data points in the formulation of the likelihood function, see Section 7.3.2 for details.

When testing high reliability components or collecting in-service data, it is likely that most samples will in fact survive the test, leading to a highly censored data set. Unfortunately, the uncertainty of the reliability estimate depends on the number of observed failures, see e.g. Equation 6-14 below. The expected number of failures for a given number of items and a given duration (cumulated hours) is proportional to the failure rate. Thus, very large data samples and/or very long durations are required for high accuracy estimates of small failure rates, which may be impossible to achieve in practice. To overcome this problem, components may be tested at higher than expected stresses in accelerated life testing.

In the following, some basic information is provided regarding parameter estimation for the Exponential, Weibull and Binomial distribution.

#### Failure rate estimation for the Exponential model

Parameter estimation is rather simple for the Exponential distribution, the constant failure rate being the only model parameter. The Maximum Likelihood Estimate for the failure rate is derived as follows:

| λ=Eλ=nfi=1nti | Equation 6-13 |
| --- | --- |

Where nf denotes the number of observed failures, n the number of items tested and ti the test duration for each item. The statistical uncertainty associated with the failure rate estimation is quantified by the following expression for the Coefficient of Variation (CoV) of the failure rate:

| CoVλ=VarλEλ=1nf | Equation 6-14 |
| --- | --- |

For homogeneous data samples (failure rate assumed to be constant both in time and across the sample), censoring does not require any specific consideration apart from the derivation of the cumulated hours in the denominator, where ti refers to

the observed time to failure for failed items that are not replaced

the test duration or censoring point for items that survived the test, or for items that failed but were replaced to continue the test.

Similar considerations are possible for data collected during operations.

It should be emphasized that Equation 6-13 and Equation 6-14 are valid only for constant failure rates. Another important assumption is that the observed failure times are from the same population, i.e. the failure rate is the same for all tested parts. The same is true for the alternative estimation methods discussed in the following.

Equations for Bayesian updating with the analytic approach using a conjugate Gamma prior are given in Section 7.5.1 (Table 7-1). Without prior information, the results are equivalent to Equation 6-13 and Equation 6-14 above. However, the failure rate is modelled with a Gamma distribution with support for positive values only, which is more consistent that the asymptotic Normal distribution assumption in the Maximum Likelihood method.

Another well-known approach to quantify the statistical uncertainty associated with data-based failure rate estimates is the use of confidence intervals. For the Exponential model, a 1-α confidence interval can be derived analytically, leading to the following interval estimate:

| Pχf,α222inti≤λ≤χf,1-α222inti=1-α | Equation 6-15 |
| --- | --- |

Here, χf,α22 and χf,1-α22 are quantiles of a Chi-Square distribution with f degrees of freedom. For a time truncated test (Type I Censoring), f=2nf+2, whereas f=2nf  becomes relevant for a failure truncated test (Type II Censoring).

Note that Equation 6-15 is based on the mathematical relationship between the Exponential, Gamma and Chi-Square distribution and fully equivalent with the Bayesian approach using a Gamma conjugate prior, see Section 7.5.1 for details.

Failure rate estimation with zero observed failures is also discussed in Section 7.5.1.

#### Parameter estimation for the Weibull distribution and other failure time distributions

The Maximum Likelihood Method can be applied also to parameter estimation for the Weibull distribution and other failure time distributions like those listed in Table 6-1. Analytic solutions are available for most two-parameter models, but can generally be applied only with complete time-to-failure data. The same is true for the closed form solutions for Bayesian updating provided in Table 7-1 (Section 7.5.1); numerical methods are required for Bayesian parameter estimation with censored data samples.

In practice, most data samples include censored observations, which is important to consider appropriately in distribution fitting for non-constant failure rates. The formulation of the likelihood function is usually straight-forward even with censored data sets, see Section 7.3.2 for details. However, the Maximum Likelihood estimate for the model parameters needs to be derived numerically:

| θ=minθ-l(θx)=minθ-lnL(θx) | Equation 6-16 |
| --- | --- |

Here, Lθx denotes the Likelihood of a parameter set θ conditional on the data x (see Section 7.3 for details), and lθx is the corresponding log-Likelihood.

The statistical uncertainty and covariance between the different parameters can be evaluated with the aid of the Fisher information matrix, which is derived from the Hessian matrix; the Hessian is usually determined as a by-product of the optimization routine used to solve Equation 6-16.

In practice, it may be sufficient to know that at least for standard distribution models and usual sampling characteristics, analytic and numerical approaches for Maximum Likelihood Estimations are implemented in various software tools for statistical and/or reliability analysis.

#### Parameter estimation for the Binomial distribution

The Maximum Likelihood Estimate for the probability of failure in the Binomial model is derived as follows:

| p=Ep=nfn | Equation 6-17 |
| --- | --- |

Where nf denotes the number of observed failures in n independent Bernoulli trials. Similar to the exponential model, for constant probability p it does not matter how many items have been tested or observed for how long and whether observations are censored – what counts is just the total number of failures and trials

The number of failures determines the statistical uncertainty associated with the estimate. Note that the normal approximation assumed for estimating the standard deviation associated with the Maximum Likelihood Estimate is not appropriate for small probabilities of failures, unless the data sample is very large. Uncertainty quantification can be improved by using the analytical Bayes approach with a conjugate prior, see Table 7-1 in Section 7.5.1 for details. Using a non-informative Jeffrey’s prior, the following interval estimate is derived:

| PBetaα2nf, n-nf+1≤p≤Betaα2nf+1, n-nf=1-α | Equation 6-18 |
| --- | --- |

Where Betaqα, β denotes the q Quantile of the Beta distribution, as defined in Annex II.A.

## Physics of failure and structural reliability methods

Physics of Failure methods for reliability prediction are based on a mathematical modelling of the relevant failure mechanisms leading to failure. It is important to make a clear distinction between the use of Physics of Failure models in the context of design versus reliability prediction:

During design, the objective is to model and account for anticipated failure mechanisms in order to avoid failures, and thus to improve reliability (design for reliability). The analysis is in most cases deterministic, using margins to account for uncertainty and variability. The probabilities of success or failure are generally not quantified.

During reliability prediction, the objective is to quantify the probability that the considered failure mechanism(s) will lead to failure for a given design. To achieve this, the relevant uncertainties and variabilities need to be quantified and appropriately considered in a probabilistic analysis.

It is also possible to combine both applications, e.g. in the context of Reliability Based Design Optimization [BR9],[BR10].

The focus of the following discussion will be on the application of Physics of Failure models in the context of reliability prediction. It should be noted that the mathematical models used to describe the failure mechanism process may be the same as the ones used for design; it is the quantification of uncertainties that makes the difference.

Thus, a sound Physics of Failure method requires a combination of physical laws describing mathematically the relevant failure mechanisms with probabilistic models that account for variability and uncertainty in the manufacturing of items and their use (e.g. the distribution laws used in the Stress Strength method). Statistical observations can be used for this task, and often are the preferred input for the required probabilistic modelling. Nevertheless, in the frame of this handbook, an approach is classified as “pure” Physics of Failure as long as no reliability data is used for the quantification (see Figure 6-1).

An important prerequisite for the successful use of Physics of Failure methods is that the physical process leading to failure needs to be well understood and the uncertainty quantification has to account for the relevant failure root causes. This can be rather straightforward in some cases, e.g. for the modelling of wear-out failures, but may be a challenge in other applications. In particular the consideration of random failures with a pure physics-based approach is certainly more difficult due to the fact that (by definition) the root cause of these failures is not known. To overcome this limitation, some existing methodologies (e.g. FIDES [RD10]  for EEE parts, NSWC for Mechanical items [RD15] ) combine Physics of Failure considerations with field return data to derive pertinent models that can be used for a wide range of applications. Note that such “combined” approaches, making use of both Physics of Failure and reliability data, are discussed separately in Section 6.6.

The following subsections provide guidance on the use of “pure” Physics of Failure reliability prediction methods, with some more details on the class of structural reliability methods provided in Section 6.5.4. A rather simple application of structural reliability methods is for the analysis of structures using the well-known stress-strength approach (see Chapter 10 Part III for details). However, the mathematical methods are well applicable to any Physics of Failure reliability prediction problem, not only for structures and including much more complex model formulations.

### Failure mechanisms identification and modelling

Applying the Physics of Failure approach first requires the understanding of the failure mechanisms contributing to the predominant failure modes for an item. Possible ways to initiate a failure mechanism analysis can be based on the following pieces of information:

Engineering analysis and judgment from design and reliability experts

Lessons learnt from previous missions

Results from a Design FMEA (DFMEA) or Process FMEA (PFMEA)

Besides gaining knowledge about all failure mechanisms that may potentially cause the item under analysis to fail, it is also relevant to know which are the dominating ones for the considered design and application. A good, though still qualitative, understanding how much different failure mechanisms may contribute (or not) to overall unreliability can help to focus the analysis and to strongly reduce the effort required by the Physics of Failure approach.

The next step is to define a mathematical model describing the failure mechanisms identified in the first step. Note that a model in this context may be an analytic function or a numerical model; the only requirement is that the failure state is clearly defined as a function of the relevant physical variables.

The effort required to develop a new failure mechanism model can be considerable. It is thus highly recommended to start by a search for published models from similar applications. A collection of models for various failure mechanisms can be found in the RiAC’s Web-Accessible Repository of Physics-based Models (WARP [RD16] ), which has been set up to foster the use of Physics of Failure methods in practical applications. Models used by the design engineers are another valuable source of information.

### Quantification of variable and model uncertainties

The failure mechanism model defines which variables need to be known to identify whether an item is in the failed or in the safe state. First deterministic estimates for each variable may often be derived from design information, typically giving a positive margin when the failure mechanism model is used with this input.

To perform a reliability calculation, it is now necessary to quantify the uncertainties associated with each variable. To reduce the effort required by this step, some variables may be considered as deterministic, provided that at least one of the following conditions is fulfilled (both criteria may be assessed relative to the other variables’ impact and uncertainty):

The variable’s impact on the failure mechanism model is negligible and/or

The variability and uncertainty associated with the quantitative estimate is small

Variables that are modelled as deterministic should be considered with their mean value if the goal is to achieve a reliability prediction that is realistic, whereas conservative estimates like lower/upper fractile values can be used if the goal is to derive a lower bound for an item’s reliability. Note that designers typically use conservative estimates.

The remaining variables must be considered as random variables, requiring modelling choices regarding the distribution type and an estimation of the distribution parameters for the considered application. The following discussion focuses on probabilistic modelling for single variables; correlation or dependence may need to be considered in case different variables are not statistically independent.

- The distribution type can be selected based on the following information:

Probability plots based on available data, e.g. test data or measurements in the field

Engineering knowledge on the underlying processes and natural bounds (e.g. strength > 0)

Published information containing any of the before mentioned

After selecting an appropriate model, the distribution parameters need to be estimated to complete the probabilistic modelling. The following sources of information can be used for this task:

Statistical data, if available in sufficient quantity

Design values, together with a statistical definition or interpretation (e.g. A- and B- value of material strength, typically defined as 1% and 10% fractile values, at 95% level of confidence)

Estimates (e.g. based on engineering judgement) for the mean value and/or coefficient of variation of the random variable distribution

Any combination of the abovementioned sources

It should be noted that a good fit of the distribution (both in terms of the selection of the distribution model and its parameters) is important especially in the part of the distribution that contributes to unreliability, which is typically the lower tail for variables governing strength, and the upper tail for many stress variables.

When judging about the variability of a random variable, e.g. to estimate the coefficient of variation, it is important to know which root causes need to be considered in the prediction. To give an example, uncertainties and variability in the design and manufacturing process may increase the variability of an item’s strength, but also that of the stresses acting on it.

Finally, it should be mentioned that also the mathematical model used to represent the failure mechanism may be associated with (model) uncertainties. Depending on the level of accuracy and sophistication of the model (e.g. simple empirical correlations versus detailed finite element code), it may be relevant to explicitly account for these model uncertainties in the reliability prediction.

### Uncertainty propagation and reliability estimation

Reliability estimation with Physics of Failure methods requires that the identified uncertainties are propagated through the failure mechanisms model to assess the probability of failure due to unfavourable combinations of random variable realizations. With the information from the previous steps, this is basically a mathematical question.

Various tools and techniques are available to perform this task, from analytic solutions for some basic problems to numerical methods, e.g. using Monte Carlo simulations. An overview on the so-called structural reliability methods is given in the following section.

### Overview of structural reliability methods

Structural reliability embraces a class of methods that can be used to derive a reliability estimate from a mathematical model describing the Physics of Failure. The starting point is the formulation of a so-called limit state function gX, defined as a function of a random vector X . The variables in X  (called “basic variables”) represent all inputs required to model the physics of failure that are associated with uncertainties. The failure domain is defined as gX≤0, bounded by the limit state gX=0, the “failure surface”.

The following steps, each corresponding to one of the previous subsections (6.5.1 to 6.5.3), are required to perform a structural reliability analysis:

- Formulation of the limit state function gX, defining failure as a function of all physical variables that are relevant for the considered failure mechanism (“basic variables”, vector X).

- Choice of suitable probabilistic models (distribution type and parameters, and dependence modelling) for these basic variables, defining the joint probability density function for the random vector X.

- Derivation of the failure probability Pf=PgX≤0 using suitable analytic or numerical methods.

To clarify the link to statistical methods for reliability prediction, it is important to understand that the probability of failure, when calculated with structural reliability methods, always refers to a specified time frame (e.g. mission life time) or event (e.g. solar array deployment), which determines the probabilistic modelling. However, the random variables in X, or possibly the limit state function itself, can be modelled as a function of time, which allows to derive the time-to-failure distribution and reliability function by performing multiple calculations for different time intervals:

| RTt=1-FTt=1-Pft=1- PgXt≤0 | Equation 6-19 |
| --- | --- |

The Physics of Failure model has to consider the dominating failure mechanism(s) contributing most to the item’s unreliability (see Section 6.5.1). The limit state function gX used for this purpose can be of any functional form, or represented by a numerical model. The straight line shown in Figure 6-3 represents the failure surface for the so-called “fundamental case” with gX=X1-X2, where failure occurs if the random “resistance” or strength X1 of an item is smaller than the “load” or stress X2 acting on it. The well-known stress-strength interference methods may thus be considered as a subclass of structural reliability methods (Chapter 10 Part III). More general models can be derived by considering more than two basic variables, nonlinear limit state functions and/or or numerical models defining the failure domain.

Figure 6-3: Illustration of structural reliability methods, where failure is defined with the aid of a limit state function gX, here depicted for a simple “strength – stress” interference model. The probability of failure is derived as the probability integral over the failure domain, i.e. where gX≤0.

Once the limit state function is defined, the second step requires a probabilistic modelling of the random variables entering the limit state function (vector X). This step is described in Section 6.5.2 above. The probabilistic models chosen may have implications on the methods that can be used for the third step. Nevertheless, basic variable models should not be selected for mathematical convenience only, as the choice may strongly affect the result. The reason becomes obvious when considering a simple stress-strength reliability problem, with the probability of failure resulting from the overlap between the two distributions. Due to the different form of the lower and upper tails of different distribution models, the reliability prediction may be very much different when switching e.g. from a Normal distribution assumption to a Lognormal model for one of the two distributions.

Finally, in the third step, a suitable structural reliability method needs to be chosen to perform the calculations. Very broadly, the following categories of methods can be distinguished.

Analytic methods
Closed-form solutions for the probability of failure exist for some special cases, mostly considering the “fundamental case” with interference of two random variables. The most well-known example (basic stress/strength approach) is a linear limit state function of two normal distributed random variables (Chapter 10 Part III).

First/Second Order Reliability Methods – FORM/SORM
FORM analysis starts with a transformation of all random variables to standardized normal space, in which the most likely point of failure (the “design point”) is identified by a search algorithm. It is defined as the point on the failure surface gX=0 having the shortest distance to the origin, defining the reliability index β. Using a first-order approximation (FORM) of the limit state gX=0, the probability of failure is a direct function of this distance β. Second-order approximations (SORM) may be used to refine the analysis.

Monte Carlo Simulations
Simulation methods are based on repeated evaluations of the limit state function with simulated realisations of the random vector X. The probability of failure is estimated by dividing the number of failures “observed” in this numerical exercise by the total number of simulations. Different variance reduction techniques are available to improve the accuracy of this estimate with reasonable numerical effort.

Surrogate modelling can be used to complement these different methods if the limit state function is not defined explicitly (i.e. using an analytic formula), but implicitly with the aid of a numerical model. The safe domain can in this case only be defined through repeated numerical analysis with different input values. A surrogate model is an analytic function that approximates the results of the numerical model, allowing to apply FORM/SORM methods and a more efficient use of Monte Carlo Simulations. Various approaches for the construction of surrogate models are discussed in the literature (see e.g. [BR11]), including Response Surfaces, Kriging, Polynomial Chaos Expansions and Support Vector Machines.

The different methods available to perform a prediction with structural reliability methods are discussed in comprehensive textbooks (e.g. [BR12],[BR13],[BR14]) and have been implemented in various software packages, both commercial [BR15],[BR16],[BR17] and freeware [BR18],[BR19]. Here, only the strengths and limitations of each method will be briefly explained.

#### Analytic methods strengths and limitations

From a practical point of view, analytic methods are clearly the method of choice whenever a quick assessment is required based on limited information, e.g. about the safety margins inherent in a design. However, the approach has strong limitations regarding the choice of limit state function and distributional assumptions, which may lead to questionable results if the underlying assumptions cannot be justified. Nevertheless, the approach can be recommended under the following conditions:

If the underlying assumptions are justified for the considered application or

If the results have been cross-checked with a benchmark based on more realistic assumptions, requiring numerical methods for solving

#### FORM / SORM methods strengths and limitations

First and Second Order Reliability Methods were originally developed to increase modelling flexibility when compared to analytic methods, while still having to deal with the computation power limitations of early computer generations. Their fast convergence is still one of their biggest strengths; a handful of iterations can be sufficient to find the design point, which provides useful additional information not only on the reliability, but also regarding the sensitivity of the prediction results to each of the basic variables.

An important prerequisite is that the limit state can be defined with an analytic formula, which implies that a surrogate model is needed to represent implicit limit states.

Possible limitations can arise when the failure surface has several likely failure points with similar distance to the origin in standardized normal space, which may lead to convergence issues in the search algorithm. It can be difficult to anticipate such behaviour due to the variable transformation prior to the search. To check convergence, it is thus recommended to run the search algorithm several times with random starting points.

Apart from this, the linear (FORM) or quadratic (SORM) approximation of the failure surface (illustrated in Figure 6-4) obviously introduces an error, whose magnitude depends on the form of the limit state function. Due to the exponential decay of the standard normal distribution when moving from the origin to the tails, the error will usually be small even with a first order approximation. Nevertheless, FORM/SORM methods should not be used with limit states that are strongly nonlinear, or in case of several limit states defining the failure domain.

Additional problems may arise in cases where the probability of failure is very high, especially for Pf>0.15. In this case, simulation techniques are more efficient and accurate and should be considered as the method of choice

Figure 6-4: Illustration of the failure surface linearization in the design point  u* in Standard Normal space. The reliability index β is defined as the shortest distance of the failure surface to the origin, allowing to derive the probability of failure from the Standard Normal distribution, Pf=Φ-β.

In summary, FORM/SORM are suitable methods to derive the probability of failure provided that the following conditions are fulfilled:

The boundary of the failure domain is defined explicitly by a failure surface gX=0, possibly derived as a surrogate model representing a numerical model.

The failure domain is defined by a single limit state function without strong nonlinearities.

No convergence issues are observed when starting the search algorithm several times with random starting values.

The probability of failure is smaller than Pf=0.15.

Monte Carlo methods should be considered if any of these conditions is not fulfilled.

#### Monte Carlo methods strengths and limitations

One of the biggest strengths of the Monte Carlo approach is its simplicity: The generation of random numbers, even using standard software is generally straightforward for most probability distributions. In addition, (crude) Monte Carlo is certainly the most intuitive way to propagate uncertainties through a Physics of Failure model, whether it is defined by an analytic function or a numerical model.

Unfortunately, the simplicity of crude Monte Carlo comes at a cost, requiring a large computational effort to perform a sufficient number of simulations for a stable reliability estimate. How many simulations are required depends on the (expected) probability of failure and the required accuracy of the results. A simple rule-of-thumb is provided by the following relationship:

| N≥2∙Pf1-Pfα∙Pf2+1 | Equation 6-20 |
| --- | --- |

With a first estimate for the required number of simulations and information about the average computation time per iteration, it may be decided whether crude Monte Carlo is a feasible approach for the considered limit state function. For some problems, the numerical effort may be too high, especially if the probability of failure is very small. In this case, it is recommended to either consider a FORM/SORM analysis (provided that the conditions of use are fulfilled, as discussed in Section 6.5.4.2 above), or to follow a Monte Carlo approach with variance reduction techniques to reduce the numerical effort required by the analysis.

The methods most commonly applied for variance reduction in the context of structural reliability estimation are importance sampling, directional simulation, Latin Hypercube sampling and conditional expectation techniques, see e.g. [BR13],[BR14],[BR20]. Of particular interest for reliability problems is the importance sampling technique, which ensures that a large share of the Monte Carlo simulations will lie in the failure region.

Another strength of the Monte Carlo approach, when compared to FORM/SORM analysis, is that it is in principle applicable also when the analytic representation of the limit state function is associated with difficulties, e.g. for limit state functions that are not differentiable or when the failure domain is defined only implicitly with the aid of a numerical model. However, the computational effort of a direct sampling approach may be prohibitive even with the use of variance reduction techniques, especially in cases where each simulation takes hours or days to perform the numerical calculations. To overcome this problem, the Monte Carlo simulations may be run with a surrogate model derived from a preliminary set of numerical results, the Design of Experiment.

In summary, the Monte Carlo method is a viable approach for all reliability estimation problems, provided that enough simulations can be made to derive small probabilities of failure with sufficient accuracy. Variance reduction techniques and surrogate modelling can help to reduce the required numerical effort, but at the cost of a more complex approach requiring more expertise from the user. FORM/SORM methods should thus be considered as an alternative whenever computational effort and thus numerical efficiency is a concern.

## Combined approach

The discussion of statistical methods and Physics of Failure so far has focused on the application of each of them as a standalone method, making use of probabilistic failure mechanisms modelling or statistical data on observed failures. Both methods have their strengths and limitations and can sometimes be difficult to apply in a specific context.

The basic idea of the combined approach is to make use of all available information, combining suitable failure data with Physics of Failure modelling to derive models that are backed both by the available statistics and by the consideration of the driving failure mechanisms and the associated contributing factors. Two different approaches are distinguished in this handbook:

Bayesian updating of Physics of Failure models, using relevant failure data

Statistical base failure rates with multiplicative factors derived from the Physics of Failure

The first approach, using Bayesian updating, uses Physics of Failure considerations to derive a prior reliability estimate, which is then updated using statistical data in order to reduce model uncertainty and to derive more realistic results. Bayesian methods for model updating are discussed more detail in Chapter 7 of this handbook.

The second approach uses statistical methods in order to establish a fixed “base failure rate” for a certain item or family of items, and the Physics of Failure is then used to determine acceleration factors depending on the considered design, its use conditions and the environment. Models based on this approach are provided e.g. in the FIDES guide [RD10] , which is considered as a basis for EEE reliability prediction in this handbook (Chapter 9 Part III), or in the NSWC handbook for Mechanical reliability prediction [RD15] , see Chapter 10 (Part III) for discussion.

Among the two approaches, the Bayesian approach is generally preferred from an uncertainty quantification point of view, as it allows to consistently account for the amount of information provided by statistics and Physics of Failure (e.g. sample size of the failure data used for updating, degree of belief in the failure mechanism modelling). However, the Bayesian approach may sometimes be difficult to apply in practice, e.g. if aggregated failure rate data is available from a different field of application, but the underlying data that could be used in a Bayesian framework cannot be accessed. In this case, the second approach with statistical base failure rates “tuned” to fit the specific application with the aid of multiplicative PoF factors may be the method of choice from a practical point of view.

## Process for the development of new models

A major requirement for the sustainability of a reliability prediction methodology is the possibility to account for technological development. Apart from model updating, which is discussed in Chapter 7, this also requires the development of completely new reliability prediction models when innovative technologies are used. The purpose of the present section is to provide practical guidance for the development of such new models, making use of the inputs and methods discussed above in consistency with the philosophy of this handbook.

It should be noted that the process provided herein is not restricted to innovative technologies – It can be used for any application where the existing models cannot be used or do not cover the required scope, whether in terms of technology, use conditions, or failure root causes.

### General process for all technical domains

The general process for the development of new reliability prediction models is outlined below.

Define the required (or intended) modelling scope
Before starting the modelling, the intended scope of the new model should be clearly defined, following the principles laid out in Part I of this handbook. The scope definition includes a discussion of:

  - The technology and item specification and boundary

  - The environmental and use conditions

  - The intended use of the prediction model

  - The coverage by failure root cause

Search for similar applications with or without existing model
Information from similar applications can help to understand the problem, and may even be considered as possible modelling inputs. Examples could be data from on-ground applications for technologies that are not (yet) used in space, models for similar technologies in space applications, or approaches to cover a specific failure root cause in a different application. Searching for similarities should always go hand in hand with understanding the differences, which is essential for evaluating the usefulness of the collected information.

Collect information from available data sources
All kinds of information that can be made available as a basis for modelling needs to be collected, considering all categories of inputs listed in Section 6.2. The search may be widened to consider also information from relevant similar applications if the available information for the specified modelling scope is insufficient.

Gain understanding of the relevant failure modes, failure mechanisms and root causes
Expertise from designers, test engineers and RAMS experts should be brought together to understand what are the potential and relevant failure causes, processes and symptoms. The Physics of Failure as well as the uncertainties and variabilities inherent in the design and manufacturing process need to be well understood in this step, even if this information is not explicitly used for modelling in the next steps.

Select an appropriate method to do the modelling

- Considering the available inputs, an appropriate method needs to be selected from the approaches listed in Section 6.3. Approaches based on the Physics of Failure are generally much more useful for the consideration of innovative technologies, as they don’t require waiting for sufficient data to accumulate. Combined methods can be used e.g. to profit from data collected from relevant similar applications, or to update a Physics of Failure model with limited data samples such as test data.

Identify and handle knowledge gaps
Any knowledge gap identified during the modelling should be documented, to make clear what kind of information must be collected to improve the model. To complete the modelling in the short run, these gaps may be closed using expert judgement.

Document modelling assumptions and evaluate uncertainties
All inputs and assumptions made during the modelling must be clearly documented, and an assessment of the model development uncertainties should be made using the checklist approach described in Section 8.3.

Anticipate future model updates
Once a preliminary model has been set up, the model can be updated with new information, e.g. by closing identified knowledge gaps or using methods from Bayesian statistics (see Chapter 7 for details).

### Development of new models by technical domain

The process for the development of new reliability models is applicable to all technical domains. Additional information can be found in the dedicated chapters of Part III:

EEE: New reliability prediction models for EEE items can be derived using different inputs, as discussed in Chapter 9 (Part III). For new technologies there is no data available from IOR, making it necessary to rely on manufacturer data or dedicated testing.

Mechanical: The general process for the development of new reliability prediction models is essentially equivalent to the approach for mechanical reliability prediction presented in Chapter 10 (Part III), where a strong focus is set on the relevant failure mechanisms and the associated Physics of Failure. Bayesian methods for model updating allow the consideration of available data (e.g. test data), even with limited sample size.

Miscellaneous: Chapter 11 (Part III) provides reliability models and data for commonly used miscellaneous items with sufficient IOR data. The reliability of "non-standard model" items, requiring a new dedicated model, can be considered based on Engineering/PoF approaches mainly based on DFMEA/PFMEA

# Bayesian methods for reliability prediction modelling

Matthias Schubert and Katharina Fischer, Matrisk GmbH

Goal of this chapter: To provide a discussion and guidance for the use of Bayesian statistical methods for reliability modelling, and for the updating of existing models from different sources.

## The Bayes rule and its application for estimation and model updating

Bayesian methods for reliability prediction can be used in two different ways:

As an approach for model updating when new data becomes available (statistical or combined approach making use of different sources of information).

As an estimation method for model development based on statistical data (pure statistical approach making use of a single data sample without prior information)

Both applications make use of the same theorem, called Bayes rule. Before explaining the two different uses, the basic principle of Bayesian statistics is briefly introduced mathematically for the case of a univariate random variable. Other applications of the Bayes rule are discussed later in dedicated subsections.

Bayes theorem in a reliability context

Consider a random variable X with probability density function fXx. If θ denotes a vector of distribution parameters, the density function of X is written as fXx,θ. If the parameters are uncertain, the density can be considered as a conditional density function, i.e. fXxθ. The vector θ now denotes a realisation of the random vector Θ and the initial probability density function for the parameters, f'Θθ, is denoted the prior density function.

Now let us assume that n independent observations (realisations) of the random variable X are available that can be used to update the initial parameter estimate. The data sample is stored in a vector x=x1,…,xnT. The new (updated) density function f''Θθx of the uncertain parameters Θ given the observations x is denoted as posterior density function. From Bayes’ rule it may be derived as follows:

| fΘ''θx=Lxθ∙fΘ'θDΘLxθ∙fΘ'θdθ | Equation 7-1 |
| --- | --- |

where Lxθ=i=1nfXxiθ is the likelihood of the given observations assuming that the distribution parameters are equal to θ. The denominator of Equation 7-1 is a normalizing constant, which ensures that the posterior density function f''Θθx integrates to unity. To understand Bayesian updating it is sufficient to know that f''Θθx∝Lxθf'Θθ.

In a reliability context, X may be for example the random time to failure of an item with a failure rate θ=λ, assumed to be constant in time. The conditional density function fXxλ (also called “sampling distribution”) is in this case defined by the Exponential distribution, conditional on the failure rate λ. The prior distribution for the failure rate, f'Λλ, may be derived e.g. from a handbook estimate, and can be updated with new time-to-failure data.

### Bayesian methods for model updating

The full potential of Bayesian analysis can be achieved by using an informative prior which already contains some information on the required model parameters. Bayesian updating in this context may be seen as finding a compromise between prior knowledge and new data, weighted with the confidence we can have in both sources of information. The concept is of interest especially in situations where prior knowledge is available and/or the new data sample is not large enough to allow for a pure statistical analysis. For large data samples, the influence of the prior vanishes and the Bayesian estimate converges to the frequentist (e.g. Maximum Likelihood) estimate.

An illustrative example with simulated data is given in Figure 7-1. The grey solid curve shows the prior distribution of a constant (i.e. time invariant), but unknown part failure rate λ, modelled as a random variable Λ. The prior represents a preliminary estimate, which may have been derived e.g. based on handbook data from a different application. The prior is now updated with artificial (simulated) data for the random time to failure, assuming a true value of λ=10-4h-1. The posterior distribution for λ (dashed curves) is derived by updating the prior with the simulated data. With growing sample size n (in the example defined as the number of items with observed failure times), the posterior distribution slowly approaches the true value, but also small data samples can already improve the estimate.

Figure 7-1: Bayesian updating of a prior estimate for a constant failure rate λ with simulated data for the random time to failure, assuming a true value of λ=10-4h-1.

### Bayes rule as estimation method

As has been discussed in Section 6.4, Bayesian statistics can also be used as a method for the estimation of model parameters from statistical data, without consideration of prior knowledge. When compared correctly, the point estimates derived from a Bayesian analysis are generally equivalent to the Maximum Likelihood Method, which takes basis in the same Likelihood function Lxθ as used by Equation 7-1. However, the uncertainty quantification provided by Bayes rule is more consistent than the Normal approximation used by the Maximum Likelihood Method.

To apply Bayesian methods for parameter estimation, the prior distribution f'Θθ should be selected as a non-informative prior, see Section 7.4.2 for details. Examples for Bayesian updating with non-informative priors can be found in Section 7.5.

## Applying Bayesian methods in practice

Applying Bayesian updating of a reliability estimate with new statistical data generally requires the following steps:

Formulation of the likelihood 
As a first step, a distribution type representing the observed data must be selected, e.g. Exponential, Weibull or another Time-to-failure distribution if the data contains observations of the time to failure (or number of cycles to failure) during several tests. The choice of the sampling distribution and the definition of the likelihood is discussed in Section 7.3. In the likelihood formulation it also has to be considered if the data set is censored and if the distribution is truncated.

Formulation of the prior distribution
In a Bayesian analysis, one or several parameters of the sampling distribution are treated as random variables. The prior distribution should represent the available prior knowledge on these parameters, leading to an informative prior. A non-informative prior can be used if no prior information is available and the Bayesian analysis is used as an estimation method. Two steps are required to define the prior distribution

  - Selection of the distribution type, e.g. considering the natural constraints of the distribution parameters. A conjugate prior may be selected to simplify the analysis

  - Parameter estimation for the selected prior distribution, e.g. based on alternative data sets, handbook data, Physics of Failure analyses or expert judgement.

- The general principles of the prior definition are discussed in Section 7.4, with some more practical guidance for specific applications provided in Section 7.6.

Bayesian updating with new data:
Updating the prior with data leads to a new, posterior distribution for the parameters of the sampling distribution.

  - With a conjugate prior, analytic formulas are available to perform this step

  - Otherwise, Markov Chain Monte Carlo (MCMC) methods can be used.

The final outcome of a Bayesian analysis is the posterior distribution of the model parameters, which may be naturally interpreted to provide credibility ranges for the parameters of the sampling distribution, e.g. for the failure rate in an Exponential model.

## Likelihood formulation

In Bayes rule (Equation 7-1), the likelihood Lxθ=i=1nfXxiθ contains the information provided by the data sample that is used for updating. The formulation of the likelihood is based on the sampling distribution fXxθ, which defines also the model parameters θ to be updated. This is why the consideration of the sampling distribution is always the first step in a Bayesian analysis.

### Distributional model for the sampling distribution

The sampling distribution is the distribution of the random variable that can be observed, i.e. the variable for which data is available for model estimation or updating. In a reliability context, this can be the time to failure or number of failures observed, but also other observable characteristics that are modelled probabilistically, e.g. the strength of a material or item w.r.t. imposed stresses, which can be measured in tests.

In a Bayesian analysis, the sampling distribution is needed for the formulation of the likelihood, which can in the simplest case – for a continuous random variable X without truncation or data censoring – be defined as follows:

| Lxθ=i=1nfXxiθ | Equation 7-2 |
| --- | --- |

The selection of an appropriate distributional model for the sampling distribution fXxθ follows the same principles as in the case of model development using statistical methods (Section 6.4.2), or for the basic variable modelling required for probabilistic Physics of Failure methods (Section 6.5.2). A formal comparison of different competing distribution types can be achieved with the aid of Bayesian information criteria [BR21].

In addition to the selection of a distribution type, aspects of truncation and censoring may have to be considered, as will be discussed below.

### Censoring and truncation

For the correct formulation of the likelihood, it is important to distinguish truncated sampling distributions from censored data samples.

Truncation applies when observations are in principle not possible below or above a certain threshold, but the selected distribution type supports values below or above this threshold. To improve the modelling and avoid unexpected results, the distribution can be truncated by “cutting off” the lower or upper tail, and renormalizing to ensure that the probability density function integrates to unity. A classic example is the truncated Normal distribution, which ensures that the distribution does not support negative values.

Censoring is considered when values below (right censored) or above (left censored) a specific threshold are theoretically possible, but could not be observed in the data used for updating. To give an example, data on observed failures may include both failed items (with observed time to failure), and items operating without failure until the end of the observation period. These “successes” represent censored data points, i.e. the exact time to failure is unknown, but it is known that it must be larger than the censoring point. Also interval censoring is possible. In this case, the specific value is not observed but it is known that the failure time lies within a given interval.

It should be noted that even if the data set is not censored, it can be necessary to work with “artificial” censoring, e.g. if the wear out phase is modelled and only failures after a certain number of operating hours are of interest. Similar considerations hold for basic variable modelling in a stress/strength approach, where the model needs to be precise especially in the lower or upper tail of the distribution. Thus, censoring needs to be applied if the focus is on a specific part of the distribution or the bath tub curve.

Note that the definitions of truncation and censoring given above are different to the terminology used to describe reliability tests, e.g. a time truncated test will lead to a censored data sample.

Censoring and/or Truncation need to be considered in the formulation of the likelihood function, see Figure 7-2 and the general formulas given below. Bayesian updating with closed-form solutions using conjugate priors (Section 7.5.1) is generally possible only with likelihoods that are neither truncated nor censored. Two important exceptions are the time-constant failure rate model (i.e. Exponential) and the Binomial distribution with constant probability of failure in each demand, which can be updated analytically also with censored data containing information on the number of failures and the cumulated hours or demands.

The likelihood L for a right censored data set is:

| L∝ i=1nfxi¦θ⋅j=1m1-Fxup¦θ | Equation 7-3 |
| --- | --- |

Where xup is the upper censoring point, θ represents the distribution parameters, xi are the observed failures, f. is the probability distribution function and F. is the probability density function. The likelihood formulation for left censoring is:

| L∝ i=1nfxi¦θ⋅j=1mFxiow¦θ | Equation 7-4 |
| --- | --- |

Where xiow is the lower censoring point. The likelihood formulation for interval censoring is:

| L∝ j=1mFxup,i¦θ-Fxlow,i¦θ | Equation 7-5 |
| --- | --- |

For the situation where the likelihood needs to be truncated, right, left or interval truncation is in principle possible. The Likelihood in the case of right truncation is given by:

| L∝ i=1nfxi¦θFxup¦θ | Equation 7-6 |
| --- | --- |

The likelihood formulation in the case of left truncation is

| L∝ i=1nfxi¦θ1-Fxlow¦θ | Equation 7-7 |
| --- | --- |

Figure 7-2: Guidance on the selection of a censored and or truncated likelihood.

## Guidance on the prior selection

In a Bayesian analysis, the distribution parameters θ of the sampling distribution – e.g. the failure rate in the case of an Exponential time-to-failure model – are considered as random variables. The prior distribution represents the available knowledge for each model parameter, prior to the consideration of the new data set used for updating.

Bayesian methods can be used with informative or non-informative priors. Guidance on the principle selection of a prior is provided in Figure 7-3. In general, if an existing model is available and the aim is to update this prior, then the prior is called informative. If no data-based prior is available then it should be checked whether other information, like engineering knowledge or handbook data, is available and can be used to build an informative prior. Only in the case that no information is available except for the new data, a non-informative prior should be used.

Figure 7-3: Guidance on the selection of an appropriate prior.

It should be noted that for informative priors, the information content is expressed by the dispersion of the prior distribution: A narrow prior distribution with small coefficient of variation will have a strong effect on the results of a Bayesian analysis and is only appropriate if sufficient information is available for the prior formulation. A wide prior distribution, in contrast, will have a small impact especially in the case of large data samples used for updating. This behaviour is sometimes used to formulate a so-called “vague” prior distribution with very large scatter, which can be considered non-informative for practical purposes, having negligible impact on the results.

### Informative prior definition

As for the sampling distribution, the definition of an informative prior first requires the selection of a distributional model. In the special case of a prior that has been defined based on statistical analysis for a previous and independent data sample, the distribution of the prior may be defined based on the theory of estimation. The chi square distribution for constant failure rate estimates (representing a special case of the gamma distribution) is a well-known example in a reliability context.

The selection of a prior based on other types of information such as engineering or expert knowledge is less straight-forward, requiring additional considerations. The difficulty is that the parameters of the sampling distribution are generally not observable, making it hard to define objective criteria for the selection of the prior distribution.

A possible starting point is to consider some basic characteristics of the parameters to be modelled:

What is the required support of the distribution (e.g. variance parameters must be positive)?

Is the parameter expected to have a symmetric or asymmetric probability distribution?

Answers to these questions allow a first selection of candidate probability distribution families, which can then be used as a starting point for the expert elicitation process described in Section 7.6.3.

A further rationale for selecting a distribution family is mathematical convenience, as some distribution families are significantly easier to handle analytically (see Section 7.5.1 for details). However, the goal of the model is always to provide the best possible mathematical representation of a phenomenon or process and mathematical convenience is only a secondary goal.

Once a suitable distribution family (or candidate families) has been identified, the parameters of the prior distribution, also called hyper-parameters, need to be determined. Different sources of information can be used depending on context, as discussed e.g. in Section 7.6.

### Non-informative prior selection

A non-informative or vague prior provides no or very little (negligible) information on the parameters, whose posterior distribution will thus be determined mainly by the data. It is important to understand that despite their name, non-informative priors will still influence the shape of a posterior, and appropriate care must be exercised when defining a prior distribution.

Non-informative priors can be selected based on the following principles:

- In case that only one parameter needs to be estimated, a Jeffreys prior can be used [BR22].

- If the problem is multivariate, then a reference prior (vague prior) can be used [BR23].

An example for the use of a Jeffreys prior for Bayesian updating in the case of an Exponential time-to-failure model is:

| f'Θθ∝1λ | Equation 7-8 |
| --- | --- |

The reference prior is used e.g. for the Weibull distribution:

| f'Θθ∝1αβ | Equation 7-9 |
| --- | --- |

For practical purposes, it is also an option to define a vague prior e.g. as a flat uniform prior or a normal prior with large coefficient of variation. The effect of the prior model (e.g. the upper and lower bound assumed for the uniform prior) on the results should be checked to avoid misleading conclusions.

## Methods for Bayesian updating with new data

Once the prior has been fully defined, the posterior distribution of the model parameter θ is derived by updating the corresponding hyperparameters with new data. The posterior distribution of θ can be used together with the sampling distribution to derive reliability estimates at various levels of confidence.

In the following, Bayesian approaches for updating prior probabilistic models with new information are discussed, considering both analytic and numerical approaches. In Figure 7-4 guidance is provided on the selection of the updating method. If the failure rate or failure probability is constant (i.e. Exponential or Binomial distribution) and a conjugate prior exists, the analytical approach can be used. For time dependent failure rates, the analytical approach can only be applied if there is no need to consider censored data or truncation and if a conjugate prior exists.

Figure 7-4: Guidance on the selection of the Bayesian methodology.

### Analytical approach with conjugate prior

In general, when applying Bayes theorem for updating, the prior distribution and the posterior distribution of the random hyper-parameters (e.g. the failure rate) do not have the same distribution type. A prior that does not change the distribution type from prior to posterior is called a conjugate prior; an analytical solution is possible if a conjugate prior exists [BR24]. Conjugate priors are available for several distributions, see Table 7-1 for some models that are of relevance in reliability applications. The use of the table can be explained as followed:

Sampling distribution
This is the distribution of the observable variable for which data is available for updating (e.g. time to failure), see Section 7.3. The analytic formulas in Table 7-1 are only applicable if the likelihood is formulated without censoring and truncation.

Conjugate prior
The conjugate prior defines the prior distribution of the sampling distribution parameters which must be selected for analytic updating. Note that for two-parameter models like Weibull, only one parameter can be updated and the other one is assumed to be deterministic. Bayesian updating of both model parameters requires a numerical approach.

Prior parameter interpretation
This column provides an (alternative) interpretation of the prior and posterior hyperparameters which can be helpful e.g. to derive the parameters of the prior distribution.

Posterior hyperparameters
Based on the prior hyperparameters (denoted with a single prime) and the observations x=x1, x2,…,xn in the new dataset, the prior distribution is updated using the analytic formulas provided in this column. In conjugate analysis, the posterior distribution of the random model parameter is of the same distribution type as the prior, but with updated hyperparameters (denoted with double prime).

Posterior mean
The posterior mean of the uncertain model parameter can be derived from the conjugate prior distribution with posterior hyperparameters. It is a suitable point estimate for the considered model parameter if realistic predictions are aimed at.

Table 7-1 can be used only if a complete (uncensored) time-to-failure data set is available for updating. For the two time-constant models, Exponential and Binomial, an alternative formulation is provided in Table 7-2, allowing to perform the updating with new information for the number of observed failures and the cumulated hours (or number of demands in case of the Binomial model). For the remaining models in Table 7-1, censoring needs to be considered with numerical MCMC methods, using the likelihood formulations discussed in Section 7.3.2.

Table 7-1: Conjugate priors for various standard life distributions, for updating with complete (uncensored) time-to-failure data (see Annex II.A for parametrization).

| Sampling distribution | Conjugate prior for unknown parameter | Interpretation of hyperparameters | Posterior hyperparameters after updating with data x | Posterior mean |
| --- | --- | --- | --- | --- |
| Exponential Failure rate λX | λX~Gammaα',β' | α time-to-failure observations that sum to β cumulated hours | α''=α'+n β''=β'+i=1nxi | μλX''=α''β'' |
| Gamma Rate βX, Shape αX | βX~Gammaα',β' αX assumed to be known | α time-to-failure observations that sum to β cumulated hours | α''=α'+n⋅αX β''=β'+i=1nxi | μβX''=α''β'' |
| Normal Mean μX, Standard dev. σX | μX~Normalμ',σ' σX assumed to be known | Sample mean μ of time-to-failure data, standard deviation of sample mean σ | μ''=σ''2⋅μ'σ'2+i=1nxiσX2 σ''=1σ'2+nσX2-12 | μμX''=μ'' |
| Lognormal Mean μX, Standard dev. σX | μX~Normalμ',σ' σX assumed to be known | Sample mean μ of logarithmic data, standard deviation of sample mean σ | μ''=σ''2⋅μ'σ'2+i=1nlnxiσX2 σ''=1σ'2+nσX2-12 | μμX''=μ'' |
| Weibull Scale αX, Shape βX | 1αXβX~Gammaα',β' βX assumed to be known | α observations with sum β of the βXth power of each observation | α''=α'+n β'=β'+i=1nxiβX | μαX'' can be estimated numerically |
| Geometric Probability pX | pX~Betaα',β' | α demands-to failure observations that sum to β total demands | α''=α'+n β''=β'+i=1nxi | μpX''=α''α''+β'' |

Table 7-2: Conjugate priors for time-constant failure rate or failure probability models, for updating with censored and/or aggregated failure data (see Annex II.A for parametrization).

| Sampling distribution | Conjugate prior for unknown parameter | Interpretation of hyperparameters | Posterior after updating with nf observed failures | Posterior mean |
| --- | --- | --- | --- | --- |
| Exponential Failure rate λX | λX~Gammaα',β' | α failures in β cumulated hours | α''=α''+nf β''=β''+T | μλX''=α''β'' |
| Binomial Probability pX | pX~Betaα',β' | α failures versus β successes | α''=α'+nf β''=β'+N-nf | μpX''=α''α''+β'' |

In the remainder of this section, two examples are provided for the use of Table 7-1, considering the Exponential distribution case.

Example: Analytical Bayes approach for the Exponential distribution

In Table 7-3 an artificial data set is provided on component level time-to-failure data. Here it is assumed that all items have been operated until failure. In practical applications in many cases also the operational hours for the parts which did not fail is available. If this is the case a censored approach should be applied and the equations provided in Table 7-2 should be used.

Table 7-3: Example data set with observed failures at certain operation hours.

| Data | Failure hour |
| --- | --- |
| 1 | 19638 |
| 2 | 67068 |
| 3 | 129493 |
| 4 | 148461 |
| 5 | 116517 |
| 6 | 6210 |
| 7 | 1698 |
| 8 | 21674 |
| 9 | 121452 |
| 10 | 45599 |

From the data set provided in Table 7-3 the distribution of the failure rate can be calculated using the conjugate prior distribution.

The parameters of the posterior gamma distribution for the failure rate λ are (see Table 7-1):

| β''=i=1nti=677810α''=n=10 | Equation 7-10 |
| --- | --- |

Assuming that a vague (improper) prior is used for the gamma distribution:

| β'=0α'=0 | Equation 7-11 |
| --- | --- |

With these parameters the posterior distribution is fully defined. In Figure 7-5 the posterior distribution of the failure rate for this example is shown.

The posterior mean and standard deviation of the failure rate are:

| μ''λ=α''β''=10 677810=1.47 ∙10-5σ''λ=α''β''=ni=1nti=4.66 ∙10-6 | Equation 7-12 |
| --- | --- |

Figure 7-5: Posterior probability distribution for the estimated failure rate using a conjugate prior distribution.

Example: Analytical Bayes approach for the Exponential distribution with zero failures

In practice sometimes no failure event is observed but an estimate of the failure rate still needs to be made. This is in principle possible if operational data is available. Assuming a constant failure rate, the random time to failure is exponential distributed. We are looking for the probability that no failure events have occurred until the moment of the prediction, which can be derived from the Poisson process. The probability that no event occurred during the cumulated hours T is:

| Pr0,T=1-e-λT | Equation 7-13 |
| --- | --- |

And thus,

| Pr0,i=1nti=1-e-λi=1nti | Equation 7-14 |
| --- | --- |

The (prior) probability that no event has yet occurred is unknown. If the probability is unknown, it can be represented by a uniform distribution, leading to the following non-informative prior.

fp=1  ∀   0≤p<1

This assumption results in

| λp=-ln1-0pfpdpi=1nti | Equation 7-15 |
| --- | --- |

The failure rate as a function of the operation hours in the case no failure event was observed is shown in Figure 7-6.

Figure 7-6: Estimated failure rate as a function of the operation hours in the case no failure event was observed, using a Bayesian approach with non-informative prior.

The average failure rate can be estimated by

| Eλ=01-ln1-0pfpdpi=1ntidp=1i=1nti | Equation 7-16 |
| --- | --- |

This means that in average one failure is expected to be observed during the cumulated hours.

The number of failures corresponding to a median failure rate is calculated by

| λ0.5=01-ln1-00.5fpdpi=1ntidp=-ln⁡(0.5)i=1nti | Equation 7-17 |
| --- | --- |

From this equation it is obvious that the failure rate decreases with the number of operating hours without failures.

Another possibility is the estimation of the failure rate using the Chi-Square distribution. The Chi-Square distribution is a special case of the gamma distribution, which represents the conjugate prior distribution of the failure rate.

Using the Chi-Square distribution χf,q2 with f degrees of freedom, the q-quantiles of the uncertain failure rate λ can be determined, see also Equation 6-15 in Section 6.4.3.1. For time truncated records, the following is valid:

| λ=χ2nf+1,1-α22⋅i=1nti | Equation 7-18 |
| --- | --- |

Where nf denotes the number of failure events, α the level of confidence and n the number of items observed (with or without failure). The equivalent number of failure events nequi,1-α at a certain confidence level can be estimated to:

| nf,equi,1-α=χ2nf+1,1-α22 | Equation 7-19 |
| --- | --- |

The equivalent number of failure events is the expected number of failures for a certain confidence level during the cumulated hours i=1nti. This might not be an integer. If the confidence level is set to 50% (corresponding to the median) and no failure is observed so far, then the equivalent number of failures are:

| nf,equi,1-α=χ20+1,1-0.522=1.38632=0.6931=-ln(α)=-ln(0.5) | Equation 7-20 |
| --- | --- |

This corresponds to the median of the posterior distribution. More generally, the equation can be formulated as (still under the assumption that no failure has been observed, i.e. nf=0):

| nequi,1-α=χ20+1,1-α22=-ln(α) | Equation 7-21 |
| --- | --- |

The Chi-Square estimator is thus for the case that no observation is made equal to the approach with the Bayesian approach assuming a Poisson process and a non-informative prior.

### Numerical approach using Markov Chain Monte Carlo Methods

In case where no analytical solution is available, numerical approaches can be used. In the Bayesian rule (Equation 7-1 in Section 7.1) the likelihood function and the prior density function are known and can be easily calculated. For calculating the posterior function, the denominator needs to be solved which is in general analytically not possible and numerically difficult. This problem is solved by a class of numerical algorithms, the so-called Markov Chain Monte Carlo (MCMC) methods [BR25]. These methods are independent from the distribution of the prior density function and the formulation of the Likelihood function, and can be applied to a wide class of problems.

Besides offering full flexibility in terms of prior and sampling distributions, MCMC methods can also be used for the development of more complex probabilistic models, such as e.g. hierarchical models to combine different non-homogeneous data samples.

Several MCMC programs and toolboxes are available, e.g. WinBUGS/OpenBUGS [BR26], JAGS [BR27] or Stan [BR28], to mention just a few. An introduction and overview on the use of Bayesian methods in a reliability context, using many MCMC examples, can be found in a NASA guide dedicated to Bayesian methods for risk and reliability [BR29], including the following topics:

Bayesian inference for common time-to-failure models

Time-trend models for the failure rate λ or binomial probability p

Treatment of uncertain data and data censoring

Using system level information to estimate component-level reliability

Bayesian model choice and model validation

In the following a numerical example of the application of the MCMC is shown. In this example the failure hour data set given in Table 7-3 is used, allowing a comparison of the results with the analytic solution. Another example for the use of numerical methods can be found in Chapter 10 (Part III) for Mechanical reliability prediction, applying Bayesian updating to improve a wear-out model derived from the Physics of Failure with In Orbit Return observations.

Example: Numerical Bayes approach for the Exponential distribution

As a model for describing the data, the exponential distribution is used (constant failure rate), assuming a complete record without censoring.

The likelihood is defined by:

| Lxλ=i=1nλ⋅e-λxi | Equation 7-22 |
| --- | --- |

Where n denotes the number of (here: time-to-failure) observations of the random variable x. Now, a prior distribution has to be defined. For the purpose of the example, the Jeffreys prior for the exponential distribution is used, which is a special case of the Gamma distribution:

| f'θ=f'λ=1λ | Equation 7-23 |
| --- | --- |

Further, an initial starting point for the MCMC has to be defined:

| λ0=120'000=5⋅10-5[/h] | Equation 7-24 |
| --- | --- |

This starting point is maybe not a good choice knowing the data but it shows that the way back to more realistic values is found during the simulation. The starting point together with the prior information is of importance to obtain a good result.

In general, the Markov chain needs a burn in phase. This burn in phase allows the parameters to converge to the relevant region of interest. The length of the burn in phase depends on several factors and a plot of the Markov chain can help to decide if the choice was appropriate. In the case the chain is non-stationary the choice might not be appropriate. Plotting several chains in one plot also helps to decide if the choice was appropriate.

If the guess of the initial point is already good, then the burn in phase is shorter. The Markov chain for the example is provided in Figure 7-7 using the data provided in Table 7-3. It can be seen that it scatters around a value of 1.5 ⋅10-5, corresponding to the closed-form solution provided in Equation 7-12. After excluding the burn-in phase, the simulated values from the Markov chain are used to numerically represent the posterior distribution of the uncertain failure rate λ.

The example calculations were performed using the so-called Metropolis algorithm. In the literature further developments can be found such as the Hamiltonian Monte Carlo. These more advanced methods follow in principle the same procedure but they are in general smarter in proposing where to jump next. [BR25].

Figure 7-7: MCMC chain for the example of failure rate estimation with non-informative prior, considering the data presented in Table 7-3.

From the Markov chain the statistical characteristics of the estimated parameter can be obtained, i.e. the mean value, the standard deviation and the distribution type. In this example, using a burn in phase of 10’000 simulations and 40’000 simulations for the chain, the mean value and the standard deviation are

| Eλ=1.467⋅10-5 [1/h]STDλ=4.731⋅10-6 [1/h] | Equation 7-25 |
| --- | --- |

The distribution of λ is the same as shown for the result using the conjugate prior (see Figure 7-5). It can be seen that the λ is not normal distributed. The advantage of the MCMC is that no assumption for the distribution type needs to be made and that the credible interval of the estimation is coming for free in this method.

## Practical guidance for Bayesian updating in specific applications

In the following, some practical guidance is provided, mainly focussed on the definition of informative priors, for different applications of Bayesian updating in a reliability context.

### Updating of estimates from previous statistical analysis

Bayesian updating for a prior derived from a previous statistical analysis on an independent data set is the most straight-forward case for the definition of a prior distribution. The parameters of the prior distribution (the hyperparameters) are in this case simply derived from the existing (“prior”) data set. The advantage of Bayesian updating in this context is that information from a previous analysis can still be considered even without having access to the original data sample. Note, however, that this requires that not only a point estimate for the model parameters, but also some information on the statistical uncertainty associated with the parameter estimate is available.

It is important to note that priors derived with the aid of statistical estimation methods provide information only on statistical uncertainty resulting from sample size limitations. A more general view on various modelling uncertainties is taken in Chapter 8 of this handbook, including a discussion of different sources of uncertainty. The analyst may choose to increase the variance of the prior distribution to account for additional uncertainty sources, e.g. if the prior data does not exactly represent the considered technology or operating conditions. An increased variance of the prior is equivalent to reducing the “equivalent sample size” of the prior data set and thus reducing the weight of the prior, and giving more weight to the new data. Obviously, this is only justified if the new data set does not introduce large additional uncertainties as well.

### Updating of reliability estimates derived from reliability handbooks

Bayesian updating of estimates derived from reliability handbooks in principle follows the same approach as outlined above. However, the prior now has to be derived based on the information provided in the handbook. In the following, the basic steps to perform a Bayesian updating in this context are briefly discussed.

As a general rule, the probabilistic model for the likelihood (sampling distribution) should be consistent with the time-to-failure distribution assumed in the handbook. Many handbooks assume an Exponential distribution for the random time to failure (constant failure rate assumption), even though this may not be appropriate for certain applications, e.g. for mechanical parts.

A simple approach to transform an Exponential model provided by a reliability handbook to a Weibull model that is equivalent in terms of average failure rates is to make an assumption on the Weibull shape parameter βX, allowing to derive the scale parameter αX from the constant failure rate estimates provided by the handbook [RD14] . As an alternative, the Weibull shape parameter βX may be derived from new data, or updated together with the scale parameter αX. The prior distribution of βX then has to be defined based on expert judgement or assuming a mean value EβX=1 (together with some standard deviation), for consistency with the Exponential distribution assumed by the handbook.

Once the distributional models have been chosen (e.g. Exponential or Weibull sampling distribution, with associated priors), the parameters of the prior distribution have to be defined based on the information given in a handbook. This will typically require some additional assumptions, as most handbooks only provide point estimates for the failure rate without a detailed discussion of their uncertainty.

As a first step, the statistical background and methodology underlying a handbook needs to be reviewed in order to determine whether the prediction may be seen as a mean value or rather as conservative estimate such as an upper confidence bound for the failure rate. Unfortunately, this is not always clearly defined in the handbooks. A common assumption is to take the handbook values as 60% confidence estimates for the failure rate if no alternative definition is available (see e.g. ECSS-Q-HB-30-08A [RD4] ).

In addition, the uncertainty of the predicted failure rate needs to be quantified based on the information given in a handbook and/or expert judgement. Many handbooks do not provide confidence bounds for the failure rate, as the estimates are based on a combination of various data sets and not all uncertainties associated with the prediction are of statistical origin. An alternative approach is to treat the ratio between the “observed” and the “predicted” (based on a handbook) failure rate as a random variable, covering all uncertainties associated with the prediction method. Quantitative estimates for this ratio are provided e.g. in the NPRD data handbook [RD14] , in 217plus [RD11] , and in [BR30] for calculations based on the FIDES guide [RD10] . The figures provided may serve as rough approximation for the “model uncertainty” of the considered handbook methods and possibly also for other, similar reliability handbooks.

### Prior definition based on expert elicitation

Formulating probabilistic models from expert judgment is called elicitation. Elicitation generally entails consulting one or multiple experts. The goal of elicitation for the definition of a prior distribution in a Bayesian analysis is twofold:

Identify a prior probability distribution family, and

estimate parameters for the prior distribution.

To achieve this, the expert(s) can be asked questions regarding both the central tendency and the variation of the prior distribution parameter. Possible questions might include:

What are the minimal and maximal values you expect for the parameter?

What do you expect the mean, median or mode of the parameter to be?

With what probability do you expect the parameter to be smaller or larger than a certain value?

In addition to an answer for each question, the expert’s confidence in the answer should be registered. In translating the answers into parameter estimations, possible cognitive biases which may “distort” the expert’s answers also need to be considered. The quality of the parameter assessment might be improved by consulting multiple experts.

The results can be used to decide between possible candidate distribution families, and to derive the (hyper-)parameters of the selected prior distribution.

For the selection and evaluation of experts, several methodologies are available, see [BR31]. For a thorough and formal description of elicitation processes refer to the following literature: A detailed method for elicitation is described in [BR32]. Further literature on the formulation of prior distributions is found in [BR29], [BR32] and [BR33]. An online tool for elicitation is described in [BR34]. An approach focussing on the definition of a prior for a constant failure rates is described in [BR8].

### Bayesian updating with Physics of Failure methods

Bayesian updating can also be used in combination with Physics of Failure methods, e.g. the structural reliability methods discussed in Section 6.5. Different applications are possible in this context:

Bayesian updating of basic variable distributions that are required for an appropriate uncertainty quantification in Physics of Failure approaches (Section 6.5.2).

Bayesian updating of reliability estimates derived from the Physics of Failure, using failure in a combined approach (Section 6.6)

The first application considers the distribution of physical variables, such as loads and material characteristics, which can be directly or indirectly observed during tests or operations. The selected probabilistic models should be updated whenever new data becomes available. The approach for updating of these basic variable distribution follows the same principles as discussed above. Also in this context, the prior may be defined based on alternative data sources, literature values, handbook data or expert judgement.

A different situation occurs if a data sample with information on failures and successes (e.g. individual time-to-failure records for several items) is used to update a prior reliability estimate that has been derived using Physics of Failure methods. The advantage of this approach is that several sources of information can be combined consistently, allowing to make use of limited data samples which would not be sufficient for a pure statistical approach. The details of Bayesian updating for a prior derived from structural reliability methods is discussed in Chapter 10 (Part III) for Mechanical reliability prediction, including also some examples illustrating the use of this combined approach in practice. However, the basic principles and mathematical formulas are of course applicable also outside the mechanical domain.

# Uncertainties in the prediction results

Katharina Fischer, Matrisk GmbH

Goal of this chapter: To discuss different types of uncertainties in the prediction that may lead to a scatter or bias in the predicted reliability estimates, and to explain the consideration of these uncertainties in the present handbook.

## Uncertainties of reliability estimates based on Part III models

Unless otherwise stated, the prediction models provided in Part III of the handbook aim at providing realistic estimates, similar to average values observed in orbit. Nevertheless, the predictions will never perfectly match reality. It is the purpose of the present chapter to discuss various sources of uncertainties affecting the prediction results and how these are considered in the methodology. Section 8.3.2 at the end of this Chapter provides a practical approach that can be used for the identification of the most important uncertainties associated with a prediction, which should be communicated together with the prediction result.

Obviously, aiming at realistic estimates also requires realistic model input. Assumptions made during model usage should be clearly marked and justified by the user. Model usage uncertainties will also be discussed in the following

## Classification and relevance of different prediction uncertainties

Apart from the fact that Reliability Prediction is always dealing with the modelling of uncertainties – a system or component may fail or not – reliability in itself can never be predicted with 100% accuracy. The models used are just a mathematical representation of reality, and the resulting estimates are generally associated with (prediction) uncertainties.

Very broadly, we may thus distinguish two different types of uncertainties:

- Aleatory uncertainties, resulting from the natural variability of the underlying processes

- Epistemic uncertainties, resulting from our poor understanding of the reliability problem

In contrast to the aleatory uncertainties, which can be modelled but not reduced, epistemic uncertainties can be reduced by collecting more information.

The modelling of aleatory uncertainties is the core of reliability prediction, assuming e.g. that failures may occur at random points in time, which can be represented by suitable probabilistic models. Based on these models, the reliability (probability of success) is estimated in relation to a certain period of time and the associated operational and environmental conditions. However, also the reliability estimates are generally uncertain, representing a second layer of uncertainties. The consideration of these epistemic uncertainties is discussed in the following.

### Epistemic uncertainties classification

Epistemic uncertainties may be further classified as follows:

Statistical uncertainties, resulting from limited data samples used to derive a model

Model uncertainties, resulting from modelling assumptions and simplifications made

Another way of classification is by the modelling step or person introducing the uncertainties:

Model development uncertainties are introduced by the model developer

Model usage uncertainties are introduced by the model user (e.g. model input assumptions)

Statistical uncertainties are generally introduced during model development, when a limited data sample is used e.g. to estimate a reliability metric such as a failure rate. Model uncertainties can be introduced both during model development and during model usage.

In the following, the focus will first be on the modelling and propagation of statistical uncertainties, showing their limited relevance for the uncertainty of system level reliability estimates (Section 8.2.2). After this, a broader perspective on epistemic uncertainties is taken, including also the more general model uncertainties (Section 8.2.3).

### The role of statistical uncertainties in system level reliability prediction

Statistical uncertainties arise whenever a reliability metric (e.g. a failure rate) is estimated from statistical data. The uncertainty resulting from the inherent randomness in the data sample is largest when the sample size is very small and decreases when more data gets available; in the theoretical limit (sample size n) the estimate converges to the true value.

Quantification of statistical uncertainties

The quantification of statistical uncertainties during the process of model development is in principle straight-forward, taking its basis in the mathematical theory of statistical estimation. The concept of estimation by interval is illustrated in Figure 8-1, using the example of a Gamma distributed failure rate λ. Different methods from frequentist or Bayesian statistics can be used to derive an estimation interval [λlo; λup] which will contain the true value with a probability – or confidence – of =1-. The width of the interval depends on the value chosen for , but also on the size of the sample used for the estimation (e.g. number of observed failures, cumulated hours).

Figure 8-1: Estimation by interval illustrated with a Gamma distribution for the failure rate λ.

The following methods are worth mentioning in this context, see Section 6.4.3 for discussion:

Confidence intervals, e.g. Chi-Square estimator for constant failure rates

Maximum Likelihood Estimation, large sample theory with Fisher Information matrix to derive the variance around the mean (defined as the Maximum Likelihood estimate).

Bayesian credibility intervals, combining Likelihood information from the data sample with a prior distribution, e.g. a conjugate Gamma prior for constant failure rate estimation.

Uncertainty propagation in system level reliability prediction

Statistical uncertainties are introduced during the development of elementary reliability models using statistical methods, used to predict the reliability of parts, assemblies or in some cases for higher levels such as for standardized equipment. In system level reliability prediction, many of these estimates are combined using system level modelling techniques such as Reliability Block Diagrams or Fault Tree Analysis. While it is in principle possible to propagate the statistical uncertainties using e.g. Monte Carlo simulations, it is not meaningful to do so. Assuming independent and unbiased estimates, statistical uncertainty will rapidly decrease at system level due to the Law of Large Numbers. Only uncertainties associated with reliability estimates for items having a very large contribution to the system level (un-)reliability may have a relevant impact and should be highlighted when discussing the uncertainties associated with the prediction.

From a system perspective this implies that statistical uncertainties associated with elementary reliability models should be addressed (only) under the following conditions:

The components or spacecraft elements considered have a large (>5-10%) contribution to the overall system unreliability and

The sample size used for statistical modelling was very small (<10 failures).

When considering epistemic uncertainties more generally (as in Section 8.2.3 below), it may be sufficient to simply highlight the elementary reliability models fulfilling these conditions, indicating that the effect of statistical uncertainties may not be negligible from a system level perspective.

### A more general view on epistemic uncertainties

As has been discussed in Section 8.2.2 above, statistical uncertainties due to limited data samples underlying the different elementary reliability estimates are of limited relevance from a system level point of view. The main reason for this conclusion is that statistical uncertainties associated with different elementary reliability models are statistically independent and therefore uncertainty reduces at system level due to the Law of Large Numbers.

When considering model uncertainties instead, arising from assumptions and simplifications made during the model development and usage, the assumption of statistical independence across elementary reliability estimates must not hold true. To give an example, uncertainties associated with the mission profile definition at spacecraft level are generally flown down to all lower levels, and thus in principle affect all elementary models in the same (or at least a similar) way. As a result, the uncertainties associated with lower level estimates are not independent anymore and the Law of Large Numbers does not apply.

In conclusion, when considering epistemic uncertainties more generally, model uncertainties are potentially much more relevant from a system level reliability prediction point of view than statistical uncertainties, whose effect is decreased by statistical independence and the Law of Large Numbers. Thus, to assess the uncertainties associated with a reliability prediction, it is important to consider all kinds of epistemic uncertainties introduced during the development of usage of the different reliability prediction models.

In such a general setting, it makes sense to switch from a quantitative treatment to a qualitative assessment of epistemic uncertainties. Nevertheless, a quantitative way of thinking is still relevant to decide which uncertainties are the most relevant ones for a system level prediction. From the considerations discussed above, we may conclude that the “system level relevance” ranking of various uncertainties associated with a prediction should be driven by the following considerations:

What is the contribution of each model to the overall system level (un-)reliability? 
The models with the largest contribution are also likely to have a large contribution to the overall epistemic uncertainty associated with the prediction.

At which level have the various uncertainties been introduced?
Uncertainty sources at higher levels (e.g. mission profile definition at system level) have a larger contribution to the overall epistemic uncertainties than uncertainties associated with part level reliability predictions.

What is the dependency structure between various uncertainty sources?
Uncertainties that are statistically dependent (e.g. the same assumption is used for various elementary reliability models) have a stronger effect than independent uncertainty sources.

What is the order of magnitude of each relevant epistemic uncertainty?
From the uncertainty sources judged to be most relevant from a system level point of view (based on the previous criteria), those leading to the largest scatter will have the largest impact on the system level reliability prediction uncertainty.

Section 8.3 contains a questionnaire approach which can be used to identify different sources of epistemic uncertainties associated with a reliability prediction model or method and its usage. For practical purposes, a full discussion and consideration of all identified uncertainties is not necessarily relevant and may even be considered as excessive information if the goal is to understand the main uncertainties associated with a prediction at system level. Instead, the focus should be on a few uncertainties having the largest impact. A rough qualitative ranking to identify e.g. the Top 5 uncertainty drivers can be established based on the criteria listed above.

## Identification of relevant uncertainties associated with a prediction

To assess the uncertainties associated with a prediction model or method and its usage, as a first step the relevant sources of uncertainties need to be identified. This can be achieved with the aid of the questionnaires given in the following.

Based on the discussion in Section 8.2, the identification of uncertainty sources will not be limited to statistical uncertainties, but considering epistemic uncertainties in a more general sense. Statistical uncertainties (focussing on those still relevant at system level) are considered only as one potential uncertainty source and the distinction between statistical and model uncertainties will not be stressed any further. Instead, the questionnaires will distinguish between model development uncertainties (including statistical uncertainties) and model usage uncertainties.

Answers to the questionnaires assessing the model development uncertainties associated with the models used in Part III of this handbook are collected in Annex II.B.

### Model development uncertainties

Model development uncertainties are introduced by the model developer and are thus inherent in the models. At least the most important ones should be communicated together with the model; the questionnaires given below can be used for this purpose. The model user has generally no influence on the magnitude of the model development uncertainties unless there is the possibility to update the model, e.g. with the aid of new data (Section 6.7).

The questionnaires are given for two different methods or modelling approaches based on which reliability prediction models can be developed: Statistical methods based on failure data collected in the field or in tests (Table 8-1) and Physics of Failure approaches based on a dedicated modelling of the underlying failure mechanisms (Table 8-2).

The classification of methods provided in Section 6.3 includes a third category, combining Physics of Failure methods with statistical data. The uncertainties associated with these approaches should be assessed with the aid of a single questionnaire (for statistical or Physics of Failure methods) if the modelling is dominated by one of the two methods. Otherwise, both questionnaires should be filled out. The answers given to each question should indicate how potential uncertainties and weaknesses are alleviated with the aid of the combined approach.

Models from handbooks and published data sources are assessed using the same approach. Questions that cannot be clearly answered based on the published information should be classified as “Unknown”.

Table 8-1: Questionnaire to identify sources of uncertainties during the development of elementary reliability prediction models based on statistical approaches.

Table 8-2: Questionnaire to identify sources of uncertainties during the development of elementary reliability prediction models based on Physics of Failure approaches.

### Model usage uncertainties

Model usage uncertainties are introduced by the model user, e.g. when making assumptions for the model input. Also in this case the relevant sources of uncertainties should be identified and the most important ones (having the largest impact on the prediction) should be communicated together with the prediction. The questionnaire given in Table 8-3 can be used for this purpose.

Table 8-3: Questionnaire to identify sources of uncertainties during reliability prediction model usage at elementary and system level.

The questionnaire should be used at the level of the system for which the prediction is made, e.g. for a single equipment, a payload, a platform or a whole spacecraft. The first part of the questionnaire relates to system level modelling uncertainties, including questions to identify major uncertainties associated with the model input from lower levels. The latter can be assessed by applying the same questionnaire to each component (if it is a system in itself), or the second part of Table 8-3 if the lower level input is based on elementary reliability prediction models.

For an efficient usage of the questionnaire in Table 8-3, in consistency with the criteria discussed in Section 8.2.3, a step-wise iterative procedure may be followed for the uncertainty assessment:

Apply the first part of the questionnaire at the highest level of assembly considered

  - Relevant uncertainty sources introduced at system level are likely to have a larger impact on the prediction that uncertainties introduced at lower levels.

  - The last question in Section 1.2 of the questionnaire, related to the uncertainties inherent in predictions at lower levels, may remain unanswered in the first iteration. To proceed with the next steps, it is only relevant to identify the components (systems or parts) having the largest contribution to overall unreliability

Apply the questionnaire to assess the uncertainties associated with predictions at the next lower level, focusing on the components having the largest contribution

  - Use the first part of the questionnaire if the component is modelled using system-level reliability prediction methods

  - Use the second part of the questionnaire if the component is modelled using elementary reliability models

  - Uncertainties associated with predictions that do not have a large contribution to system level unreliability do not have to be considered in detail.

Go further down in the system hierarchy to assess uncertainties associated with predictions at lower levels, as necessary.

  - The number of “dominating” components considered may be reduced in each step

  - Component models may be grouped if the same modelling approach is used for a variety of parts (e.g. models for EEE components taken from the same source).

Summarize the uncertainty assessment by making a (qualitative) statement about the major (e.g. Top 5) uncertainties associated with the predictions at the highest level of assembly.

  - The criteria discussed in Section 8.2.3 may be used to decide which uncertainty sources are most relevant at system level.

When following this procedure, some uncertainties introduced at the highest level of assembly considered may in fact be flown down to all lower levels. These uncertainties are likely to dominate the system level reliability prediction. Large uncertainties introduced at lower levels may become relevant if the contribution of the item’s probability of failure to system level unreliability is very large. It should be noted that this becomes more and more unlikely the further the assessment goes down in the system hierarchy. Going into the details of each part level model is thus certainly not necessary to assess the major uncertainties associated with the prediction.

Probability distributions

In the following, the parametrization of the probability distributions introduced in Section 6.4 (Model development: Statistical methods) and used in Chapter 7 (Bayesian methods) is given as a common reference.

Exponential distribution

| fX(x)= λ∙exp(-λx) FXx= 1-exp-λx | X≥0 λ>0 | EX=1λ VarX=1λ |
| --- | --- | --- |

Weibull distribution

| fX(x)= βα∙xαβ-1exp-xαβ FXx= 1-exp-xαβ | X≥0 α>0 β>0 |  | EX=α∙Γ1+1β VarX=α∙Γ1+2β-Γ21+1β β≈VarXEX-1.09 |
| --- | --- | --- | --- |
| The mean approximation bias of β≈vx-1.09 is shown in the diagram to the right. For coefficients of variation 0.05≤vX≤1.35, the approximation error is smaller then ±5%. |  |  |  |  |

Gamma distribution

| fX(x)= βαΓα∙xα-1∙exp(-βx) FXx=Γα,βxΓα | X≥0 α>0 β>0 | EX=αβ VarX=αβ |
| --- | --- | --- |

Gamma function: Γα=0∞tα-1exp-tdt

Lower incomplete gamma function: Γα,βx=0βxtα-1exp-tdt

Normal distribution

| fX(x)= 1σ2π∙exp-12x-μσ2 FXx= Φx-μσ | X∈R μ∈R σ>0 | EX=μ VarX=σ |
| --- | --- | --- |

Standard Normal distribution: Φx= 12π∙-∞xexp-t22dt

Lognormal distribution

| fX(x)= 1σ2π∙exp-12lnx-μσ2 FXx= Φlnx-μσ | X≥0 μ∈R σ>0 | EX=expμ+σ22 VarX=expμ+σ22expσ2-1 |
| --- | --- | --- |

Geometric distribution

| pXx=p∙1-px-1 PXx=1-1-px | X=1,2,3,… 0<p<1 | EX=1/p VarX=1-pp |
| --- | --- | --- |

Binomial distribution

| pXX= nxpx1-pn-x PXx=i=0xnipi1-pn-i | X=1,2,…n 0<p<1 | EX=np VarX=np1-p |
| --- | --- | --- |

Beta distribution

| fXx=1Bα,βxα-11-xβ-1 fXx=Bx;α,βBα,β | 0≤X≤1 α>0 β>0 | EX=αα+β VarX=1α+β∙αβα+β+1 |
| --- | --- | --- |

Beta function: Bα,β=Γα∙Γβ/Γα+β

Incomplete beta function: Bx;α,β=0xtα-11-tβ-1dt

Inverse Gamma distribution

| fX(x)= βαΓα∙exp(-βx)xα+1 FXx=Γα,β/xΓα | X≥0 α>0 β>0 | EX=βα-1 VarX=βα-1∙α-2 |
| --- | --- | --- |

Prediction uncertainties for Part III models

In the following, the model development uncertainties associated with the prediction models presented in Part III of this handbook are summarized, using the format proposed in Section 8.3.1. Note that the uncertainty of the prediction results depends not only on the uncertainties inherent in the model, but also on the use of these models, see Section 8.3.2 for details.

Model development uncertainties associated with the EEE models

The models provided in Chapter 9 (Part III) are based on the FIDES [RD10]  approach, making use of a combined method with base failure rates derived from statistics (field return from on-ground applications) and scaling factors derived from the Physics of Failure. Table 8-4 and Table 8-5 summarize the uncertainties associated with these two modelling aspects.

Model development uncertainties associated with the Mechanical models

Besides providing own models based on Structural Reliability Methods, Chapter 10 (Part III) makes reference also to external data sources such as the NPRD [RD14]  and the NSWC [RD15] , which is based on a combined method using both statistics and Physics of Failure. The limitations of these data sources when used for space applications are discussed more in detail in Chapter 10 (Part III). Here, only a quick overview is given using the common checklist format for the assessment of model development uncertainties from Section 8.3.1.

Model development uncertainties associated with the models for Miscellaneous items

The “standard” model for Miscellaneous items, as defined in Chapter 11 (Part III), has been developed based on IOR for an equivalent class of items. Model development uncertainties associated with this approach are listed in Table 8-10. For “non-standard” items only a general process for model development is provided and the model development uncertainties have to be assessed by the user.

Table 8-4: Model uncertainties associated with the statistical modelling behind the EEE component models (base failure rates).

Table 8-5: Model uncertainties associated with the PoF modelling behind the EEE component models (acceleration factors).

Table 8-6: Uncertainties associated with the mechanical reliability models based on (simplified) structural reliability methods (Physics of Failure).

Table 8-7: Model uncertainties associated with the NPRD [RD14]  when considered as a data source for Mechanical reliability prediction in space applications.

Table 8-8: Model uncertainties associated with the statistical modelling behind the NSWC [RD15]  models when considered as a data source for Mechanical reliability prediction in space applications.

Table 8-9: Model uncertainties associated with the PoF modelling behind the NSWC [RD15]  models when considered as a data source for Mechanical reliability prediction in space applications.

Table 8-10: Model uncertainties associated with the “standard model” for Miscellaneous items.

Part II Bibliography

Bibliographic references for Part II

Rome Laboratory Reliability Engineer’s toolkit – An application Oriented Guide for the Practicing Reliability Engineer
The Rome Laboratory, April 1993

Failure mode / mechanism distributions 
FMD-2016, Quanterion

Handbook of Parameter Estimation for Probabilistic Risk Assessment. 
NUREG/CR-6823 / SAND2003-3348P, Sandia National Laboratories, 2003.

NIST/SEMATECH Engineering Statistics Handbook – e-Handbook of Statistical Methods
http://www.itl.nist.gov/div898/handbook/, accessed 08.04.2020

Mechanical System Reliability and Cost Integration Using a Sequential Linear Approximation Method.
Kowal, M. T. (1997); NASA Contractor Report.

Design Space Decomposition using Support Vector Machines for Reliability-based Design Optimization
Harrison Sanchez A., Missoum S. and Martinez Teuscher J.P. (2006); 11th AIAA/ISSMO Multidisciplinary Analysis and Optimization Conference; AIAA 2006-6927

Meta-models for structural reliability and uncertainty quantification
Sudret, B. (2012). http://arxiv.org/abs/1203.2062.

Methods of Structural Safety 
Madsen, H.O., Krenk, S. and Lind, N.C. (1986); Prentice-Hall, New Jersey

Structural Reliability Methods 
Ditlevsen, O., and Madsen, H.O. (2007) 
http://od-website.dk/books/OD-HOM-StrucRelMeth-Ed2.3.7.pdf, accessed 08.04.2020

Structural Reliability Analysis and Prediction 
Melchers, E. R. (1999); John Wiley and Sons, England

STRUREL System of Programs for Probabilistic Reliability Analysis
RCP Consult GmbH, www.strurel.de, accessed 08.04.2020

NESSUS Software
Southwest Research Institute, www.nessus.com, accessed 08.04.2020

UQLab - MATLAB®-based Uncertainty Quantification framework
www.uqlab.com, accessed 08.04.2020

FERUM - Finite Element Reliability Using Matlab, 
www.sigma-clermont.fr/en/ferum, accessed 08.04.2020

OpenTURNS: An industrial software for uncertainty quantification in simulation
www.openturns.org, accessed 08.04.2020

Variance Reduction Techniques in Monte Carlo Methods. 
Kleijnen, J.P.C., Ridder A. and Rubinstein R. (2010); SSRN Scholarly Paper

Information theory and an extension of the maximum likelihood principle,
Akaike, H. (1973), 2nd International Symposium on Information Theory, 1971
Republished in Kotz, S.; Johnson, N. L., eds. (1992), Breakthroughs in Statistics, pp. 610–624.

An invariant form for the prior probability in estimation problems. 
Jeffreys, H. (1946); Proc. Roy. Soc. A., 186, 453-461.

Bayesian Theory
Bernardo J and Smith A (1994); John Wiley & Sons.

A Compendium of Conjugate Priors,
Fink, D. (1997)

Bayesian Data Analysis. 3nd ed. 
Gelman, Andrew (2013); Texts in Statistical Science, Chapman & Hall.

The BUGS project. 
http://www.mrc-bsu.cam.ac.uk/software/bugs/. Accessed 08.04.2020

JAGS – Just another Gibbs Sampler. 
http://mcmc-jags.sourceforge.net/. Accessed 08.04.2020

Stan: Project homepage. 
http://mc-stan.org/index.html. Accessed 08.04.2020

Bayesian Inference for NASA Probabilistic Risk and Reliability Analysis, 
Dezfuli H., Kelly D., Smith C., Vedros K., and Galyean W. (2009); NASA/SP-2009-569

New FIDES Models For Emerging Technologies 
Carton P., Giraudeau M., Davenel F.; RAMS Orlando, 2017

Development of an expert judgment elicitation and calibration methodology for risk analysis in conceptual vehicle desing. 
Unal, R., Keating, C., Conway, B., Chytka, T., & Lepsch, R. (2004); NASA NCC- 1-02044

Statistical methods for eliciting probability distributions, 
Garthwaite, P. H., Kadane, J. B., & O’Hagan, A. (2005)

NUREG: Recommendations for Probabilistic Seismic Hazard Analysis: Guidance on Uncertainty and Use of Experts,
NUREG/CR-6372 / UCRL-ID-122160 Vol 1; Lawrence Livermore National Laboratory (1997)

A web-based tool for eliciting probability distributions from experts, 
Morris D.E., Oakley J.E., Crowe J.A. (2014); 
Environmental Modelling & Software, Volume 52, Pages 1-4, 2014 http://optics.eee.nottingham.ac.uk/match/uncertainty.php. Accessed 08.04.2020

Reference Documents

The following list is repeated from the Handbook Preface, using the same numbering of Reference Documents across all Parts.

- Space product assurance – Dependability
ECSS-Q-ST-30C Rev.1, February 2017

- Space product assurance – Failure modes, effects (and criticality) analysis (FMEA/FMECA)
ECSS-Q-ST-30-02C, 6 March 2009

- Space Engineering -– Fault Tree Analysis 
ECSS-Q-ST-40-12C, 31 July 2008

- Space product assurance – Components reliability data sources and their use
ECSS-Q-HB-30-08A

- Space engineering – Structural general requirements
ECSS-E-ST-32C Rev. 1

- Space engineering - Structural factors of safety for spaceflight hardware
ECSS-E-ST-32-10C Rev. 1

- Space engineering – Mechanisms
ECSS-E-ST-33-01C Rev. 2

- ECSS system – Glossary of terms
ECSS-S-ST-00-01C

- Standard Framework for Reliability Prediction of Hardware
IEEE Std 1413TM-2010

- Reliability Methodology for Electronic Systems
UTE C80-811, FIDES Guide 2009 Ed A Rev 2010

- Reliability Prediction Models (Quanterion),
HDBK-217Plus:2015 Notice 1 – Handbook of 217PlusTM

- Reliability prediction of Electronic Equipment
MIL-HDBK-217F Notice 2 (1995)

- Electronic Reliability Design Handbook, Dept. Defence. 
MIL-HDBK-338B (1988)

- Nonelectronic Parts Reliability Data (Quanterion)
NPRD-16 (2016)

- Handbook of Reliability Prediction Procedures for Mechanical Equipment
NSWC-11 (Naval Surface Warfare Center, 2011)

- Web-Accessible Repository of Physics-based Models (RiAC)
http://www.theriac.org/WARP/index.php, accessed 08.04.2020

- TEC-QQD Study – Reliability Prediction Data Sources and Methodologies for Space Applications
Final Report, ADS-RPDSM-FR, dated 23/8/2016

- TEC-QQD Study – Reliability Prediction of Mechanical Systems & Parts
Final Report, AO/1-8017/14/NL/KM, dated 26.08.2016

- TEC-QQD Study – Use of quantitative reliability requirements for space applications
Final Report, TASF-RAMS-0010, dated 18Feb16