Annex

A	Development and verification of the reliability.space methodology	A-3

A.1	Overview on the handbook development and verification	A-3

A.2	List of Technical Notes prepared in the study	A-5

A.3	Development of the reliability.space methodology	A-6

A.4	Verification of the reliability.space methodology	A-9

A.5	Identified shortcomings of the reliability.space methodology	A-16

B	Annex Bibliography	B-19

C	List of Changes	C-21

###### Development and verification of the reliability.space methodology

The goal of this annex is to provide short summary information on the development and verification of the reliability.space methodology and handbook, and on the gaps and open questions identified during its verification.

Overview on the handbook development and verification

The reliability.space handbook is the final outcome of the study “New Reliability Prediction Methodology Aimed at Space Applications” which was started in 2017 under a programme of and funded by the European Space Agency. The objective of the study was the development of a new reliability prediction methodology dedicated to space, aiming to overcome the limitations and shortcomings of the approaches used in practice before. The outcome of the study, i.e. this handbook, will serve as an input for the development of future ECSS handbooks and standards.

The study was structured in seven tasks, whose primary objectives are briefly summarized below. An overview of the Technical Notes prepared for the study and their interrelation is given in Figure A-1.

- Task 1 was to investigate the limitations and shortcomings of the existing RP methodologies used in the space industry (TN-1).

- Task 2 aimed at the development of an approach for improvements of the in-use RP methodology to overcome the identified limitations and shortcomings (TN-2).

- Task 3 was to define the requirements and ground rules for the new RP methodology (TN-3).

- Task 4 was the formulation of criteria and requirements for the verification of the new RP methodology (TN-4).

- Task 5 was the development of the new reliability prediction methodology (TN-5).

- Task 6 aimed at a proof of concept for the new reliability prediction methodology by application to a real case study (TN-6), leading to an update (TN-7) of the methodology developed during Task 5.

- Task 7 was the preparation of the final reliability.space handbook (TN-8) based on the material summarized in the previous notes.

Figure A-1: Overview on the content and interrelation of the Technical Notes.

Before the end of Task 5, a workshop was held at ESTEC with relevant stakeholders from the space industry. The workshop objectives were the following:

- to share the status and progress of the study and give an overview on the ongoing work

- to discuss choices made during the development and underlying assumptions

- to get feedback at an intermediate step during the progress of the study

The discussions during the workshop showed that the development of the new reliability prediction methodology was progressing well and was generally going in the right direction. The detailed discussions and comments were considered during the finalization of Task 5.

A list of all Technical Notes prepared in the study is given in Annex A.2 below. Details on the main development and verification tasks are given in Annex A.3 and A.4.

List of Technical Notes prepared in the study

The following Technical Notes were prepared during the study:

New Reliability Prediction Methodology Aimed at Space Applications – Limitations of the in-use reliability prediction methodologies. TN-01 Rev. B, 8.12.2017

New Reliability Prediction Methodology Aimed at Space Applications - Approach for the development of the new reliability prediction methodology. TN-02 Rev. B, 19.12.2017

New Reliability Prediction Methodology Aimed at Space Applications – Requirement specifications and ground rules for the development of the new reliability prediction methodology for space applications. TN-03 Rev. B, 25.5.2018

New Reliability Prediction Methodology Aimed at Space Applications – Criteria and requirements for the verification of the new reliability prediction methodology for space applications. TN-04 Rev. C, 20.6.2018

New Reliability Prediction Methodology Aimed at Space Applications – Development of the new reliability prediction methodology for space applications (preliminary). TN-05 Rev. B, 25.11.2019

New Reliability Prediction Methodology Aimed at Space Applications – Proof of concept of the new reliability prediction methodology. TN-06 Rev. B, 13.02.2020

New Reliability Prediction Methodology Aimed at Space Applications – Development of the new reliability prediction methodology for space applications (final). TN-07 Rev. B, 02.10.2020

Development of the reliability.space methodology

The technical background of the methodology is documented mainly in [TN5] and its update ([TN7]) after the proof of concept. Aiming at readability from a user’s perspective, the final handbook ([TN8]) does not contain all background information provided in these two Technical Notes. Some summary information on the development of the methodology for EEE, Mechanical, Miscellaneous and System level reliability prediction is given in the following.

EEE components and assemblies

For the modelling of EEE components, the FIDES [RD10] methodology was used as a basis. To improve its applicability in space, it has been customized as follows:

- Description of the methodology and process to be followed for the definition of a mission profile considering the environmental and use conditions in space applications, with example mission profiles provided for GEO and LEO missions.

- Recommendations for the modelling of different component families and technologies listed in the European Preferred Parts List (EPPL). This includes:

  - recommendations for technologies not covered by FIDES

  - recommendations for innovative technologies and FIDES model updates based on an ongoing study led by the French Ministry of Defence (PISTIS study)

- Space customization of quality related factors, accounting for the specific manufacturer and component quality assurance processes followed in the space industry.

- Space customization and simplification of factors related to the component’s placement and usage, considering the specific aspects of space missions.

- Adaptation and simplification of the audit used to derive the process factor with suggested answers for each audit question based on the typical processes followed in space.

In addition to this space customization of FIDES [RD10] , a dedicated discussion of systematic, wear-out and extrinsic failures shows how these root causes can be covered, if needed, depending on the use of the reliability prediction.

Mechanical parts and assemblies

The approach for Mechanical items takes basis in a previous ESA study investigating the use of different inputs and methods for the assessment of mechanical reliability [RD18] . The credibility and space applicability of existing mechanical reliability data sources is low, which is why a strong effort was put into the identification and modelling of relevant failure mechanisms and their probabilistic representation in a Physics of Failure based reliability prediction approach. To limit the efforts required for the assessment of mechanical items based on these methods, a process was defined starting with the identification of the dominating failure mechanisms (to support the decision what should be modelled), for which then suitable methods are selected to make best use of the available data and engineering knowledge (i.e. providing guidance on how the dominating failure mechanism should be modelled).

The use of structural reliability methods beyond the pure structural domain is the core of the methods proposed for mechanical domain. Limit state functions and simplified analytic models are proposed for several failure mechanism, including different types of mechanical wear, fracture/fatigue, and radiation degradation. These models can be applied to any item or technology for which the considered failure mechanism is of relevance, making use of design information enhanced by a probabilistic modelling of the uncertainties associated with the different input variables.

Finally, an approach for Bayesian updating with test data or in-orbit return has been developed, allowing to combine a prior model based on the Physics of Failure with observations. This combination of different inputs is highly effective to improve the prediction with the aid of new or existing data, even if the sample size of the data is too small for a pure statistical analysis.

Miscellaneous items

The category of Miscellaneous items includes mainly parts and assemblies that are difficult to classify into one of the two previous categories, but also items that are either EEE or Mechanical by nature but difficult to model with the domain specific methods. The process proposed for reliability prediction of these items is a step-wise procedure starting with the selection of an initial model (e.g. base failure rate or probability of failure for one-shot items) which is then tuned to the specific technology and application and updated with available data using a Bayesian approach. Failures due to wear-out, systematic and extrinsic root causes are considered separately.

In-orbit return from the fleets of the two satellite prime contractors ADS and TAS was collected and merged, allowing to provide reliability data for a list of standardized miscellaneous items typically used in satellite missions. A process for the derivation of similar data is defined for all items for which this approach has not been successful, e.g. because of insufficient sample size. This process can also be followed for items that are not standardized, prohibiting the use of In-orbit return as the main input for modelling.

System level reliability prediction

The existing approaches for system reliability prediction were generally considered sound and did not require any new developments during the study. An overview on the different existing methods is given, highlighting their strengths and limitations to provide guidance for method selection. In addition to this, the following aspects of system level reliability prediction were discussed specifically due to their relevance for space applications:

- Reliability assessment of phased missions to consider the sequence of relevant mission phases during the life of a space system

- Importance measures to direct efforts for reliability improvement to the components having the largest effect on system level reliability

- Reliability assessment of degraded systems to consider different levels of performance degradation as an input for risk assessment

- Quantitative modelling of common cause failures, e.g. resulting from systematic or extrinsic failure root causes

- System level considerations of wear-out with different models for wear-out modelling at component level

For the quantification of systematic failures, a model at satellite level has been developed based on major anomalies observed during the operation of satellites in-orbit. The model also allows a break-down of the overall systematic anomaly rate by sub-system and/or severity (most anomalies having no impact on the mission).

Software failures can be considered as a special case of systematic failures, the largest concern being errors related to the requirement specification and/or the interface between hard- and software in a mixed system. Existing software reliability models focus on software development errors (assuming no errors related to the software specification) and do not provide reliable results for the low failure rates required for space applications. Therefore, the use of these existing models in the context of reliability prediction is not recommended. Note that in most of the times a full recovery from software errors is possible by uploading patches while the satellite is in safe mode.

Verification of the reliability.space methodology

As indicated in Figure A-1, the development process of the reliability.space methodology also included a proof of concept, which was based on the preliminary version of the methodology described in [TN5]. The results of this proof of concept are documented in [TN6], and have led to a planned update of the methodology in [TN7].

In the following, a brief summary of the main verification results is provided.

Proof of concept strategy

For the quantitative proof of concept, two real (or at least realistic) space missions – a GEO telecommunication satellite and a LEO Earth Observation mission – were selected, for which the following reliability estimates could be derived and compared:

- Predicted reliability derived with the new methodology

- Predicted reliability derived with existing methodologies

- Achieved reliability derived from In Orbit Return data

In this comparison, the in-orbit achieved reliability is certainly the most important benchmark for the new methodology, at least when sufficient IOR is available to derive reliable estimates. The achieved reliability was estimated using IOR data from the satellite fleets of the two involved satellite prime contractors, Airbus DS and Thales Alenia Space. To maximize the IOR availability for the benchmarking, the selected missions had to be old ones, avoiding new technologies for which IOR is not sufficient yet. At the same time, the missions had to be recent enough to allow automatic extraction of the required input data such as e.g. parts lists for the considered satellite equipment. Other criteria for mission selection were the coverage of different reliability prediction models, as well as the range of environments, use conditions, technologies and applications covered.

The benchmarking between predicted and achieved reliability was performed at equipment level. At this level, the in-orbit cumulated hours could be maximized by using data from a “reference fleet” of satellites using the same equipment under comparable conditions.

The following existing methodologies were considered, focussing on reliability prediction for EEE components:

MIL-HDBK-217-F+N1 or N2 [RD12] 
Historically, this standard was the most commonly used reliability prediction methodology in space applications, thus predictions could be taken directly from the CDR reliability predictions of the selected missions. It should be noted that the CDR predictions were not necessarily based on the original MIL-HDBK-217-F, but on industry tailored versions to allow for technology extensions, and to improve the accuracy of the predictions compared to IOR.

FIDES Guide 2009 Ed A Rev 2010 [RD10] 
The FIDES methodology was used as the basis for the development of the reliability.space methodology for EEE reliability prediction, making it irrelevant as a benchmark for the proof of concept. Predictions with the original FIDES Guide 2009 were made for platform units and could be used as a reference to address specific aspects of the space customization. In the following, these results are not addressed any further.

217Plus:2015 Notice 1 [RD11] 
For one platform unit, another benchmark was made based on the 217Plus methodology, allowing a detailed evaluation also of this alternative prediction methodology.

The quantitative results of the benchmarking between methodologies and with IOR achieved reliability are summarized in the following. In addition, a qualitative evaluation of the different methodologies from a user’s perspective was performed, which will also be summarized below.

Quantitative benchmarking results

In the following, the main results of the quantitative proof of concept are summarized, focussing on reliability prediction for EEE units where comparison with alternative methodologies and IOR was possible. For Mechanical items, the information available from the documentation of the old missions considered was not sufficient to derive a meaningful benchmark. For Miscellaneous items, alternative methodologies are missing and the models proposed in reliability.space are based on the same IOR that could be used for benchmarking, making a stringent (quantitative) IOR verification impossible.

Figure A-2 and Figure A-3 show the main results of the quantitative benchmarking performed at unit level for the platform and the telecommunication payload of a satellite in GEO orbit. Only EEE units with sufficient IOR to perform the benchmarking are shown in these figures. The orange bars correspond to the predictions based on the preliminary version of the new reliability prediction methodology as described in [TN5], the blue bars show the predictions made for the CDR (mainly based on MIL-HDBK-217F [RD12] ) when the satellite was built.

The figures present relative numbers, derived as the predicted failure rate for each unit divided by the IOR failure rate estimate for the same unit. Red colour shows the IOR reference (dotted line at 1.0 in relative figures), and a band representing a factor of 2.0 around this IOR estimate (dashed lines), which was specified in [TN4] as a tentative target for the accuracy of the new methodology. For payload units (Figure A-3), the IOR reference is defined as the point estimate for the failure rate, i.e. number of failures divided by the cumulated hours of units in orbit. For platform units (Figure A-2), there were no failures observed and thus the Chi Square estimator at 60% Level of Confidence was used as IOR reference. To illustrate the uncertainty associated with the benchmarking, IOR estimators at 60% and 90% Level of Confidence are shown in black, using the same symbols and estimators in both figures.

The platform results in Figure A-2 show a clear trend of original CDR predictions being pessimistic, and considerable improvements of accuracy when switching to the new methodology. For the On Board Computer, a prediction with 217Plus [RD11] was made as well, being almost as pessimistic as the CDR prediction: the 217Plus predicted failure rate was a factor 9.16 higher than the IOR reference.

Figure A-2: Summary of the proof of concept for platform units (GEO telecommunication satellite), predicted failure rates versus IOR Chi Square estimator at 60% Level of Confidence (no failures).

For payload units, the predictions from CDR were already closer to the IOR estimate, leading to smaller improvements when switching to the new methodology. However, the comparison with IOR clearly shows that the new methodology results are reaching the tentative target defined in [TN4].

Figure A-3: Summary of the proof of concept for payload units (GEO telecommunication satellite), predicted failure rates versus IOR point estimate.

The benchmarking results for the LEO Earth Observation platform are shown in Figure A-4. The cumulated hours in orbit for these platform units (no failure observed) were considerably smaller than for GEO satellites, and thus not sufficient to conclude about the accuracy of the predictions: even though from the figure, it may seem that the predictions with the new methodology (and the CDR predictions as well) are rather optimistic, the ratio may be expected to improve with more observations – at least as long as no failure occurs. The comparison between the predictions with the new methodology and those made for CDR shows a similar trend as for platform units in GEO satellites, i.e. failure rates predicted with reliability.space are generally smaller than those derived from MIL-HDBK-217F [RD12] .

Figure A-4: Summary of the proof of concept for platform units (LEO Earth Observation satellite), predicted failure rates versus IOR Chi Square estimator at 60% Level of Confidence (no failures).

For payload units, the in flight experience was not sufficient to assess the unit level failure rate (achieved reliability) in LEO missions.

Results of the qualitative evaluation

In addition to the quantitative benchmarking, a qualitative assessment of the different methodologies was made based on a set of criteria defined in [TN4], i.e. prior to the development of the new methodology. These verification criteria were derived from a discussion of different needs for reliability predictions in space applications. They are formulated as open questions addressing specific aspects of the methodology, which can be grouped under the headings “practicality”, “applicability” and “pertinence” (accuracy) of the prediction methodology. For each criterion, a set of evaluation requirements is specified, allowing to assess the strengths and weaknesses of the different methodologies.

The qualitative assessment results for the new reliability prediction methodology are summarized in Table A-1, addressing the methodology as a whole (i.e. covering the methodology for EEE, Mechanical and Miscellaneous items). For comparison, the results for MIL-HDBK-217-F [RD12]  and 217Plus:2015 [RD11] (both focussing on EEE) are also provided in the table, although it should be noted that the criteria statuses have been derived with a somewhat less rigorous approach than in the case of the reliability.space methodology.

Table A-1: Qualitative assessment results for the different reliability prediction methodologies based on verification criteria defined before the development of the new methodology.

| Groups of verification criteria | Criteria statuses by methodology (“Yes” implies full compliance with a criterion) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | reliability.space |  | MIL-HDBK-217F |  | 217Plus:2015 |  |
| Practicality: 5 Criteria to assess the feasibility and practicality of the new reliability prediction methodology from a model user’s perspective | Yes: Partly: No: | 3/5 2/5 0/5 | Yes: Partly: No: | 4/5 1/5 0/5 | Yes: Partly: No: | 3/5 0/5 2/5 |
| Applicability: 7 Criteria for the area of application and coverage of the new methodology from a space domain perspective | Yes: Partly: No: | 6/7 1/7 0/7 | Yes: Partly: No: | 2/7 1/7 4/7 | Yes: Partly: No: | 2/7 0/7 5/7 |
| Pertinence: 8 Criteria for the relevance of the methodology regarding its scope and foundations, and for the comparison between prediction and reality | Yes: Partly: No: | 5/8 3/8 0/8 | Yes: Partly: No: | 2/8 0/8 6/8 | Yes: Partly: No: | 2/8 0/8 6/8 |

The table generally shows satisfactory assessment results for the new reliability prediction methodology, as only few criteria are assessed to as “partly compliant”. The highlighted drawbacks were caused mainly by the following reasons:

- Practicality: for EEE reliability predictions, the methodology is not fully self-contained, referring to the FIDES 2009 guide [RD10] for detailed models. The reliability.space handbook is seeking a compromise between document size and completeness, which still implies some references to FIDES. Note that the FIDES guide is available for free, and all model equations are explicit both in the handbook and in FIDES.

- Applicability: Despite an improvement achieved compared to other methodologies, the reliability.space methodology does not cover all technologies used in space applications.

- Pertinence: All partial compliances in this group refer to the lack of (publicly available) information on the statistical background of the models, especially those for EEE components. Note that none of the existing methodologies offers a higher level of transparency with respect to the statistical background.

The comparison with the other methodologies shows that the new methodology has led to improvements on all axes. Only in terms of practicality, the simple models provided in MIL-HDBK-217F [RD12]  received a rating that is slightly better, yet at the price of many limitations in terms of applicability and pertinence of the models when used in space applications.

Update of the preliminary methodology

Besides the overall assessment, the proof of concept has also led to a number of detailed recommendations for the methodology update in [TN7].

The updates made for the modelling of EEE components are briefly summarized in the following.

Clarifications and simplifications: To ease the application of the handbook, several clarifications and justified simplifications have been made for various component level models and for different parameters and factors used by all models.

Model updates: The models for the following components have been updated after observing unexpected results during the proof of concept

  - Ceramic capacitors: Classification of Type II ceramic capacitors adapted to account for a well-known weakness of this FIDES model

  - Surface Acoustic Wave filters: Changed categories to achieve coherency with current technologies used in space applications

  - Dual state relays: Use a common value for the base failure rate of mono- and bi-stable relays (bi-stable relays are not covered by FIDES 2009 [RD10] )

Missing models: Some comments regarding missing component models (for Ferrite switches and Thyristors) in the preliminary methodology version could be solved by adding clarifications and/or proposing the use of models for similar technologies. The remaining identified gaps are listed in Section A.5 below.

Simplified methods: Experimentation with the parts count and family count methods proposed by FIDES [RD10] showed inconsistent results that could not be fully explained during the proof of concept of the new methodology. Therefore, the final handbook does not recommend the use of these methods for space applications, proposing the use of the parts stress method with default values when quick preliminary assessments are required.

For Miscellaneous items, the comments from the proof of concept mainly addressed missing models. The following two updates were implemented for the final methodology:

- Dormant failures: A clarification has been added to explain that dormant failures must be modelled by the user considering the specific characteristics of the item under analysis and its conditions of use.

- Amplification channel: A model has been proposed to divide the overall failure rate for the channel (standard model based on IOR) into separate estimates for the Electronic Power Conditioner (EPC), Channel Amplifier (CAMP) and Travelling Wave Tube (TWT).

For Mechanical reliability prediction, due to the lack of quantitative modelling possibilities during the proof of concept, the recommendations have led only to some clarifications in the methodology description.

Identified shortcomings of the reliability.space methodology

After the methodology update described in the previous section, a number of identified shortcomings and gaps remain open in the final reliability.space methodology. In the following, these are briefly summarized together with first proposals for future improvements.

- Missing EEE component models considered in the PISTIS study: the following missing component models may be added after completion of the PISTIS study, an ongoing study led by the French Ministry of Defence which is aiming at an update of the FIDES guide including new models for GaN transistors and MMIC, as well as for DSM components. Updates for the factors accounting for sensitivity to over-stresses foreseen by the PISTIS study should also be assessed and considered for an update of the reliability.space methodology.

- Missing models for optronic and laser components: Optronics are not covered by FIDES 2009 [RD10] nor by most other existing reliability prediction methodologies. Some models for optronic and laser components are proposed in the Telcordia SR-332 [BR35] standard and could be used as a starting point for the modelling of these components. However, an assessment is required whether the Telcordia models are suitable for space applications and provide credible results.

- Missing models for optical elements: Also models for optical components classified as Miscellaneous items (optical fibers and connectors) are missing. A possible basis or starting point for the modelling of these components could be the models provided in IEC TR 62380 [BR46]and Telcordia SR-332 [BR35], again requiring an assessment regarding their applicability to space. Failure mechanisms that are of relevance for static elements such as prisms, lenses or mirrors can be considered with the methodology proposed for Mechanical reliability prediction.

- Missing models for RF passive items: The IOR compiled for the modelling of Miscellaneous items with the “standard model” approach was not sufficient to derive the low failure rates expected for RF passive items. This gap may have a large influence on the satellite reliability prediction as many single point failures at payload level are due to RF passive items. A future model could be derived by combining different inputs, e.g. from existing methodologies (as a reference or starting point) and Physics of Failure considerations with the available IOR data in a Bayesian approach.

- Limitations in the coverage of wear-out failures: Wear-out models are available for Mechanical items, but only limited information is provided for the modelling of wear-out failures of EEE and miscellaneous items. New developments are required in particular to support decisions regarding satellite life time extensions. The Bayesian approach proposed for mechanical items, combining Physics of Failure models with data, may be promising also outside the pure mechanical domain.

- Limitations in the modelling of systematic failures: A dedicated model for the consideration of systematic failures at system and subsystem level has been provided, but does not consider parameters that are of relevance for practical trade-offs and decision-making, such as e.g. maturity or test coverage. The proposed model gives average figures only and is thus limited to improving the overall reliability estimate from a system level perspective.

###### Annex Bibliography

Bibliographic references for the Annex

The following references are repeated from Chapter 10 (EEE) in Part III.

Reliability Prediction Procedure for Electronic equipment
Telcordia SR-332 issue 2 – September 2006

- Reliability Data Handbook - Universal model for reliability prediction of Electronics components, PCBs and equipment
IEC TR 62380 – July 2010

Reference Documents

The following list is repeated from the Handbook Preface, using the same numbering of Reference Documents across all Parts.

- Space product assurance – Dependability
ECSS-Q-ST-30C Rev.1, February 2017

- Space product assurance – Failure modes, effects (and criticality) analysis (FMEA/FMECA)
ECSS-Q-ST-30-02C, 6 March 2009

- Space Engineering -– Fault Tree Analysis 
ECSS-Q-ST-40-12C, 31 July 2008

- Space product assurance – Components reliability data sources and their use
ECSS-Q-HB-30-08A

- Space engineering – Structural general requirements
ECSS-E-ST-32C Rev. 1

- Space engineering - Structural factors of safety for spaceflight hardware
ECSS-E-ST-32-10C Rev. 1

- Space engineering – Mechanisms
ECSS-E-ST-33-01C Rev. 2

- ECSS system – Glossary of terms
ECSS-S-ST-00-01C

- Standard Framework for Reliability Prediction of Hardware
IEEE Std 1413TM-2010

- Reliability Methodology for Electronic Systems
UTE C80-811, FIDES Guide 2009 Ed A Rev 2010

- Reliability Prediction Models (Quanterion),
HDBK-217Plus:2015 Notice 1 – Handbook of 217PlusTM

- Reliability prediction of Electronic Equipment
MIL-HDBK-217F Notice 2 (1995)

- Electronic Reliability Design Handbook, Dept. Defence. 
MIL-HDBK-338B (1988)

- Nonelectronic Parts Reliability Data (Quanterion)
NPRD-16 (2016)

- Handbook of Reliability Prediction Procedures for Mechanical Equipment
NSWC-11 (Naval Surface Warfare Center, 2011)

- Web-Accessible Repository of Physics-based Models (RiAC)
http://www.theriac.org/WARP/index.php, accessed 08.04.2020

- TEC-QQD Study – Reliability Prediction Data Sources and Methodologies for Space Applications
Final Report, ADS-RPDSM-FR, dated 23/8/2016

- TEC-QQD Study – Reliability Prediction of Mechanical Systems & Parts
Final Report, AO/1-8017/14/NL/KM, dated 26.08.2016

- TEC-QQD Study – Use of quantitative reliability requirements for space applications
Final Report, TASF-RAMS-0010, dated 18Feb16

###### List of Changes

| Revision | Changes |
| --- | --- |
| A | - |
| B | §4 Reliability prediction taxonomy and handbook coverage - 4.2.1 Space product classification and handbook scope: Reformulated sentences regarding environmental conditions to improve clarity - 4.2.2 Levels of assembly from part to system: Defined elementary parts as the lowest level of a system - 4.3 Classification of failures by root cause: Avoided the term “failure root cause”, using simply “root cause” instead; this terminology simplification was also implemented in the remainder of Part I - 4.3.4 Extrinsic failures: Added an example for non-destructive SEE that can be of relevance for system level reliability - 4.4 Coverage of spacecraft elements and failures in this handbook: Adapted introductory sentences to improve clarity §5 Reliability prediction framework, process and use - 5.4.1 Deliverables of the reliability prediction process during Life Cycle: Added a note below Table 5-6 to explain the use of reliability prediction in early life cycle phases; Adapted Table 5-7 for consistency with Table 5-6 - 5.4.2.2 Allocation of reliability requirements: - Adapted sentences explaining the role of functional failure analysis for the cascading of top-level requirements to improve clarity; - Proportional Allocation: Adapted header of Table 5-11 and added a note above the table to explain that the allocation of failure probability targets to subsystems was based on a minimal cut set approximation in this example. - Feasibility-Of-Objectives Method: Adapted header of Table 5-12 to make clear that the entries of the first columns represent grading values. §6 Model development basic concepts - 6.2.1 In-orbit return: Deleted the condition that the cumulated hours are sufficient from a statistical point of view, which applies to all data sources and a corresponding statement can be found in the beginning of Section 6.2 §II.B (Annex Part II) Prediction uncertainties for Part III models - Re-entered Table 8-6 and Table 8-9 to avoid truncated text in the answers for some of the questions §9 Reliability prediction for EEE components and assemblies - 9.1.3 Root causes coverage: Rewording of the explanations regarding Wear-out and Extrinsic failures to improve clarity - 9.1.4.2 Model from reliability.space approach for EEE items: Improved explanation of the FIDES approach to make clear what refers to components and what is related to the equipment - 9.2.2 Parts stress method: Added an explanation for the choice of Pi_LF=1 for lead-free soldering in the context of space applications - 9.2.3.1 Temperature and thermo-electrical stress: Added Arrhenius’ law for current as a second example besides voltage - 9.2.4.2 Influence of the usage environment: Added explanation that the Weight Pos_k is independent of the level; extended equations to clarify the derivation of the values for Pi_applications; corrected wording of Table 9-4 for consistency - 9.2.4.3 Influence of the policy for over-stresses: Several editorial changes in Table 9-6 (Recommended parameters for Pi_ruggedising) for consistency with other sections. - 9.4.2 Process factor for RF HF components: Added an explanation below Table 9-14 to clarify the meaning of Criterion #8 - 9.5.4 Quantification of temperature cycling data: Added an explanation on the derivation of a thermograph from thermal analysis (minor change in Section 9.5.3 to explicitly mention thermal analysis therein, too) - 9.5.5 Quantification of humidity: Added explanation on the quantification of humidity e.g. during on-ground storage, and on the sensitivity of components to this stress - 9.5.7 Quantification of chemical stress: Deleted PCB, relays and switches in the sentence listing components that require a quantification of chemical stresses. - 9.6.10 Relays (family 09): Deleted Pi_RH in Equation 9-101 - 9.6.10 Switches (family 14): Deleted Pi_RH in Equation 9-136 - 9.9.7.3 Coffin-Manson and Norris-Landzberg’s laws for thermo-mechanical stress: Added definitions for the symbols Fu and Fs below Equation 9-178 - 9.9.8.2 Deterministic models: Added an explanation on the quantification of t_f1 and t_f2 below Equation 9-181 - 9.12 EEE Annex B: list of questions in the process questionnaire: Added explanations at the beginning of this annex to explain the relation to EEE Annex C and to clarify the meaning of the different recommendation levels; editorial changes in some of the recommendations for consistency with Annex C - 9.13 EEE Annex C: modified questions in the process questionnaire: Added explanations at the beginning of this annex explaining the relation to the space customization of the process factor (Section 9.4.1) and to EEE Annex B §11 Reliability prediction for miscellaneous items - Improved terminology throughout the whole chapter: - “Current document” refers to the reliability.space handbook - “(Reliability) Handbooks” refers to other reliability handbooks - “Standard model” applies to “generic items” - “Non-standard model” applies to «specific items” - 11.3.4 Engineering / Physics of Failure: Corrected erroneous bibliographic reference for PFMEA (new reference added in Section 11.9). - 11.5.3 Step 3: consideration of reliability contributors: Adapted Table 11-4 and added explanations below to improve clarity of the example - 11.5.5 Step 5: wear-out modelling: Corrected some equations and improved clarity in the section “Guidance to derive the Wear Out distribution characterstics” (including Figure 1-6 and Figure 1-7) §12 System level reliability prediction - 12.1 Reliability prediction at different levels: Minor additions in the text to improve clarity - 12.2 Root causes coverage: Added and corrected references in the part of Table 12-1 addressing Extrinsic failures - 12.3 System level methods: Corrected entries for Petri networks (regarding Repair, Reconfiguration) in Table 12-3; changes in the text / list below to improve clarity - 12.3.1 Reliability Block Diagrams: Improved text in the paragraph addressing the independence of individual blocks - 12.3.6 Bayesian networks: Corrected Equation 12-26; improved explanation on mutually exclusive events - 12.4.3 Combinatorial methods for phased mission based on mini-components: Deleted erroneous sentence above Equation 12-31 - 12.4.4 Boolean phase algebra: Corrected Equation 12-39 - 12.5.1 Birnbaum importance or marginal importance: Section revised, Equations and explanations improved - 12.6 Reliability assessment of degraded systems: Minor changes in the introduction of this section to improve clarity; corrected erroneous entries for System States 8 and 11 in Table 12-13 - 12.7.1 Contributing factors to Common Cause Failure: Improved explanation for the contribution of competence / training - 12.8.1 Wear-out model on component level: Added one sentence to improve explanation of Model 1 - 12.9.2 Systematic failures classification: Corrected example provided to illustrate Manufacturing error |
| C | Editorial changes only (page footer adapted) |
| D | Title page: classification changed to “not confidential” |