The first part of the handbook sets up the framework and taxonomy and links it to the Reliability Prediction process and usage. The goal is to provide guidance on the handbook usage when planning predictions.

                - Methodology

Intended audience:

- Users who are in charge of planning or specifying reliability predictions for space applications

- Users who want to check the basic concepts (e.g. the taxonomy of elements and failures) underlying the handbook approach.

Overview on Part I:

- Chapter 4 defines the overall framework, the classification of space systems and elements covered in the handbook (Section 4.2), and a classification of failure root cause (Section 4.3)
  Section 4.4 links the provided classification with the overall handbook structure

- Chapter 5 explains the reliability prediction process (Section 5.2), including a discussion of basic assumptions underlying a prediction (Section 5.2), the intended use and required scope of the prediction (Section 5.3) and the implementation throughout the project life cycle (Section 5.4)

Part I Table of Contents

4	Reliability prediction taxonomy and handbook coverage	I-5

4.1	Project phases and reliability in time	I-5

4.1.1	Project life cycle and space mission phases	I-5

4.1.2	Reliability as a function of time	I-6

4.2	Classification of space systems and spacecraft elements	I-8

4.2.1	Space product classification and handbook scope	I-8

4.2.2	Levels of assembly from part to system	I-8

4.2.3	Spacecraft elements and technical domains	I-9

4.3	Classification of failures by root cause	I-10

4.3.1	Random failures	I-10

4.3.2	Systematic failures	I-11

4.3.3	Wear-out failures	I-12

4.3.4	Extrinsic failures	I-13

4.4	Coverage of spacecraft elements and failures in this handbook	I-14

4.4.1	Elements coverage	I-14

4.4.2	Root cause coverage	I-14

5	Reliability prediction framework, process and use	I-17

5.1	Reliability prediction framework underlying this handbook	I-17

5.2	Assumptions and ground rules for the prediction	I-18

5.3	Scope and focus of the prediction for different reliability prediction uses	I-21

5.3.1	Reliability prediction versus reliability management	I-21

5.3.2	Overview on different reliability prediction uses	I-22

5.3.2.1	Reliability prediction as input to the design	I-24

5.3.2.2	Establishment and verification of quantitative reliability requirements	I-25

5.3.2.3	Reliability prediction supporting the choice of engineering design margins	I-25

5.3.2.4	Reliability prediction supporting the choice of a test strategy	I-25

5.3.2.5	Reliability prediction as input for business planning and design of constellations	I-26

5.3.2.6	Reliability prediction to support decisions about life time extensions	I-26

5.4	Reliability prediction during project life cycle	I-27

5.4.1	Deliverables of the reliability prediction process during Life Cycle	I-27

5.4.2	Management of reliability requirements	I-37

5.4.2.1	Establishment of reliability requirements	I-37

5.4.2.2	Allocation of reliability requirements	I-38

5.4.3	Reliability assessment for system architecture development during conceptual design	I-44

5.4.4	Reliability requirement verification for compliance demonstration	I-45

5.4.5	Reliability assessment for life time extension and safe disposal	I-46

I.A	Part I Bibliography	I-47

Goal of this chapter: To present the classification of space systems and spacecraft elements and the classification of failures by root cause used in this handbook, and to set this taxonomy in relation to the handbook structure.

# Reliability prediction taxonomy and handbook coverage

Katharina Fischer, Matrisk GmbH

Goal of this chapter: To present the classification of space systems and spacecraft elements and the classification of failures by root cause used in this handbook, and to set this taxonomy in relation to the handbook structure.

## Project phases and reliability in time

The following subsections give a brief introduction to the project life cycle and to the different phases describing an item’s reliability over time. These concepts are needed for the taxonomy provided in this Chapter and for the reliability prediction process discussed in Chapter 5.

### Project life cycle and space mission phases

Space projects are typically divided in seven life cycle phases from 0 to F, as defined in Table   4 -1 (cf ECSS-M-ST-10C [BR1]). The production and verification of the actual flight item takes place in Phase D, followed by its launch and utilization in Phase E and the disposal at end of life in Phase F.

The duration of the mission is determined by the specified lifetime, counting from launch to end-of-life (disposal) of a spacecraft; sometimes the disposal phase is included to consider deorbiting / reorbiting success calculations. Possible lifetime extensions increase the duration of operations in-orbit. In addition, also information about mission phases prior to launch may need to be considered as an input for reliability prediction, e.g. to consider design and manufacturing process contributions, or to account for stresses experienced during verification testing.

Table 4-1: Project life cycle and mission phases for typical space projects.

| Project life cycle phases | Mission phases (flight item) |
| --- | --- |
| Phase 0 – Mission analysis / needs identification | Specification and Design |
| Phase A – Feasibility |  |
| Phase B – Preliminary Definition |  |
| Phase C – Detailed Definition |  |
| Phase D – Qualification and Production | Production and Assembly Qualification, Verification and Validation |
| Phase E – Utilization | Launch and Orbit raising Commissioning and In Orbit Testing Normal operations in-orbit |
| Phase F – Disposal | Deorbiting / Reorbiting |

### Reliability as a function of time

Reliability generally varies as a function of time, although for some items it is more appropriate to consider reliability as a function of (or conditional on) demand - e.g. constant probability of failure for one shot items, stress-strength approach for shock loading. Nevertheless, from a system level perspective also these models can be transformed to a time scale by making predictions on the occurrence of the relevant stresses and demands during the mission.

At part level, time dependent reliability is often explained with the well-known bathtub curve, as illustrated in Figure   4 -1. The curve is divided in three phases: In the initial phase, production errors and workmanship defects are a significant source of failure. When a component has survived a certain time it implies that such birth defects are not present and consequently the reliability increases. At the end of a component’s useful life, ageing and wear-out failures lead to an increasing failure rate. Between these two phases the failure rate of many parts (e.g. most EEE components) remains roughly constant on a low level. For other parts (e.g. mechanical ones), such a steady-state phase cannot be observed, and the failure rate is slowly increasing already during the useful life.

Figure 4-1: The bathtub curve - failure rates as a function of time.

To design a reliable product, all three phases illustrated in Figure   4 -1 need to be understood.  However, this does not necessarily mean that the whole life cycle has to be modelled in a reliability prediction context.

Early life failures are avoided with the aid of screening, testing and quality control. Assuming that these processes are effective, only components that have survived the infant mortality period will be used in the final product. It is thus justified to neglect early life failures during reliability prediction, at least those due to component level defects. Non-part related systematic failures, resulting from system level design or manufacturing errors, may show a similar pattern and can be considered in the prediction using non-constant failure rates.

Wear-out failures are considered during the design of a space system, e.g. by selecting suitable components or materials for the design life time and conditions of use. The objective is to ensure that all components remain within their useful life during the specified lifetime of the system; typically, a margin is considered to account for variability and uncertainties in the prediction of the component’s life expectation. Modelling wear-out effects, requiring a departure from the constant failure rate assumption, becomes relevant for items that exhibit an increasing failure rate already during the design lifetime (safe life qualification not possible, or not fully effective), or more broadly in the case of a life time extension, see Section 5.3.2.6 for details.

## Classification of space systems and spacecraft elements

### Space product classification and handbook scope

In space applications, reliability predictions need to consider different kinds of products with completely different attributes, e.g. related to the physical environment, the use conditions or the possibilities to repair a system once operations have started.

The handbook scope is limited to Reliability Predictions for systems that operate in space, e.g. a spacecraft. The following types of spacecrafts can be considered:

- Satellite: unmanned spacecraft in Earth orbit

  - GEO satellites, e.g. telecommunications

  - MEO satellites, e.g. navigation

  - LEO satellites, e.g. Earth Observation

- Space probe: spacecraft leaving earth orbit to explore further into outer space

- Constellation of satellites, from two to hundreds or thousands of satellites

- Man-tended vehicles and experiments or modules installed in the Space Station

The handbook is generally applicable to all spacecraft missions, although limited to unmanned spacecraft technology in terms of technology coverage.

Due to the environments and technologies considered, it is not recommended to directly use the handbook for other types of space products, such as launchers or ground segments, even though parts of the methodology can still be applicable. Also, it should be noted that the focus is on systems that are non-repairable, possibly requiring additional considerations for repairable systems.

Finally, it should be noted that the handbook generally assumes a level of quality assurance as usual for spacecrafts manufactured by major satellite prime contractors. The methodology and methods can of course be used also in other applications such as microsats or for “New Space” applications. However, the models, background data and underlying assumptions may not be fully applicable, especially when dealing with process and quality related contributions to unreliability, e.g. for reliability prediction of EEE components and assemblies (Chapter 9 Part III) or when modelling systematic failures based on IOR (Chapter 12 Part III).

### Levels of assembly from part to system

For the purpose of this handbook, a system is considered in the generic sense, and can be defined at any level as a set of components or functions that are interrelated or interact to achieve a specified objective. The lowest level of a system is an elementary part (or component).

For a more specific description of the level of assembly considered, the following levels can be distinguished (in the case of a spacecraft considered as the highest level of assembly):

- Spacecraft level (only functional)

- Interfaces level (electrical, mechanical or physical)

- Equipment/Unit level (EEE, mechanical or miscellaneous)

- Part/Component level (EEE, mechanical or miscellaneous)

The spacecraft level as well as the interface level (dealing with the boundaries where different products interact) are considered in the frame of system level reliability prediction. For the equipment and component level, three technical domains are distinguished in this handbook, as outlined in Section 4.2.3 below.  The methods and models provided for these levels focus on elementary reliability prediction and may have to be combined with general system level models for equipment level reliability prediction (i.e. considering the equipment as a system of elementary parts). Additional levels (intermediate or beyond) may be considered as required by the analysis.

### Spacecraft elements and technical domains

At each level of assembly, different types of elements can be used as a part of the overall system. The handbook distinguishes three types of hardware elements, or technical domains:

- EEE
A EEE component performs an electrical, electronic or electromechanical function. At equipment level, an electrical unit is considered as a set of EEE parts assembled in a single package with structure, boards, wires and connectors to support the specified functions.

- Mechanical
A mechanical part is defined as a non-electronics part, i.e. a part that does not perform any EEE function. At equipment level, a mechanical unit (or mechanism) is assembled of mechanical parts - and possibly some EEE components - to support the specified functions. Also structural elements are considered as mechanical items.

- Miscellaneous
Mainly for modelling reasons, the handbook considers a third category of “miscellaneous” items, as parts or assemblies that are difficult to classify as either EEE or mechanical, e.g. due to their EEE-mechanical hybrid aspects.

At spacecraft level, elements from these three categories, which focus on the classification of hardware, will generally interact with software to perform the required functions. Software failures are discussed in the context of system level reliability prediction, as a subclass of (non-part related) systematic failures.

## Classification of failures by root cause

Typically, multiple root causes contribute to an undesired outcome, but it is generally possible to discriminate which cause is the dominant one; for the methodology, we will consider one main root cause for each failure addressed as the one being the most probable. The classification of failures is based on this main root cause.

The four classes of failure listed in Table   4 -2 are defined to be mutually exclusive and exhaustive, i.e. each failure observed in orbit can be classified into one of the four categories. In the following, each category is briefly discussed to further clarify the classification.

Table 4-2: Classification of failures into four categories based on the root cause of failure.

### Random failures

For the purpose of this handbook, random failures are defined as failures due to residual physical defects (or weaknesses) at part level that have not been identified during screening. The physical root cause (the defect) is either unknown or considered as one-off after investigation. Therefore, whereas the origin of systematic, wear-out and extrinsic failures can be identified, the random failures represent the failures that can occur at a rate to be determined without understanding what triggered them. We may say that the root cause of random failures is unknown.

Obviously, every failure has a root cause, and – at least in theory – we might be able to classify each random failure as either systematic, wear-out or extrinsic. It is, however, not realistic to achieve this in practice nor is it required from a modelling perspective.

Random failures are always part-related and mutually independent from one system to another.

Examples of random failures:

- Capacitor short circuit without identified cause

- Pyrotechnic device fails to ignite

### Systematic failures

The root cause of systematic failures originates from a human error during the design phase (design error), during the manufacturing phase (manufacturing error) or during the operational phase (operations error).

“Systematic” means that the failure would occur if similar elements (with the same errors) are operated under identical circumstances. To give an example, in case of a design error at equipment level, both nominal and redundant channels are impacted in the same manner even though not necessarily at the same time and with an associated probability of occurrence. As a result, systematic failure occurrence may be affected by common cause effects and cannot be assumed to be independent as in the case of random failures. However, the degree of dependence between two systematic failures resulting from the same root cause depends on the case, and may range from nearly independent (“random-like”, e.g. manufacturing error on a single hardware element) to almost fully dependent (e.g. design error affecting identical equipment operating under the same conditions) in different settings.

Systematic failures may in principle be part-related (due to part design or manufacturing errors) or non-part related, i.e. the error can be introduced at higher than part level. In the frame of this handbook it is assumed that the occurrence of part-related systematic failures is effectively reduced by using space qualified components and by the processes in place in case of an alert issued by a manufacturer. At this quality assurance level, part related failures due to design or manufacturing errors may be assumed covered by the random failure rate. Therefore, the focus of systematic failure modelling in this handbook will be on the consideration of non-part related systematic failures.

Examples of systematic failures:

- Gold pollution during the assembly of a EEE package (part level, manufacturing)

- Poor lubricant choice leading to increased friction in a mechanism (equipment level, design)

- PCB cracked by the manufacturing tool (equipment level, manufacturing)

- Wrong connection of a connector (interface level, manufacturing)

- Underestimated single event effect flux prediction (spacecraft level, design)

- Sign error in the command chain of a reaction wheel (spacecraft level, software coding)

- Wrong procedure leading to send a dangerous command (spacecraft level, operations)

### Wear-out failures

Wear-out failures result from time dependent physical processes due to calendar aging or driven by operational and/or environmental stresses. The root cause is a degradation process (modification of the design characteristics) at part level.

Space system design aims at avoiding wear-out failures by ensuring that limited-lifetime items are “safe life” qualified, which means that these items are designed to withstand the design lifetime (with acceptable performances degradation) and qualified by tests, generally with a safety factor. Therefore, no failure due to wear-out is expected before the end of the specified lifetime. Failures due to “early” degradation before this point are classified either as systematic in the case of a design or manufacturing error, or as random if an unknown residual part defect has accelerated the degradation process.

In case of a lifetime extension beyond the specified design life, the safe life qualification is no longer valid and wear-out failures may need to be considered in the prediction. The same holds true for technologies or items that already show (moderate) degradation during the normal design life, such as e.g. in the case of various mechanical parts. Reliability models used for these items need to consider increasing failure rates due to the non-negligible impact of internal wear-out processes.

The different stresses that may lead to wear-out failures are classified as follows:

- Operations-related: Use in time, On/Off cycling, mechanical wear

- Environment-related: Thermal cycling, cumulated radiation, UV light, mono atomic oxygen

To ensure mutually exclusive categories, and as a matter of convention, failures resulting from wear-out due to environmental effects are classified as Wear-out (not Extrinsic) failures.

When modelling wear-out, it is important to be clear about the definition of “failure”. The time dependent physical degradation of an item as such is not considered as a failure before a critical threshold is surpassed, leading either to a definitive failure at part level or to a failure at system level without part level failure. An example for a definitive wear-out failure would be fracture of a mechanical part after successive crack growth due to cyclic loading, whereas in the case of battery discharge due to calendar aging failure is better defined at system level by specifying a performance threshold for the battery capacity (Ah).

Examples of wear-out failures:

- Progressive parameter drift at EEE parts level due to cumulated radiation effects, displacement charge effects (TID/TNID)

- Mechanical bearing failure due to lubricant wear leading to mechanism failure

- Energy and/or power loss (below specified threshold) in batteries due to calendar ageing

### Extrinsic failures

Extrinsic failures are sudden failures induced by the space environment on the spacecraft elements. The root cause is the environment, imposing the following stresses:

- Physical conditions (defined in the technical specification)

  - Vacuum: outgassing, cold welding, heat transfer

  - Thermal effects: solar radiation, solar albedo, Earth Outgoing Longwave radiation

  - Geomagnetic field: depending on solar wind, solar activity

- Launch effects

  - Mechanical vibration: vibrations during launch

  - Shocks: e.g. pyro activation

- External particles

  - Plasma: Electro Static Discharge

  - Heavy ions, Single Event Effects: destructive, non-destructive

  - Micro meteorite impacts

  - Space debris impacts

Failures due to cumulated extrinsic effects are (by convention) classified as Wear-out failures, see Section 4.3.3 above. Thus, failures due to the following stresses only need to be considered when wear-out predictions become relevant, e.g. to support decision-making on possible satellite life time extensions:

- Wear-out due to extrinsic effects

  - Single atomic oxygen

  - Radiations: ionizing, non ionizing

  - Ultra Violet light

Finally, failures due to extrinsic effects that were underestimated during the design are classified as Systematic, as the root cause is a design error rather than the space environment alone.

Examples of extrinsic failures:

- Destructive Single Event Effects (e.g. Single Event Lath up or Single Event Burnout)

- Non-destructive SEE causing a non-detected corruption of data used in a system control loop

- Outgassing of materials leading to contamination of optical devices

- Debris impact causing failure critical components that are not protected by shielding

- Fracture of a structural element due to launch vibrations or shock loads

## Coverage of spacecraft elements and failures in this handbook

The following subsections give a brief overview on the coverage in terms of spacecraft elements and failure root causes provided in the handbook’s model part (Part III).

### Elements coverage

To simplify the application of the handbook for users in practice, Part III is structured based on the technical domains introduced in Section 4.2.3:

- Chapter 9 for the modelling of EEE components and assemblies

- Chapter 10 for the modelling of Mechanical and structural parts and assemblies

- Chapter 11 for the modelling of Miscellaneous items

- Chapter 12 for System level considerations, independent of the technical domains

In terms of levels, Chapter 9 to 11 focus on elementary reliability modelling for parts or low levels of assemblies (up to function / equipment level) in each technical domain, whereas the approaches discussed in Chapter 12 can be applied at any level to systems in the generic sense.

### Root cause coverage

For the coverage in terms of failure categories, the methodology has a clear focus on random failure modelling, leaving the consideration of other root causes to dedicated model extensions. It should, however, be noted that the distinction between random, systematic, wear-out and extrinsic failures is not always clear-cut and may sometimes be a modelling choice, or a matter of definition, rather than resulting directly from the “nature” of the failures.

Each chapter in Part III of the handbook (Models) provides information about the coverage in terms of root causes of failure. A quick overview for all chapters is given in the following:

- RANDOM FAILURES: Random failures are clearly a relevant input for the design of reliable systems. They are fully covered by the methodology provided in this handbook.

- SYSTEMATIC FAILURES: The models provided in this handbook do not guarantee a complete coverage of all systematic failures for all spacecraft elements.

  - For EEE items, a certain coverage of systematic failures is achieved with the aid of process factors adapted to space applications.

  - For Mechanical items, the methodology allows the consideration of failures due to minor design uncertainties and manufacturing variability. Gross errors require a dedicated modelling at system level.

  - For Miscellaneous items, the methodology allows the consideration of systematic failures from a simple statistical perspective, referring to the modelling at system level as well.

  - General considerations for systematic failure modelling are given in the system level chapter, including a statistical model based on IOR data.

- WEAR-OUT FAILURES: Also wear-out failures are not completely covered by the models provided in this handbook:

  - For EEE items, the models provided generally assume constant failure rates during the useful life of a component, but a methodology for the separate consideration of wear-out failures is proposed in case an explicit wear-out modelling is needed, e.g. in the context of a life time extension.

  - The methodology for Mechanical reliability prediction includes an explicit consideration of relevant wear-out failure mechanisms, which may be used to model premature failures before the end-of -life, as well as wear-out failures during a potential life time extension.

  - For Miscellaneous items, the methodology includes the consideration of wear‐out failures, modelled beyond the mission lifetime based on an increasing failure rate and with model parameters related to the lifetime qualification of the items.

  - System level considerations of wear-out modelling can be found in the system chapter, including approaches for the combination of a constant failure rate for random failures with a separate wear-out model implying increasing failure rates.

- EXTRINSIC FAILURES: Many of the environmental stresses considered for the definition of extrinsic failures are in fact part of the normal environment for a spacecraft and its components. Some of them are thus considered as “stress contributors” for the random failure modelling. Others are assumed to be covered and effectively avoided by different design disciplines and thus excluded from the base methodology.

  - A dedicated discussion of Extrinsic root causes by technical domain can be found in each of the three chapters related to reliability prediction for EEE, Mechanical and Miscellaneous items.

# Reliability prediction framework, process and use

Katharina Fischer, Matrisk GmbH

With the contribution of Sarel Consult GmbH (Section 5.4)

Goal of this chapter: To provide guidance on the general reliability prediction framework, the selection and documentation of basic assumptions and the required scope of a reliability prediction, and to discuss the process of reliability predictions throughout the project life cycle.

## Reliability prediction framework underlying this handbook

Any reliability prediction should be clear, specific and useful.

- "Clear” means that the assumptions underlying the prediction are clearly defined and communicated together with the prediction results, allowing for a meaningful interpretation.

- “Specific” means that the characteristics of the product and mission under analysis are considered appropriately, and the prediction accounts for the relevant variables.

- “Useful” means that the prediction is giving answers to the questions that are of relevance for the required use of the prediction results.

To achieve this, the framework of this handbook starts the prediction process with a discussion of the assumptions underlying the prediction (Section 5.2), allowing to be at the same time specific to the problem at hand and clear regarding the meaning and interpretation of the prediction results. A key concept is the intended use of the prediction, which is driving the required scope and focus of the prediction in order to be useful for the trade-offs that will be supported (Section 5.3). Throughout the project life cycle, the prediction will

serve different purposes, and different reliability prediction tasks are performed to coordinate the prediction throughout the system development (Section 5.4).

## Assumptions and ground rules for the prediction

When planning a reliability prediction, the ground rules and key assumptions underlying the prediction must be agreed upon in a first step. Which assumptions are appropriate, depends on the following aspects:

- The characteristics of the system or item under analysis,

- the project life cycle phase in which the prediction is performed and

- the intended use of the prediction results.

Thus, sufficient information on these three points should be available before starting with the prediction, and even before defining any ground rules and assumptions.

Table   5 -3 gives an overview of different areas in which assumptions need to be made before starting a system level reliability prediction. The associated ground rules give a rough indication of the general principles that should be followed to achieve maximum accuracy (i.e. realistic predictions). It is not mandatory to be compliant with each ground rule, but the assumptions made should be justified considering the characteristics of the system, the project life cycle phase in which the prediction is made and the intended use of the prediction results.

It should be clear that the assumptions made can have a tremendous impact on the results. If the main goal is to receive reliability predictions that are comparable (e.g. between different manufacturers or suppliers), one must ensure that the same assumptions are used. At least the key assumptions with the largest impact on the result should be agreed by all parties, and/or specified in the supply chain.

Any reliability prediction report should provide full information on the assumptions made for the prediction, as listed in Table   5 -3. This ensures that the required elements of a reliability prediction, as defined in IEEE 1413:2010 [RD9] , Clause 4, are provided with the prediction. The relation between each of these required elements, the assumptions listed in Table   5 -3 and the relevant sections of this handbook is given in Table   5 -4.

Table 5-3: List of assumptions to be agreed upon and associated ground rules for predictions at system level (equipment level or higher).

Table 5-4: Relation between the assumptions listed in Table   5 -3, the required elements of a prediction according to IEEE 1413:2010 [RD9]  and the relevant sections of this handbook.

| Required element of a reliability prediction (IEEE 1413:2010, Clause 4) | Table 5 -3 Assumptions | Handbook Chapters |
| --- | --- | --- |
| Identification and description of the item for which the prediction is made and the life cycle phase upon which the prediction is performed | - Basic information - System definition - Project life cycle phase | --- §§ 4.1.1, 5.4 |
| Intended use of the prediction results | - Intended use of the prediction | § 5.3.2 |
| RP coverage: No required element in IEEE 1413:2010 | - Coverage - Mission phase coverage - Elements coverage - Failure modes coverage - Failure mechanisms coverage - Root causes coverage | § 4.1 § 4.2 § 6 (Part II) § 6 (Part II) §§ 4.3, 4.4 |
| List of inputs used for the selected methodologies | - Inputs - Mission definition - Design lifetime - Operational conditions - Environmental conditions - Product design information - Methods, models - Data | --- --- --- § 4.3.4 --- § 6 (Part II) § 6 (Part II) |
| Modelling: No required element in IEEE 1413:2010 | - Modelling - Redundancy considerations - Degraded system modes - Dormant phases - Common cause effects - Distribution functions | § 12 (Part III) § 12 (Part III) § 12 (Part III) § 12 (Part III) § 12 (Part III) |
| Prediction metrics: Definitions and values | - Outputs - Prediction metrics | § 6 (Part II) |
| Uncertainties and limitations of the prediction | - Prediction uncertainties | § 8 (Part II) |
| Statistical confidence in the prediction | - Conservativism | § 8 (Part II) |

## Scope and focus of the prediction for different reliability prediction uses

The scope and focus of a reliability prediction can be defined in terms of different axes, see Table   5 -4 above (RP coverage).

As a general rule, all elements and associated failure modes and mechanisms need to be covered by the prediction, unless their contribution to overall system (un-)reliability – or to the decision that will be supported – may be assumed to be negligible for practical purposes. Similar considerations hold for the coverage of mission phases during the prediction.

The required coverage in terms of root causes (failure categories, see Section 4.3 for classification) depends on the intended use of the prediction. This is discussed in the following subsections.

### Reliability prediction versus reliability management

Achieving a high reliability product is an important objective during the design and production of any space system. Considering all root causes of failure is a prerequisite, and different mitigation processes are in place to avoid the occurrence of each of them, see Table   5 -5 for examples. Apart from measures to avoid the different root causes, system level design aims at mitigating the effect of lower level failures on the success of the mission.

The objective of reliability predictions is to provide quantitative estimates for the (remaining) probability of failure despite the implementation of these measures. Some of the mitigation measures are explicitly considered in the prediction, e.g. quality level of EEE parts, or redundancy at system level. Others may be used as a justification to neglect certain root causes in the prediction, provided that the mitigation measures are sufficiently effective to avoid their occurrence. To give an example, calculations from radiation engineering may provide evidence that the rate of destructive Single Event Effects is negligible compared to the random failure rate. Similar considerations become relevant for wear-out failures of EEE components, which can in most cases be effectively avoided by safe life qualification (with appropriate margins), at least when the prediction is limited to the specified design lifetime.

In addition, depending on the intended use of the prediction, there may be no added value to make a quantitative prediction for a certain root cause, because it does not make a difference for the trade-offs that will be supported by the prediction. These aspects are discussed in the following sections.

Table 5-5: Failure categories (from Table   4 -2) with examples of mitigation measures.

### Overview on different reliability prediction uses

Different reliability prediction uses become relevant throughout the project life cycle of a space mission; see Table   5 -6 for an overview. The Table includes some classical reliability prediction uses related to the management and verification of reliability requirements or to the support of design trade-off decisions. However, also some new stakes are addressed, e.g. related to the design of constellations or to the safe disposal of satellites.

Table 5-6: Possible reliability prediction uses throughout the project life cycle.

| Reliability prediction uses | Description | Phases |
| --- | --- | --- |
| Input to the design, support trade-offs and comparisons | During the design phase, reliability prediction may be used to compare competing designs or trade-off options, to identify weak parts of the design and to assess the impact of design changes. The level of detail increases with the project phases. | A – D |
| Establishment, management and verification of quantitative reliability requirements | The purpose of quantitative reliability requirements and their management and verification is to ensure acceptable (as specified) reliability of space products through contractual specification. | 0 – D |
| Support decisions on the choice of engineering design margins | Reducing excessive margins is one way to reduce cost, but should be justified by an appropriate rationale or analysis. On the other hand, it may sometimes be reasonable to increase a specific margin to avoid a catastrophic single point failure. | A – C |
| Choosing a test strategy at part, equipment or higher levels | Another way to reduce cost is to reduce the effort dedicated to testing. Additional tests may be useful e.g. to verify a design for identified stresses, to avoid costly redesigns. Also these decisions should be justified by an appropriate rationale or analysis. | D |
| Support business planning for single spacecrafts and for the design of constellations | Reliability predictions - or specified reliability requirements - form an important input to support space customer’s business planning. This holds particularly for the design of constellations as “system of systems”, e.g. to decide on the number of spares, replenishment scenarios or redundancy management. | 0 – E |
| Health monitoring and decision-making on life time extension vs. safe disposal | Spacecrafts are designed to be reliable for a specified lifetime and any life time extension needs to be justified based on RP. To support the decision, the reliability prediction needs to be revisited, in particular for the functions relevant for satellite safe disposal (space debris mitigation). | E – F |

The reliability prediction methodology presented in this handbook intends to embrace different reliability prediction uses, although the focus is clearly on the “classical” uses related to the development and design of a single spacecraft. The first use listed in Table   5 -6 - reliability prediction for design support - is considered as the base case. Recommendations on the root cause coverage required for this use are provided in Section 5.3.2.1 , followed by a discussion of the remaining uses in the following subsections.

#### Reliability prediction as input to the design

Providing input to the design of a spacecraft may be seen as the classical use for reliability predictions in space applications, and is the focus of this handbook. To be useful for design, the methodology needs to account for the relevant design variables in order to support the required trade-offs, and for the affected categories of failures.

Guidance on the root cause coverage required for this reliability prediction use is given in Table   5 -7 below. It should be noted that the recommendation made for systematic failure modelling is driven by the limitations of the available modelling approaches, which do not account for the relevant decision variables (e.g. impact of maturity category, test strategy). Other design decisions, such as redundancy sizing or the margin policy, are not effective to avoid systematic failures. For these reasons, the added value to consider this failure category for design support is small despite its clear relevance for the overall failure count.

Table 5-7: Root causes coverage for reliability prediction as input to the design.

| Failure category | Required coverage for reliability prediction as design support |
| --- | --- |
| Random failures | Full coverage required |
| Systematic failures | Not generally required to cover systematic failures, unless full root causes coverage is needed e.g. to support the design of a constellation (Section 5.3.2.5). |
| Wear-out failures | Wear-out after the specified lifetime is out of scope for this use (see Section 5.3.2.6 for life time extensions). Premature wear-out (excluding systematic failures) needs to be considered for technologies for which safe life qualification is not possible, or not fully effective. |
| Extrinsic failures | Relevant stress contributors resulting from the spacecraft environment should be considered in the prediction of random and wear-out failures. Explicit consideration of extrinsic failures with dedicated models is only required if the rate of occurrence of additional failure modes (e.g. destructive SEE, space debris impact) cannot be neglected when compared to the random failure rate. |

The recommendations regarding root causes coverage are generally valid also for preliminary reliability predictions, e.g. for the Preliminary Design Review (PDR). However, the level of detail used in the modelling can be reduced in this context to reduce prediction efforts and to account for limited input availability in early project phases.

#### Establishment and verification of quantitative reliability requirements

To allow for a meaningful verification, the specification of quantitative reliability requirements should always go hand in hand with a clear definition of the requirement’s scope in terms of failure categories, as well as elements coverage.

The recommendations on root cause coverage for design support in Table   5 -7 may be used also in this context. The quantitative reliability requirements specified between customers, prime contractors and suppliers can then directly be used as a design driver, with the goal to find the best architecture and detailed design to comply with the requirements under the given schedule and budget constraints.

After completion of the detailed design, the prediction may be extended to account for systematic failures as well, e.g. when full root causes coverage is needed to support the business planning for the owner of a single satellite or a satellite constellation (Section 5.3.2.5).

#### Reliability prediction supporting the choice of engineering design margins

The recommendations made in Table   5 -7 are based on the assumption that the occurrence of certain root causes is effectively reduced by different mitigation measures, as listed in Table   5 -5. However, one possible use of quantitative reliability predictions is to assess the risk associated with a reduction of design margins, or the benefit of increasing a specific margin. The reliability prediction then needs to account for the effect of this margin policy decision, which may require the consideration of additional root causes. Safe life qualification to avoid wear-out failures before end-of-life is a case in point; wear-out models that are based on Physics of Failure allow quantifying the effect of the associated margins (e.g. radiation margins) on the item’s reliability. More generally, the effect of design margins can be quantified with the aid of any reliability model that consider the effect of the stress contributors that are addressed by this margin.

#### Reliability prediction supporting the choice of a test strategy

Also the choice of a suitable test strategy may be based on quantitative reliability predictions to assess the effect on the reliability of the flight item. Part level tests, such as e.g. lot acceptance tests, reliability tests, lifetime tests or radiation tests, generally have a clear relation to a specific failure category and the risk associated with a specific test plan (e.g. sample size, duration) can be quantified using statistical methods.

System level qualification and verification tests are performed mainly as a means of quality control, to identify possible design and manufacturing errors that may otherwise lead to systematic failures during operations in orbit. However, with the approaches for systematic failure modelling presented in the current handbook, it is not possible to consider the effect of testing and thus to quantify the risk associated with a specific test strategy.

#### Reliability prediction as input for business planning and design of constellations

To support business planning on the customer side (for single satellites, and especially for constellations), or for the insurance of space systems, reliability predictions need to be as realistic as possible. To achieve this, the scope of the prediction should follow the recommendations made for design support (Table   5 -7), with an extension to account for systematic failures as well.

#### Reliability prediction to support decisions about life time extensions

The limited coverage of wear-out effects proposed in Table   5 -7 for design support is justified by the fact that components are generally qualified for the specified lifetime. This condition is violated in the case of a lifetime extension, requiring additional considerations for the associated reliability predictions.

To support decisions on life time extensions, it may not be required to revisit the scope of the prediction for all spacecraft elements; e.g. for space debris mitigation only the functions needed for safe disposal are of interest, and health monitoring may be used to better assess the risk of failure in redundant system architectures.

Where a quantitative prediction is required, the scope of the prediction needs to be extended to account for additional wear-out failures that may become relevant due to the lifetime extension.

## Reliability prediction during project life cycle

This section explains how the system reliability prediction process interacts with the system development process and which activities and deliverables should be performed throughout the system life cycle. The typical system life cycle of space products according to ECSS-M-ST-10C [BR2] consists of seven phases, as shown in Figure   5 -2 (see also Section 4.1.1).

Figure 5-2 Interaction of the reliability process with the system life cycle

The establishment and cascading of reliability requirements will be explained in chapter 5.4.2.1. The evaluation of system architectures from reliability perspective in early design phase is introduced in chapter 5.4.3. The verification of system reliability requirements is part of chapter 5.4.4 and the aspects of reliability prediction for life-time extension and safe disposal are handled in chapter 5.4.5. The contribution of reliability prediction to the system development during the different life cycle phases is shown in chapter 5.4.1.

### Deliverables of the reliability prediction process during Life Cycle

The scope and aim of each system life cycle phase, is highlighted in Figure   5 -2, taken from ECSS-M-ST-10C [BR1].

Figure 5-3  System Life Cycle.

The contribution of reliability prediction to each phase and the associated reviews are explained as follows in Table   5 -8. Table   5 -9 gives an overview of reliability documents to be provided per review during the system life cycle.

Table 5-8 Reliability Deliverables per Project Milestones/Reviews

|  |  | Phase |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ECSS Document | 0 | A | B |  | C | D |  |  | E |  |  |  | F |
| Document Title |  | MDR | PRR | SRR | PDR | CDR | QR | AR | ORR | FRR | LRR | CRR | ELR | MCR |
| Failure modes and effects analysis/ failure modes, effects and criticality analysis (as input for system level analysis, e.g. FTA) | ECSS-Q-ST-30-02 [RD2] |  |  | *) | X | X |  |  |  |  |  |  |  |  |
| Fault tree analysis (FTA) (to support reliability prediction) | ECSS-Q-ST-40-12C [RD3] |  |  |  | X | X | X | X |  |  |  |  |  |  |
| Reliability Prediction | ECSS-Q-ST-30C [RD1] |  | X *) | X *) | X | X | X | X |  |  |  |  |  |  |

*) Although ECSS-Q-ST-30C mentions a reliability prediction at PRR and SRR, this is not done for all projects and may be used only to assist apportioning of requirements to lower level. A FMEA at SRR may be required for specific missions and can be used e.g. to assist safety analysis.

Table 5-9 Contribution of reliability prediction in different mission phases

| Phase 0: Mission analysis - needs identification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main objectives | Associated reviews | Involvement of Reliability Prediction |  |  |
|  |  | Task | Input | Output |
| Define mission needs and expected performance Identify constraints and boundary conditions with respect to physical and operational environment Define possible mission concepts | Mission Definition Review (MDR): Definition of mission baseline. | In Phase 0 the reliability prediction activities focus on capturing top level requirements and boundary conditions. The Mission Definition Review (MDR) provides the mission profile that is to be used for reliability prediction. Top level reliability requirements are derived from customer needs. A first high level reliability prediction could be done, e.g. for quotation | Mission Profile At this stage of the development process no system architecture is available. Only data from similar projects | Top level requirements included in preliminary technical specification First rough reliability estimation |

| Phase A: Feasibility |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main objectives | Associated reviews | Involvement of Reliability Prediction |  |  |
|  |  | Task | Input | Output |
| Assess the technical and programmatic feasibility of the possible concepts by identifying constraints related to implementation, costs, schedule, organization, operations, maintenance, production and disposal. Identify critical technologies and propose pre‐development activities. | Preliminary Requirement Review (PRR): Assess feasibility of user requirements to allow a solid start of preliminary design. | In Phase A the following points are to be addressed by the reliability activities: Assessment of feasibility to achieve the system level reliability requirement Breakdown of system level reliability requirement to lower level to establish a requirement basis to start with preliminary definition in Phase B, which is an input for Preliminary Requirement Review (PRR) | The input data for activities in Phase A are based on a concept of the system architecture, as the detailed design is usually not available and also no FMEA/FMECA. To perform a reliability assessment as well as the partitioning of reliability requirements, often historical data of similar systems and components are used as initial values. | System reliability requirements Reliability requirement breakdown to lower level. Reliability Prediction in such an early phase may be done as rough estimate to check feasibility and to support apportioning of requirements to lower levels. |

| Phase B: Preliminary definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main objectives | Associated reviews | Involvement of Reliability Prediction |  |  |
|  |  | Task | Input | Output |
| Conduct “trade‐off” studies and select the preferred system concept, together with the preferred technical solution(s) for this concept. Establish a preliminary design definition for the selected system concept and the preferred technical solution(s). | System Requirements Review (SRR): Freeze of high level requirements Preliminary Design Review (PDR): Freeze of mission baseline and requirements down to subsystem level. Freeze of design concept at system level. | In the early phase of the development, quantitative methods are used to support the definition of the system and to refine the allocation of requirements. During the development activities, decisions and trade-offs are supported by reliability prediction. For each proposed system solution a preliminary reliability prediction is performed as decision basis to support the Preliminary Design Review (PDR). Preliminary version of FMEA, FMECA are to be prepared as input for system level analysis. | Input data for the preliminary reliability assessment are preliminary system architecture, preliminary versions of FMEA/FMECA and preliminary reliability data on sub-system, equipment and component level. Top level reliability requirements System level reliability requirements | The allocation of system level reliability requirements to lower levels is refined as system architecture evolves. For the System Requirements Review (SRR) the reliability requirements are to be validated. Preliminary reliability prediction including FTA to support PDR. |

| Phase C: Detailed definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main objectives | Associated reviews | Involvement of Reliability Prediction |  |  |
|  |  | Task | Input | Output |
| Completion of the detailed design definition at all levels in the customer‐supplier chain. Production, development testing and pre‐qualification of selected critical elements and components. Production and development testing of engineering models, as required by the selected model philosophy and verification approach. | Critical Design Review (CDR): Confirmation of detailed design, release of final design. Authorisation to complete qualification and build flight units. | During Phase C, the reliability assessment is updated based on the detailed system definition to demonstrate that reliability requirements are met, supporting Critical Design Review (CDR). FMEA, FMECA and component reliability prediction are to be prepared based on detailed design | Input data based on a detailed system architecture are updated, including FMEA, FMECA, FMES and component level reliability prediction. | Reliability Prediction including Fault Tree Analysis or other equivalent methods for system reliability assessment |

| Phase D: Qualification and production |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main objectives | Associated reviews | Involvement of Reliability Prediction |  |  |
|  |  | Task | Input | Output |
| Complete qualification testing and associated verification activities. Complete manufacturing, assembly and testing of flight hardware/software and associated ground support hardware/software. | Qualification Review (QR): Demonstrate that the system meets all requirements and verification proof is complete Acceptance Review (AR): Acceptance of the System by the Customer Operation Readiness Review (ORR): Verify readiness of the operational teams and procedures and their compatibility with the flight system. | The reliability prediction is updated for customer acceptance considering test results from qualification. | Input is based on final system design with updated FMEA, FMECA and FMES. | Reliability Prediction including updated Fault Tree Analysis or other equivalent methods for system reliability assessment |

| Phase E: Operations / Utilization |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main objectives | Associated reviews | Involvement of Reliability Prediction |  |  |
|  |  | Task | Input | Output |
| Prepare launch of the system Perform launch and in-orbit testing. Perform in-orbit operations | Flight Readiness Review (FRR): Verify that the flight and ground segments are ready for launch. Launch Readiness Review (LRR): Performed right before launch to provide authorization to proceed for launch. Commissioning Results Review (CRR): Verify system performance after in-orbit testing. End-of-Life Review (ELR): Verify that the system has completed its useful life, ensure safe disposal | Depending on the result of in-orbit testing during commissioning, for example if redundancies are not available, the reliability assessment may have to be re-evaluated to support CRR. Update of reliability prediction based on in-orbit feedback. Reliability assessment for safe disposal. | FMEA, FMES In-orbit test results In-orbit reliability data | Update reliability prediction with in-orbit data for deorbiting function to support End-of-Life Review (ELR) and decision making on life extension. |

| Phase F: Disposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Main objectives | Associated reviews | Involvement of Reliability Prediction |  |  |
|  |  | Task | Input | Output |
| Perform disposal, deorbiting of the space system | Mission Closure Review (MCR): Ensure that disposal is completed | Comparison of initial reliability prediction with In-Orbit Return |  | Derive lessons learnt to support future projects |

### Management of reliability requirements

#### Establishment of reliability requirements

In the following, the process steps to support the establishment of appropriate reliability requirements will be explained according to [RD19] . The first step is the classification of the type of space mission, as shown in Table   5 -10. Details on the coverage of this handbook in terms of mission types can be found in chapter 4.2.1.

Table 5-10 Space Mission Classification

|  | Mission Type |
| --- | --- |
| Class A | Human Flight or Material Transport Flight |
| Class B | Telecommunication, Observation and Navigation for application with high integrity requirements |
| Class C | Telecommunication, Observation Mission, Space Probe |
| Class D | Test and demonstration mission |
| Class L | Launcher, Launch base |

Within each class, the missions can be further categorized based on the following criteria [RD19] :

Table 5-11 Category of space mission

|  | Category 1 | Category 2 | Category 3 |
| --- | --- | --- | --- |
| Development time | More than 2 years | More than 2 years | Less than 2 years |
| Criticality to customer strategic objectives | High | Medium | Low |

The type of mission as well as the categories of mission within each class (Table   5 -11) should be taken into account for the establishment of reliability requirements. As a general rule, the larger the economic loss a failure of the mission would cause, the more stringent the reliability requirement should be. Furthermore, the criticality of the mission success to the strategic objectives of the customer may justify a higher reliability requirement.

Reliability requirements are expressed as expected probability of success over a given time period under consideration of:

- Customer commercial objectives, revenue and return on investment

- Insurer requirements

- Regulation, e.g. avoidance of space debris and safe disposal

- Technical feasibility

- Cost, weight and volume constraints

For example

The probability that the satellite achieves its performance requirements shall be better than 90% after a mission duration of 10 years in orbit.

Each quantitative requirement should be linked to an explanation to ensure correct interpretation. This includes definition of the scope, principles and boundary conditions that are to be applied for the reliability prediction, as discussed with the ground rules and assumptions for reliability prediction in Section 5.2.

The reliability requirements could also be defined considering partial losses leading to reduced functional capability and graceful degradation, for example:

The probability that at least k out of n antenna links are operative after y years in orbit shall be greater than 98%.

Where n denotes the total number of units installed and k an acceptable degraded mode, with k < n. The duration y is usually defined in years and corresponds to end of mission, end of in-orbit testing or end of LEOP, but it can also refer to operation cycles or hours.

#### Allocation of reliability requirements

The allocation of requirements may start with assigning historical data of similar systems to sub-systems and equipment level and is then refined as more details become available.

The allocation of reliability requirements then consists of the following steps:

- Analysis of input requirement to formulate functional and performance requirements

- Define functional architecture to ensure system performance.

- Functional failure analysis at system and sub-system level to identify failure scenarios that would lead to violation of reliability requirements

- Create a high level system model that consists of the relevant subsystems based on the functional failure analysis.

- Assign reliability targets to sub-functions. Besides the use of historical data of similar systems different approaches could be used to assign initial reliability targets, for example, see page 44:

- Equal allocation

- Proportional allocation (ARINC method)

- Feasibility-Of-Objectives (FOO) Method

- Review of sub-system targets with regard to feasibility, cost, schedule etc. and refine allocation if deemed necessary. This may involve iterations to find a well-balanced apportionment of reliability targets.

The allocation of requirements to lower level starts with the identification of system functions. Based on the customer top level reliability requirements, a system function for the design can be identified. This overall system function is decomposed into its sub-functions [BR4]. The functional failure analysis determines the failure effects and repercussions at system level. The following generic failures could be used as guideline to assess each function and sub-function:

- Total loss of function

- Partial loss of function

- Un-commanded or spurious functioning

- Erroneous functioning

The results of the functional failure analysis allow identifying the functions; those failures would affect the ability to perform the required system function. The cascading of the top-level system reliability requirement to the contributing sub-functions should consider the results of the functional failure analysis. That means, the reliability target allocated to functions and sub-functions should consider the relevant failure modes to ensure functional integrity.

An example of the functional failure analysis for the power supply system is shown in Table   5 -12. A similar analysis needs to be performed for all functions of the satellite, whereby in early phases of the development the main functions are considered and as design evolves the functional breakdown can be refined and more details are included.

Table 5-12 Example Functional Failure Analysis Power Supply

| Functional Failure Analysis Function: Provide electrical power |  |  |
| --- | --- | --- |
| Function / Sub-function | Functional failure | Failure effect |
| 1. Provide electrical power | Total loss of electrical power | Total loss of power supply. No data communication. Loss of satellite control. |
| 1.1 Photovoltaics | Total loss of photovoltaics capabilities | Total loss of power supply. No payload Total loss of satellite |
|  | Partial loss of photovoltaics capabilities | Degraded performance, battery not fully charged, power interrupt in Earth’s shadow possible. Payload interruptions. |
| 1.2 Charge Battery | Loss of battery charging | No power supply in Earth shadow in one orbit Interruption of payload Possible loss of satellite |
|  | Overcharge of battery | Permanent damage of battery possible No power supply in Earth shadow in one orbit Interruption of payload Possible loss of satellite |
| 1.3 Output voltage regulation | Erroneous function output regulation - too low | Insufficient voltage supply to satellite. No data communication |
|  | Erroneous function output regulation – voltage too high | Permanent damage to electronic components possible if no over-voltage protection is implemented |

The analysis of functional failures can also be represented with a reliability block diagram. Quantitative methods are used to support the system definition and to refine the allocation of requirements to sub-system level.

In the following, different approaches for requirement allocation are introduced, including:

- Equal allocation

- Proportional allocation (ARINC method)

- Feasibility-Of-Objectives (FOO) Method

These methods are applicable for a serial system structure only. For more complex system architectures, the reliability allocation should also make use of system level reliability assessment methods (see Chapter 12 Part III).

The apportionment of the system level reliability target to sub systems is based on the following relation for serial system.

| Rit=RStwi | Equation 5-1 |
| --- | --- |

where

RSt denotes the reliability target on system level

Rit denotes the reliability target on sub-system level, and

wi denotes the weighting factor of sub-system i

It is obvious that for different values for Rit the same system reliability RSt can be obtained. Thus, there are infinite possible solutions to allocate the system level reliability target to the sub-system level. Therefore, practical methods are needed to assist the system design in the apportionment of reliability requirements.

Equal Allocation

This method equally distributes the system reliability on all the sub-systems below the system level [RD13] . The weighting factor is equal for each sub system and given by the number of sub systems. The reliability target for the sub-systems is given by

| Rit=RSt1n | Equation 5-2 |
| --- | --- |

Where

RSt denotes the reliability target on system level

Rit denotes the reliability target on sub-system level, and

n denotes the number of sub-systems.

For example, a reliability target of 0.9 on system level would result in a target of 0.9826 for each sub system, if the system consists of 6 sub-systems. If the target on system level is given as failure probability or failure rate the target on subsystem level is obtained from the following equations.

| Fit=FStn | Equation 5-3 |
| --- | --- |
| λit=λStn | Equation 5-4 |

Where

FSt denotes the failure probability target on system level

Fit denotes the failure probability target on sub-system level

λSt denotes the failure rate target on system level

λit denotes the failure rate target on sub-system level, and

n denotes the number of sub-systems.

The equal allocation is very easy to apply, but does not consider the technical feasibility and experience from similar projects. This could result in very stringent requirements for some sub-system that cannot be achieved or only with increased effort. Thus, it might be necessary to refine the allocation in order to get to a more balanced share between sub-systems.

Proportional Allocation (ARINC Method)

The proportional allocation also known as ARINC method takes historical data on sub-system reliability into account to distribute the reliability to sub-system level. The weighting factor is determined by the ratio of the observed sub-system failure probability to the total system failure probability, as shown in Equation   5 -5. The new reliability target is allocated to sub-system proportional to this factor.

| wi=Fi.oldtFS.oldt | Equation 5-5 |
| --- | --- |

Where

FSt denotes the failure probability target on system level

Fit denotes the failure probability target on sub-system level

FS.oldt denotes the historical failure probability on system level

Fi.oldt denotes the historical failure probability on sub-system level

Given that 15% of the system failures are caused by a failure of the power supply sub-system a reliability target of 98,43% is derived for the power supply system using Equation   5 -1 to achieve a reliability of 90% on system level, as shown in Table   5 -13. Please note that in the example the failure probabilities of the sub-system were derived from the minimal cut set approximation to simplify the calculation.

Table 5-13 Example of proportional allocation of reliability targets to sub-systems

| Sub-System | Weighting Factor wi | Failure probability Target (approximation) | Reliability target Rit=RStwi |
| --- | --- | --- | --- |
| Power | 0.15 | 0.015 | 0.9843 |
| Tele-Command/Telemetry | 0.20 | 0.02 | 0.9791 |
| Propulsion | 0.10 | 0.01 | 0.9895 |
| Orbit Control | 0.15 | 0.015 | 0.9843 |
| Structure | 0.10 | 0.01 | 0.9895 |
| Data Communication (Pay load) | 0.30 | 0.04 | 0.9689 |
| System level | 100% | 0.1 | 0.90 |

Feasibility-Of-Objectives (FOO) Method

The FOO method allows users to assign grading factors to sub-systems and their components in order to determine how reliability targets are cascaded from top level to lower level. A sub-system with high grading factors is allocated a lower reliability than a sub-system with low grading factors. Default grading categories are complexity, technical level (state of art), operating time and environmental condition. Users may change these categories. Each rank is based on a scale from 1 to 10 and is estimated using both design engineering and expert judgment [RD13] :

- System Complexity. Complexity is evaluated by considering the probable number of parts or components making up the sub system and also is judged by the assembled intricacy of these parts or components. The least complex sub system is rated at 1, and a highly complex sub system is rated at 10.

- Technology level. The state of present engineering progress in all fields is considered. The least developed design or method receives a value of 10, and the most highly developed is assigned a value of 1.

- Operating Time. An element that operates for the entire mission time is rated 10, and an element that operates the least time during the mission is rated 1.

- Environmental conditions are also rated from 10 through 1. Elements expected to experience harsh and very severe environments during their operation are rated as 10, and those expected to encounter the least severe environments are rated as 1.

The first stage for this allocation method is to calculate the total grading value for each sub-system. This is obtained by multiplying the grading factors from each category:

| Gi=jngij | Equation 5-6 |
| --- | --- |

The weighting factor for each sub system is given by the following equation:

| wi=jngijiGi | Equation 5-7 |
| --- | --- |

An example for reliability allocation using FOO method is shown in Table   5 -14. The system level reliability target of 0.9 is distributed to sub-systems based on weighting factor obtained from Equation   5 -7.

Table 5-14 Example of graded allocation of reliability targets to sub-systems

| Sub-system | Categories grading values gij |  |  |  | Weighting Factor wi | Sub System Target Rit |
| --- | --- | --- | --- | --- | --- | --- |
|  | Complexity | Technology Level | Operation time | Environmental Conditions |  |  |
| Power | 6 | 5 | 5 | 5 | 0.12550 | 0.98686 |
| Tele-Command/ Telemetry | 6 | 4 | 7 | 6 | 0.16867 | 0.98239 |
| Propulsion | 5 | 6 | 5 | 5 | 0.12550 | 0.98686 |
| Orbit Control | 8 | 8 | 5 | 7 | 0.37483 | 0.96128 |
| Structure | 4 | 2 | 10 | 8 | 0.10710 | 0.98878 |
| Pay load | 7 | 6 | 7 | 2 | 0.09839 | 0.98969 |
| System level |  |  |  |  | 1.0 | 0.90 |

### Reliability assessment for system architecture development during conceptual design

The outcome of the conceptual design phase are concepts that will be implemented during the next stages of the system development. The selection of the preferred system architecture is essentially a trade-off among the various architecture options. According to ECSS-E-ST-10C [BR2], a trade-report should contain the result of the evaluation of every identified alternative design solution with regard to the key technical requirements. For each alternative design solution, the following should be performed:

- Assessment of all the key technical requirements / evaluation criteria,

- Presentation of the pros and cons of the design solution, and

- Identification of the technical and programmatic risks.

The reliability prediction is an important part of the trade-off studies and could be applied to support system engineering, subsystem engineering and equipment level design engineering. It provides a quantitative assessment of alternative design solutions regarding the achievement of reliability requirements. In the trade-off study, a sensitivity analysis can support system engineering through quantifying how reliability on system level changes if certain parameters change. Importance measures (see Chapter 12 Part III) could be used to perform such a sensitivity analysis. Furthermore, the reliability prediction for the trade-off should identify the equipment failure modes that significantly impact the system reliability. If the correlation between equipment failure modes and system reliability can be established, the aim of system reliability improvement is to eliminate or significantly reduced these failure modes by improving equipment quality or reconfiguring the system architecture. Different design concepts may cause different types of failure modes. The system designers should be aware of the underlying failure causes to achieve a robust design. To achieve the desired quality, the development process should be accompanied by an appropriate quality assurance procedure, see e.g. ECSS-Q-ST-10C [BR3].

### Reliability requirement verification for compliance demonstration

The verification activities should demonstrate that the design and architecture is compliant with the reliability requirements for the corresponding level. This includes to demonstrate that:

All requirements have been taken into account and that the design is compliant with the requirements.

There is sufficient confidence that the final product will meet the requirements.

There is sufficient confidence in the correctness of the design on system level so that the specification and design of the next lower level can progress.

The verification activities should demonstrate that the design and architecture is compliant with the reliability requirements for the corresponding level. But the verification should also demonstrate that the sub-systems on the next lower level are compliant with the requirements for the next lower level. It is important that compliance of each building block with the requirements is substantiated and that potential non-compliances are identified.

For reliability requirements, the verification is performed through quantitative analysis, see Chapter 12 Part III. To finally demonstrate compliance with the top level reliability requirements, the final reliability prediction should be performed when the data for the sub-systems and component failure behaviour are available. Depending on the performance requirements, a consideration of degraded system operability might be useful.

### Reliability assessment for life time extension and safe disposal

To support life time extension decisions, the reliability prediction may need to be updated and in particular the reliability of the functions for safe disposal has to be considered to demonstrate compliance with space debris mitigation requirements, see also chapter 5.3.2.6. The following aspects should be considered:

- If the life extension exceeds the qualification time, the assumptions made in the beginning of the project have to be revisited.

- Wear-out may need to be considered for life time beyond qualification time.

- The result of in-orbit testing and reliability estimation based on in-orbit feedback have to be taken into account to support decision on life time extension.

It is important to note that the probability of success of safe disposal has to be demonstrated already during the design phase as part of the space debris mitigation requirements. The analysis for safe disposal is then reassessed if the lifetime in-orbit is completed and should be further extended or a failure occurred during the lifetime. That means the requirements for safe disposal determine to what extent a life time extension is possible.

- Part I Bibliography

Bibliographic references for Part I

- ECSS Space project management – Project planning and implementation
ECSS-M-ST-10C Rev. 1, March 2009

- ECSS Space Engineering – System engineering general requirements
ECSS-E-ST-10C Rev.1, February 2017

- ECSS Space Engineering - Product assurance management
ECSS-Q-ST-10C, Rev 1, 15 Mar 2016

- Analytical method for mapping function to failure during high-risk component development. I. Y. Tumer, R. B. Stone. Proceedings of the Design Engineering Technical Conferences. 2001.

Reference Documents

The following list is repeated from the Handbook Preface, using the same numbering of Reference Documents across all Parts.

- Space product assurance – Dependability
ECSS-Q-ST-30C Rev.1, February 2017

- Space product assurance – Failure modes, effects (and criticality) analysis (FMEA/FMECA)
ECSS-Q-ST-30-02C, 6 March 2009

- Space Engineering -– Fault Tree Analysis 
ECSS-Q-ST-40-12C, 31 July 2008

- Space product assurance – Components reliability data sources and their use
ECSS-Q-HB-30-08A

- Space engineering – Structural general requirements
ECSS-E-ST-32C Rev. 1

- Space engineering - Structural factors of safety for spaceflight hardware
ECSS-E-ST-32-10C Rev. 1

- Space engineering – Mechanisms
ECSS-E-ST-33-01C Rev. 2

- ECSS system – Glossary of terms
ECSS-S-ST-00-01C

- Standard Framework for Reliability Prediction of Hardware
IEEE Std 1413TM-2010

- Reliability Methodology for Electronic Systems
UTE C80-811, FIDES Guide 2009 Ed A Rev 2010

- Reliability Prediction Models (Quanterion),
HDBK-217Plus:2015 Notice 1 – Handbook of 217PlusTM

- Reliability prediction of Electronic Equipment
MIL-HDBK-217F Notice 2 (1995)

- Electronic Reliability Design Handbook, Dept. Defence. 
MIL-HDBK-338B (1988)

- Nonelectronic Parts Reliability Data (Quanterion)
NPRD-16 (2016)

- Handbook of Reliability Prediction Procedures for Mechanical Equipment
NSWC-11 (Naval Surface Warfare Center, 2011)

- Web-Accessible Repository of Physics-based Models (RiAC)
http://www.theriac.org/WARP/index.php, accessed 08.04.2020

- TEC-QQD Study – Reliability Prediction Data Sources and Methodologies for Space Applications
Final Report, ADS-RPDSM-FR, dated 23/8/2016

- TEC-QQD Study – Reliability Prediction of Mechanical Systems & Parts
Final Report, AO/1-8017/14/NL/KM, dated 26.08.2016

- TEC-QQD Study – Use of quantitative reliability requirements for space applications
Final Report, TASF-RAMS-0010, dated 18Feb16