| Stormborn |  |  |  |
| --- | --- | --- | --- |
| Adaptation of the NRPM Handbook into an ESA Handbook following the ECSS |  |  |  |
| MEC Handbook |  |  |  |
| Part of TN-02 |  |  |  |
| Consortium: |  |  |  |
| Matrisk GmbH Alte Obfelderstrasse 50 CH – 8910 Affoltern a.A. Switzerland Tel.: +41 43 340 04 27 contact@matrisk.com www.matrisk.com | Technical Note | Status: Draft | Rev. A |
|  | Classification: confidential | Date: | 2022-02-24 |
|  | ESA Contract: 4000121065/17/NL/PS | Authors / Contributors | BOURBOUSE S. (ADS) SCHUBERT M. (Matrisk) |
|  |  | Checked: |  |
|  |  | Approved esa: |  |
|  |  | Pages: | 10 |
| Send to | Revision: | Date: | By: |

# Change Log

| ESA Handbook issue 1A July 2022 | First issue |
| --- | --- |

# Table of Contents

#### Requirements for the Table of Contents

Expected contents:

The table of contents is included for a quick reference to the contents of the standard. It includes the sections and clauses in the order that they appear in the standard, and then a list of Figures and a list of Tables.

Requirements:

REQ_TOC_01: The table of content shall include first all the sections and clauses in order in which they appear in the [handbook], and then a list of figures and a list of tables.

REQ_TOC_02: The table of contents shall be automatically (electronically) generated.

REQ_TOC_03: The table of contents shall not contain requirements.

REQ_TOC_04: The layout of the table of contents shall be as in the following example.

Example:

Change log……………………………………………………………………….3

Introduction……………………………………………………………………..8

1. Scope……………………………………………………………………………9

2. Normative references………………………………………………….11

3. Terms, definitions and abbreviated terms……………………13

3.1 Terms from other standards/handbooks……………………13

3.2 Terms specific to the present handbook…………………….14

3.3 Abbreviated terms and symbols…………………………………16

3.4 Conventions and nomenclature………………………………….18

3.4.1 Convention………………………………………………………………18

3.4.2 Nomenclature………………………………………………………….19

4. General Methodology…………………………………………………..20

5. Detailed analysis……………………………………………………………23

6. Conclusion……………………………………………………………………150

ANNEX A…………………………………………………………………………..156

ANNEX B……………………………………………………………………………170

Bibliography………………………………………………………………………211

Figures

Figure 1-1: a correctly formatted entry……………………………….12

Figure 1-2: example…………………………………………………………….13

Tables

Table 1-1: a correctly formatted entry……………………………….20

Table 1-2: example…………………………………………………………….25

Table A-1: example 2……………………………………………………….157

Table of Contents	2

1	Introduction	3

1.1	Study overview	3

1.2	Logic and content of this Technical Note	4

1.3	Documentation	4

1.3.1	Reference Documents	4

1.3.2	Applicable Documents	6

2	Survey	7

3	Requirements	9

3.1	Failures taxonomy	9

3.1.1	Insurers data	10

ADD TABLE OF FIGURES

ADD TABLE OF TABLES

10	Reliability prediction for mechanical and structural parts and assemblies	III-257

10.1	Process for mechanical reliability prediction	III-260

10.2	Root causes coverage	III-261

10.2.1	Random failures	III-261

10.2.2	Wear-out failures	III-262

10.2.3	Extrinsic failures	III-263

10.2.4	Systematic failures	III-265

10.3	Data sources and methods	III-266

10.3.1	Inputs	III-266

10.3.1.1	Analysis of the failure mechanisms	III-266

10.3.1.2	In-orbit return data	III-266

10.3.1.3	Manufacturer reliability data	III-267

10.3.1.4	Test data	III-267

10.3.1.5	Models from existing data sources	III-268

10.3.2	Methods	III-270

10.3.2.1	Statistical methods	III-271

10.3.2.2	Physics of Failure methods	III-272

10.3.2.3	Handbook methods	III-272

10.3.2.4	Combined approach	III-272

10.4	Failure mechanisms identification and focus	III-273

10.4.1	Failure dominance and focus of the prediction	III-273

10.4.2	Failure mechanisms completion	III-275

10.5	Method selection for different items and failure mechanisms	III-278

10.5.1	Categories and combinations of mechanical failure mechanisms	III-278

10.5.2	Selection of methods for part level mechanical reliability prediction	III-279

10.5.3	Selection of methods for assembly level mechanical reliability prediction	III-282

10.6	Structural reliability methods and inputs	III-283

10.6.1	Structural reliability methods for mechanical reliability prediction	III-283

10.6.2	Simplified structural reliability modelling with analytic solutions	III-284

10.6.2.1	Generic approach for the formulation of the limit state function	III-284

10.6.2.2	General process for the application of the simplified structural reliability method	III-286

10.6.3	Stress strength methods for structural items	III-287

10.6.4	Basic variable modelling for mechanical reliability applications	III-290

10.6.4.1	Modelling of strength variables	III-291

10.6.4.2	Modelling of stress variables	III-292

10.6.5	Bayesian updating of structural reliability estimates	III-294

10.7	Structural reliability models and equations	III-299

10.7.1	Modelling of failures due to mechanical wear	III-300

10.7.1.1	Adhesive wear	III-301

10.7.1.2	Solid lubricant wear	III-304

10.7.1.3	Fluid lubricant wear	III-306

10.7.2	Modelling of failures due to fatigue	III-309

10.7.2.1	High cycle fatigue modelling with the S/N curve approach	III-310

10.7.2.2	Fatigue crack growth modelling	III-315

10.7.3	Modelling of failures due to corrosion	III-320

10.7.4	Modelling of failures due to material degradation	III-321

10.7.4.1	Radiation degradation modelling	III-321

10.8	Examples	III-323

10.8.1	Simplified wear modelling: Solid lubricant wear	III-323

10.8.2	Updating of reliability estimates derived from structural reliability methods	III-324

10.8.3	Updating of structural reliability methods using right censored data	III-328

10.9	MEC Annex A: Bibliography for the Mechanical chapter	III-331

# Introduction

From NRPM: Goal of this chapter: To provide guidance (methodology, methods and models) for the modelling of miscellaneous items, as defined hereafter.

#### Requirements for the Introduction

Expected contents:

Introduction is an optional element, and therefore this section can be deleted if not used. The following are examples of subjects that can be included in the Introduction:

- Specific description on the technical content

- Reasons prompting the preparation or the update of the [handbook]

- History of the [handbook]

- Relation with other ECSS standard [or handbooks]

REQ_IN_02: The introduction, if present, shall: 1. Not contain sub-sections and sub-headers and 2. Have the layout of the “introduction” of the following example.

# Scope

xxxx

#### Requirements for the Scope

Expected contents:

The purpose of the scope clause is to describe, in a few lines, the [handbook] as a whole in such a way that the reader can assess if the [handbook] is applicable or not to a particular case. The scope clause does not intend to recreate an expanded version of the TOC, chapter by chapter.

With this view, only a brief description of the subject of the [handbook] and statements of applicability are expected to appear in the scope clause. Statements of applicability state explicitly in which kind of projects or situations the standard is applicable (and for which is not applicable).

Any other descriptive material can be included in the introduction, e.g. history of the standard, rationale behind its preparation or relationships with other standards.

Requirements:

REQ_SC_01: The scope clause shall only describe: 1. The subject of the [handbook] and aspects covered by it and 2. The applicability of the [handbook].

REQ_SC_02: The scope clause shall not contain requirements.

REQ_SC_03: The scope clause shall have the layout of the “scope” of the example.

Example:

The present template sets the actual layout and specifies the requirements for drafting an ECSS handbook. It is applicable for the draft of the ESA handbook related to the reliability.space study.

It is only applicable for this specified handbook.

# Normative References

TO BE TAKEN FROM TN8

#### Requirements for Normative References

Expected contents:

The purpose of the Normative References clause is to list the documents referred to in requirements. Availability of the documents referred to is, which are therefore necessary to apply the [handbook] in drafting.

Documents called for the non-normative parts are included in the bibliography section.

Requirements:

REQ_NR_01: The “normative references” clause shall include an introductory sentence containing the following text:

“The following normative documents contain provisions which, through reference in the text, constitute provisions of this ECSS [handbook]. For dated references, subsequent amendments to, or revisions of any of these publications do not apply. However, parties to agreements based on this ECSS [handbook] are encouraged to investigate the possibility of applying the most recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.”

REQ_NR_02: The “normative references” clause shall list only the documents which 1. In whole or in part, are made applicable by explicit references in normative statements (requirements, recommendations and permissions) and 2. Contain definitions made applicable to this [handbook] (i.e. included in clause 3.1 “terms from other standards/handbooks”).

REQ_NR_03: Normative references to non-ECSS documents shall be dated, or the number of the version included.

REQ_NR_04: Normative references to ECSS documents shall be undated.

REQ_NR_05: The documents listed in “normative references” shall be published and publicly available. Note: publicly available does not necessarily mean available for free.

REQ_NR_06: The normative references clause shall not contain requirements.

REQ_NR_07: The normative references clause shall have the layout of the normative references of the example.

Example:

The following normative documents contain provisions which, through reference in the text, constitute provisions of this ECSS [handbook]. For dated references, subsequent amendments to, or revisions of any of these publications do not apply. However, parties to agreements based on this ECSS [handbook] are encouraged to investigate the possibility of applying the most recent editions of the normative documents indicated below. For undated references, the latest edition of the publication referred to applies.

ECSS-S-ST-00-01	  ECSS system – Glossary of terms

ECSS-D-00-01          Drafting rules and template for ECSS standards

## Bibliography for the Miscellaneous chapter (SEE WHERE TO PUT IT)

DFMEA, PFMEA
SAE J1739 (2009)

Bruceton statistical method
GTPS 11C (2011)

Statistical Method of PROBIT
GTPS 11A (2014)

Severe test statistical method
GTPS 11F (2012)

One-shot Statistical method
GTPS 11B (2002)

Improvements to the Neyer method of estimating reliability of one-shot systems used in pyrotechnic
LambdaMu n°20 (2016)

Process Failure Mode and Effects Analysis (PFMEA) and Control Plans

- AS13004™

# Terms, definitions and abbreviated terms

TO BE ADAPTED FROM WHAT IS IN TN8

## Terms from other standards

#### Requirements for Terms from other standards

Expected contents:

The purpose of the “Terms from other standards” clause is to list the terms used in requirements in the current document that are already defined in the ECSS-S-ST-00-01 “ECSS system - glossary of terms” or in the other ECSS standards.

Requirements:

REQ_TE_01: The “terms from other standards/handbooks” clause shall include: 1. The sentence: “for the purpose of this [handbook], the terms and definitions from ECSS-ST-00-01 apply, in particular for the following terms.” And 2. The terms from ECSS-S-ST-00-01 used.

REQ_TE_02: If terms from other ECSS standards/handbooks are used, for each ECSS standard/handbook used a sub-clause shall be added and include 1. The sentence “for the purpose of this [handbook], the following terms and definitions from “reference of the [handbook]” apply, and 2. The terms from the ECSS standard.

REQ_TE_03: The layout of the “terms from other standards/handbooks” clause is compliant with the example below.

REQ_TE_04: The “terms from other standards/handbooks” clause shall not contain requirements.

Example:

For the purpose of this handbook, the terms and definitions from ECSS-S-ST-00-01 apply, in particular for the following terms:

      - Recommendation

      - Requirement

      - Standard

      - State of the art

## Requirements for Terms specific for the present handbook

Expected contents:

The purpose of the “terms specific for the present handbook” clause is to define terms specific to the [handbook].

Requirements:

REQ_SP_01: The following logic for deciding whether a term needs defining shall apply:

1. The term is either used in a requirement or in the definition of another term. (Y/N)

If Yes, go to 2. If No, then informative term are explained in an informative Section 4 of this document

2. The term is defined in the Oxford English Dictionary. And the definition is sufficient (Y/N)

If Yes, no further action is needed. If no, go to 3.

3. The term is defined in ECSS-S-ST-00-01 “ECSS Glossary” and the definition is suitable (Y/N)

If Yes, do not generate new entry in 3.2, instead list term in 3.1. If No, go to 4.

4. The term is already defined elsewhere in ECSS and the definition is suitable (Y/N)

If Yes, list term in 3.1 and prepare a proposition for the raise of a CR to move the definition to ECSS-S-ST-00-01” ECSS Glossary for the case the handbook becomes an ECSS handbook  eventually. If No, go to 5.

5. The term is already defined in an SDO document and the definition is sufficient (Y/N)

If Yes, include term and definition in Section 3.2 with reference to SDO source. If No, go to REQ_SP_02.

REQ_SP_02: In creating a term, the following shall apply; 1. Terms is to be in the singular form and 2. Term consisting of more than one word is only valid when the definition provided is other than the collective definitions of the words of the term and 3. Synonymous terms are not to be defined and 4. A term cannot be a unit or a physical phenomenon already commonly defined.

REQ_SP_03: In creating a definition, the following shall apply: 1. A definition provides the meaning of a term; Note: the objective is to provide the meaning of the term and not to detail a specification or explanation, and 2. A definition is a partial single sentence; Note: the sentence starts in the lower case and is not punctuated at the end of the sentence, and 3. A definition is worded so it can be directly substituted into the text instead of the term; Note: a definition that fails this criteria may be a convention that is entered in clause 3.4 and not clause 3.2. And 4. The definition cannot contain formal verbs “shall”, “should”, “may” and “need not” as defined in 3.4.2, and 5. A definition of a term does not repeat the term, nor any word used in the term, and 6. If 5. Cannot be met, all or part of a term may be used in the definition where the term is a category (e.g. “Class 1 change” is a category of “change” with a unique definition in ECSS).

REQ_SP_04: Section 3.2 formatting shall comply with 1. Definitions being listed in alphabetical order, and 2. Words used in a definition that are existing ECSS defined terms appear in bold, and 3. Abbreviation of the term is not included after the term; Note: abbreviations are listed separately in “Abbreviated terms” (see 3.3) and 4. The layout of the “terms specific to the present handbook” clause is compliant with the example.

Example:

Recurring product

Product which conforms to a qualified design and is produced according to the corresponding production master file.

## Requirements for Abbreviated Terms and Symbols

Expected contents:

The purpose of the “abbreviated terms and symbols” clause is to include in a single list:

- The abbreviations used uniquely in the [handbook], and their expanded meaning

- The symbol used in the standard, and their meaning.

Requirements:

REQ_AB_01: The list of abbreviated terms and symbols shall be introduced by the following boilerplate sentence: “for the purpose of this [handbook], the abbreviated terms and symbols of ECSS-S-ST-00-01 and the following apply:”

REQ_AB_02: A term from which an abbreviation is derived shall be understood as a collection of words which together represent an understandable single concept or idea.

REQ_AB_03: An abbreviated term or a symbol shall be defined and listed in this clause 3.3 when 1. It is used in the [handbook], and 2. Its meaning is not included every time that the abbreviated term or the symbol is used and 3. The use of an abbreviation assists with the formatting, reading and understanding of the document.

REQ_AB_04: The same abbreviated term or symbol shall not be used within a document for two different concepts.

REQ_AB_05: The abbreviated term or the symbol shall appear in bold and the expanded meaning shall be written using lowercase letters (unless they are proper names) and in regular.

REQ_AB_06: For abbreviations derived from an ECSS defined terms, the expansion is given in italic font.

REQ_AB_07: The layout of the “abbreviated terms and symbols” shall be as in the example provided in the example.

REQ_AB_08: The “abbreviated terms and symbols” clause shall not include requirements.

REQ_AB_09: Abbreviated terms already listed in ECSS-S-ST-00-01 “ECSS system – glossary of terms” are not to be repeated in the [handbook] where these have the same meaning as in ECSS-S-ST-00-01.

REQ_AB_10: The same abbreviation shall not be used within a document for two different terms.

REQ_AB_11: Abbreviations shall consist of 1. At least two consecutive letters or more, 2. Taken from the first letter of each term they are replacing, and 3. Letters.

REQ_AB_12: Abbreviations shall not 1. Be a statement of a unit (e.g. N = Newton), 2. Contain another abbreviation, 3. Consist of characters other than letters from the English alphabet, 4. Contain slashes or parenthesis.

REQ_AB_13: Parameters may be defined as Abbreviations

REQ_AB_14: Abbreviations shall be in line with normal industry practice and be commonly applied.

Example:

For the purpose of this standard, the abbreviated terms and symbols from ECSS-S-ST-00-01 and the following apply:

Abbreviation	     Meaning

DRD		     document requirements definition

ECSS		    European Cooperation for Space Standardization

ITT		    invitation to order

SEP	             system engineering plan

## Requirements for Convention and Nomenclature

Expected contents:

The purpose of the “convention” clause is to explain the specific conventions used in the standard, if any.

A convention provides a “rule-set”, generally in the form of a formulae, typology and logic, that establishes the relationship between at term and desired outcome or process. Examples would be:

- Graph indicating the relationship of terms expressed on the axes

- Equation or formulae expressing the terms and their relationship

- Choice of units for the significant terms or associated

This clause also includes the mandatory aspect of “nomenclature” where the meaning of the formal words that are used in ECSS to differentiate between informative and normative provisions.

Requirements:

REQ_CO_01: If specific conventions not included in the present requirements are used within the standard, these conventions shall be explained in clause 3.4.

REQ_CO_02: A convention shall provide a “rule-set” (formulae, typology or logic) that establishes the relationship between a term and desired outcome or process.

REQ_CO_03: A convention shall state the formal relationship between entities (e.g. an expression of an equation).

REQ_CO_04: A convention shall not comply with 3.2 REQ_SP_03. Note: a convention differs from a definition, as a convention is to be understood as means to express how something shall be derived whilst a definition is a statement of the exact meaning of a term.

REQ_CO_05: A convention shall provide a reference or source, from which this rule-set is adopted in other than ECSS.

REQ_CO_06: Each convention shall be included in a separate sub-clause within clause 3.4.

REQ_CO_07: The “conventions” clause shall not include requirements.

Hereafter is the table of the EPPL components:

# Reliability prediction for mechanical and structural parts and assemblies

Goal of this chapter: To provide guidance (methodology, methods and models) for the modelling of mechanical and structural parts and assemblies.

For the four different sections (System, EEE, Mechanical and Miscellaneous), the same principle will be applied with for each section a clear definition of the objective(s) and then a clear explanation of the choices. When relevant, examples will be used to illustrate.

The level of recommendation/rule or permission will be clearly indicated so that it is easy for the user to adapt its application of the method based on its needs.

## Process for mechanical reliability prediction

The approach for mechanical reliability prediction in space applications presented in this chapter is based on the following three steps, which are described in different Sections:

- Failure mechanism identification and focus (Section 10.4)
The goal of this step is to decide what needs to be modelled, allowing to focus the analysis on items and failure mechanisms with non-negligible probability of failure.

- Selection of suitable methods and models (Section 10.5)
The method selection is centred around the dominating failure mechanisms identified in the first step with a focus on physics of failure methods for reliability prediction. A more general overview considering all potential methods and inputs is found in Section 10.3.

- Method implementation (Sections 10.6 and 10.7, Section 10.8 for examples)
Methods and Models to derive reliability estimates for different failure mechanisms are discussed with a focus on Physics of Failure based methods. The implementation of other methods (e.g. statistical methods for a data-based approach) is discussed in Section 10.3

Before addressing these three steps, the following information is provided:

- Section 10.2 contains a discussion of the different failure root causes (as classified in Chapter 4 Part I) and their coverage during Mechanical reliability prediction.

- Section 10.3 contains a discussion of the different reliability prediction methods and inputs introduced in Chapter 6 (Part II).

## Root causes coverage

Table 10-1 gives an overview of the failure categories defined in Chapter 4 (Part I) and their coverage by the methods described in the present chapter. In the following subsections, the interpretation of each failure category within the mechanical chapter is briefly explained.

Table 4-1: Coverage of failure root causes by the mechanical reliability prediction methodology presented in this chapter

| Root cause category | Coverage by the mechanical reliability prediction methodology |
| --- | --- |
| Random failures | Considered by the methods described in Section 10.5, including early wear-out failures and failures due to foreseen extrinsic stresses, provided that these cannot be explained by gross design or manufacturing errors (cf. systematic failures). |
| Wear-out failures | Considered by the methods described in Section 10.5, especially by those based on structural reliability theory with time-dependent degradation models. Safe life qualification alone is not sufficient as a justification to discard the probability of premature mechanical wear-out failures before the end of the nominal lifetime. The consideration of additional failure mechanisms, which are assumed to be effectively handled during nominal lifetime, may be required in case of lifetime extensions (using the same methods, but with an extended modelling scope). |
| Extrinsic failures | Foreseen extrinsic effects due to the launch environment (vibrations, shocks) and physical conditions in-orbit (e.g. temperature) should be considered as stress contributors in the modelling of random and wear-out failures, see Table 10-2 in Section 10.2.3 for the most relevant interactions. A separate modelling may be required if the probability of additional failure mechanisms induced by the space environment cannot be neglected – e.g. space debris impact, see Section 10.2.3 for details. |
| Systematic failures | Process related failures due to minor design and manufacturing weaknesses can be considered with the methods described in Section 10.5, provided that the failure process or mechanism remains unchanged (cf. random failures). Gross design or manufacturing errors must be considered separately, see Section 12.9 in the System level chapter. |

### Random failures

The Physics of Failure (PoF) of mechanical parts are mostly well-understood and give strong indications also on the failure root cause. Nevertheless, from a modelling perspective it may be meaningful not to go into the details of the root cause that has led to a certain failure mechanism. To give an example, a premature bearing wear failure may be due to a residual undetected part level defect or weakness (random), due to “normal” aging or wear-out (wear-out), induced by environmental effects such as thermal loads leading to distortion (extrinsic) or due to a misalignment at assembly level (systematic). Despite the difference in the failure root cause, all these failures are due to the same failure mechanism, which can be modelled with the same physical law.

In this line of thought, all failures due to minor uncertainties in the design and due to manufacturing variability will be considered as random failures for mechanical items, whereas gross errors, possibly leading to new failure mechanisms and unforeseen events, are considered as systematic failures. With this classification, it is possible to consider the contribution of design and manufacturing process variability in the mechanical reliability prediction, e.g. by increasing the variability of the basic variables when using structural reliability methods as discussed in Section 10.6.4. Only gross errors, classified as systematic failures, need to be considered separately.

    - Wear-out failures

In the frame of this handbook, wear-out failures are generally assumed to occur after the specified mission life time, assuming that safe life qualification processes have successfully mitigated the risk of premature failures. However, for mechanical items, this assumption may not necessarily be valid, as experience shows that there is a certain risk of premature wear-out despite safe life qualification, which may lead to a (slowly) increasing failure rate already during the course of the mission, as illustrated in Figure 10-1. The root cause of these premature wear-out failures will mostly be related to the design or manufacturing process. However, following the same line of thought as in Section 10.2.1 above, failures due to minor design or manufacturing variability can nevertheless be considered as wear-out rather than systematic failures, and considered with the methodology presented in this chapter. Only wear-out failures due to gross design or manufacturing errors are left for the systematic failure modelling.

Figure 10-1: Failure rate as a function of time for typical mechanical components.

When exceeding the qualification time, e.g. for a satellite life time extension, the modelling of wear-out failures becomes even more relevant. Additional degradation mechanisms whose effect is negligible during the specified lifetime may need to be considered, and failures due to all degradation mechanisms become more likely.

Section 10.4.1 contains guidance on the identification of dominant failure mechanisms – including wear-out mechanisms - for several part types, though focusing on reliability prediction for the design lifetime, i.e. within safe life qualification domain. Failure mechanisms that are normally considered effectively handled by qualification, such as e.g. radiation degradation of optical equipment, may need to be added when considering a life time extension.

    - Extrinsic failures

External effects acting as loads on mechanical parts are considered during the design, generally reducing the risk of extrinsic failures. To quantify the remaining risk, reliability calculations must consider relevant extrinsic contributors.

For mechanical reliability assessment, the following aspects need to be considered:

- Where possible, the failure modes related to the environmental condition shall be mitigated by design. For example, outgassing in vacuum conditions can be reduced by careful material selection; solar wind induced effects can be mitigated by grounding protection etc. Errors in the design solutions or specification are classified as systematic failures.

- Similarly, failures due to UV degradation, radiation, or atomic oxygen effects within the duration of the nominal mission life are considered as systematic failures. For life time extensions, failures due to these cumulative effects may have to be considered in the reliability prediction (classified as wear-out failures).

- Failure mechanisms considered during the reliability prediction can be driven by a combination of environmental parameters. For example, extreme thermal loads affect material fatigue and fracture properties; thermal parameters define the viscosity of the lubrication, thus affecting the friction and lubrication wear. To account for the relevant environmental parameters, the defining variables must be adjusted accordingly.

The extrinsic effects contributing most to mechanical failure mechanisms are summarized in Table 4-2. The full list of the environments which might cause extrinsic failures is presented in Chapter 4 (Part I).

Table 4-2: Environments with the highest contribution to the extrinsic failures.

| Environment | Failure mechanism |
| --- | --- |
| Vibrations | distortion, fatigue, fracture |
| Thermal loads | distortion, lubrication wear, thermal fatigue |
| Shocks | distortion, fracture |
| Space debris impact | Ultimate failure (in the failure mode sense) |

Accounting for these effects with the methodology for mechanical reliability prediction presented in this chapter is possible as long as the failure modes and mechanisms considered remain unchanged. An example for an extrinsic failure root cause that introduces an additional failure mode is space debris (or micro meteorite) impact, which will be discussed in the remainder of this section.

Space debris and micrometeoroid impact

Extrinsic failures due space debris or micrometeoroid impact are typically relevant for components with significant exposed area (e.g. reflectors, solar arrays). Accounting for these failures may require a dedicated modelling if it cannot be shown that the impact on mission reliability is negligible.

The calculation is performed in the following three steps:

- Determine the debris and meteoroids environment (average flux as a function of particle size, velocity distribution) based on the mission parameters [BR93], e.g. using tools like ESA MASTER 2009 [BR94] to assess the debris or meteoroid flux on a spacecraft on an arbitrary earth orbit.

- Obtain minimum critical particle size for each direction of the component on the spacecraft, considering directionality and velocity of the impact. The term “critical” relates to the effects of an impact on the satellite elements, e.g. penetration of the shielding leading to damage of shielded elements.

- Combine the results from the previous steps to calculate the probability of collision and number of collisions with the particles larger or equal to the critical particle size, see e.g. ECSS‐E‐ST‐10‐04C [BR93].

Collision failures can be neglected in the frame of reliability prediction if the corresponding probability of failure is at least one order of magnitude smaller than the probability of failure due to other failure root causes.

Space debris and micro meteoroid impact will not be discussed any further in the present chapter.

    - Systematic failures

Failures due to design and manufacturing weaknesses can to a certain extent be considered with the methodology presented in this chapter, provided that the failure process or mechanism remains unchanged. Failures due to gross errors during design, manufacturing or operations are discussed in Section 12.9. They are out of scope for the present chapter.

## Data sources and methods

The following subsections discuss the inputs and reliability prediction methods introduced in Chapter 6 (Part II), focussing on their use in mechanical domain.

### Inputs

The order of preference for the use of different data sources and inputs for mechanical parts reliability prediction is as follows:

- In-orbit return data

- Manufacturer reliability data

- Test data

- Analysis of the failure mechanisms

- Models from existing data sources

Each input will be briefly discussed below from a mechanical reliability prediction perspective.

Using a data source for reliability prediction – e.g. in-orbit return as the first preference – requires that relevant data is available in sufficient quantity. If this condition is not fulfilled, the next preferred input needs to be considered. Nevertheless, relevant data, even if not available in large enough quantities, can still be used to validate, tune or update the modelling in a combined approach.

Despite its late appearance in the preference listing given above, a good understanding of the failure mechanisms driving the (un-)reliability of mechanical parts should always form the basis for the reliability prediction, even if it is only used to decide whether the available data can be used for the prediction.

4.3.1.1 Analysis of the failure mechanisms

A first analysis and understanding of the potential failure mechanisms for mechanical parts and equipment should generally be available from the design process and should always be considered as an important input for the prediction, even if a pure statistical approach is used to estimate an item’s reliability. The approach described in Sections 10.4 and 10.5 is based on this idea, focussing on the identification of relevant failure mechanisms although not necessarily requiring Physics of Failure methods for the mathematical modelling.

4.3.1.2 In-orbit return data

In-orbit return failure data provides valuable information on the in-service reliability performance in space applications. For mechanical equipment, it may in certain cases be possible to track the failure cause down to part level, making the information even more valuable. However, the use of in-orbit return data for reliability prediction is hindered by the small degree of standardization of mechanical items (compared to EEE) and the large effect of the design details and the operational conditions on mechanical part reliability. As a stand-alone input, it will thus be restricted to highly standardized applications. Apart from this, in-orbit return can provide the following information when combined with other inputs:

- Information on observed failure modes and root causes to select the focus of the prediction

- In-orbit measurements of physical data to model the relevant variables in PoF approaches

- Failure/success data to update a model based on other inputs

4.3.1.3 Manufacturer reliability data

Mechanical space equipment suppliers are highly specialized companies, delivering their products for many different missions. Manufacturer reliability data can thus be a highly relevant source of information for mechanical reliability prediction. This holds especially for standardized items with a long space heritage, where manufacturer’s in-orbit return (considered in Section 10.3.1.2) can be used for the reliability prediction. Also test data available on the manufacturer’s side may be considered. Manufacturer reliability data derived from analysis should be checked for consistency with the methodology presented in this handbook.

Apart from this, more general manufacturer data, such as design information, material specifications, or equipment level FMEA analyses are also important inputs for reliability predictions of mechanical parts.

4.3.1.4 Test data

Test data for mechanical parts tested at operational conditions would be a relevant source of information for reliability prediction, but is in most cases not available in large enough quantities. Small data samples can be used to calibrate or update models derived from analysis, or to check the modelling assumptions.

Another possible use of test data is for the derivation of physical quantities needed in reliability models based on PoF, such as e.g. wear rates or resistances against specific stresses. The advantage is that with this approach test data can be useful even if the test is performed at different than operational conditions and/or the design is not exactly the same as for the flight item. An important limitation is that in practice most items are not tested to failure and therefore the data is censored and less informative for modelling purposes.

Finally, one specific feature of pre-flight acceptance tests should be mentioned: Due to the fact that these tests are performed on the flight item, they can provide relevant information about its strength or resistance against the applied stresses. The fact that the item has passed the tests may be used to exclude lower fractile values of the strength distribution in PoF approaches. Note that this “proof load” approach is only valid for failure mechanisms resulting from peak load effects, and cannot be applied when cumulative stresses are driving failure. More details on the use of proof load test results for reliability prediction can be found e.g. in [RD18] , [BR64].

4.3.1.5 Models from existing data sources

Two well-known reliability handbooks are presently available providing input data for mechanical reliability prediction: the NPRD [RD14] and the NSWC [RD15] . Both handbooks cannot be recommended as a stand-alone method for mechanical reliability prediction in space applications, but may be used as a complementary source of information, as will be discussed in the following.

Non-electronic parts reliability data (NPRD)

NPRD [RD14] is a data source providing part and assembly level failure rate estimates from various sources and industries. Its application is straight-forward when used without extrapolation beyond the covered part types, operational and environmental conditions. Due to the following limitations, it cannot be used as a general reliability data source for mechanical reliability prediction in space applications:

- The coverage of part types used in space and space-graded parts is limited

- Most data come from terrestrial sources, with limited coverage of space applications

- The information provided is not sufficient to justify extrapolations beyond data coverage

- For assemblies listed, there can be difficulties of interpreting whether the failure rate provided relates to the assembly as a whole, or only to one of its components

- Only industry- and time-averaged failure rate estimates are provided due to the way the data has been collected, which does not allow to derive time-dependant failure rates

- It is unclear which categories of failures are included in the data (e.g. random vs. wear-out)

- Parts of the data presented are very old, as data collection started in the early 1970’s

Despite these drawbacks, NPRD may still be used to derive “ballpark” estimates for the reliability of mechanical parts, to be used e.g. in early design phases, as a comparison benchmark for predictions based on other methods, or as a prior that can be updated with relevant data in a Bayesian approach. Due to the large impact of terrestrial data sources on the summary statistics, it is recommended to search for dedicated data from space applications.

The confidence in the NPRD data strongly depends on its relevance with respect to the part type and quality, the use conditions, the environment and the dominating failure mechanisms. It may therefore only be used as a reference for specific applications if sufficient justification can be provided that the data is relevant with respect to the part type and quality, the use conditions, the environment and the considered failure mechanisms.

Naval Surface Warfare Center (NSWC) Reliability Prediction Handbook

The NSWC Handbook for Mechanical Reliability Prediction [RD15] provides models derived from a combined approach similar to the one applied for EEE components in Chapter 9: Base failure rates, derived from statistical data at reference conditions, are scaled to a specific design and operating conditions with the aid of multiplicative factors accounting for the specificities of material, design and environment. The scaling factors are based on PoF relationships to determine the mean time to failure MTTF at reference and operating conditions:

| λ=λref∙λopλref=λref∙MTTFrefMTTFop=λref∙C | Equation 10-1 |
| --- | --- |

The advantage of this approach is the increased flexibility, as generic models for different part “families”, e.g. springs, can be tuned to a wide variety of specific part types and applications. Despite this interesting feature, the direct application of the NSWC models for reliability predictions in space applications is not recommended due to the following limitations:

- Base failure rates are from naval/terrestrial sources only and do not reflect the use conditions nor the quality level of mechanical parts used in space

- The sample size of the data used to derive the base failure rates is unknown and may in some cases be very small [BR65].

- The scaling factors are based on the Physics of Failure in terrestrial applications, which may in some cases not be applicable in space

- Scaling factors are based on deterministic relationships, which may lead to a large bias when the effect of the variable entering the scaling factor is highly nonlinear

- The model output is a constant failure rate, which is often not consistent with the failure mechanisms that have been analysed to derive the scaling factors

As a result, the NSWC should not be used as a data source for mechanical reliability prediction in space applications. The data can only be used to derive “ballpark” estimates, e.g. for early design phases, for comparison with other methods, or as a prior for Bayesian updating.

Beyond this, the NSWC method is recommended only as a scaling method for specific part types, allowing to tune statistical estimates from “space relevant” data sources (the NSWC base failure rates should not be used). The approach is applicable only if the failure mechanisms that are of relevance in space applications are the same as those considered by the NSWC models, see Table 10-6 in Section 10.5.2 for guidance. It is based on the following steps:

- Estimate a new base failure rate with statistical methods using relevant data. The data should be “space relevant” in terms of design, quality level and dominating failure mechanisms, but must not exactly fit the required application.

- Select the parameters from the NSWC Handbook method that are needed to scale the new base failure rate to the considered application.

  - Determine scaling factors for the new base failure rate (new “reference conditions”, implicit in the statistical data used)

  - Determine scaling factors for the considered application (“operational conditions”)

  - Determine ratio between the “operational” and “reference” scaling factors

- The ratios determined in the previous step can now be used as scaling factors to adapt the new base failure rate to the considered application.

It should be noted that only the criticism of the NSWC Handbook regarding the space relevance of the base failure rates can be corrected with this procedure. To avoid or reduce potential biases due to the scaling method, it should not be used in case of large deviations between “reference” and “operating” conditions, and physical variables entering the scaling factor calculations should be estimated as realistic mean values.

The implicit assumption of constant failure rates can be mitigated by fitting a model that allows for time dependent failure rates in the first step. The NSWC scaling method can then be applied to adjust the location parameter of the selected time to failure distribution, e.g. as multiplicative factors for the inverse of the Weibull characteristic life.

### Methods

The following methods can be used for the derivation of reliability estimates for mechanical items:

- Statistical methods for the derivation of a suitable failure time distribution are applicable provided that relevant statistical data is available in sufficient quantity for the mechanical item under consideration and for comparable operating and environmental conditions.

- Physics of Failure methods (PoF) based on structural reliability theory and an analysis of the relevant failure mechanisms are the preferred way of modelling when the available data is inappropriate or not sufficient for a pure statistical approach.

- Handbook methods are not the recommended choice for mechanical reliability prediction in space applications, as has been discussed in Section 10.3.1.5, addressing NPRD and NSWC.

- A combined approach based on one of the methods listed above and Bayesian updating with relevant data (additional and independent from the prior estimate) is the preferred approach whenever suitable data is available, but not in sufficient quantity for a pure statistical approach.

Which method is most appropriate depends on the information available to do the prediction, in particular the availability of suitable reliability (or failure) data, and the engineering knowledge regarding the design, the use conditions and the Physics of Failure. General guidance on method selection is presented in Figure 10-2 below. The upper row in the matrix represents statistical methods, the middle row a combined approach with Bayesian updating, and the lower row either handbook methods or data (NPRD, NSWC) or – generally preferred in the absence of statistical data – structural reliability methods (SRM) based on the Physics of Failures.

More specific recommendations are given in Section 10.5.

Figure 4-2: Method selection for mechanical reliability prediction depending on the available input.

In the following subsections, the implementation of the different methods in mechanical domain is briefly discussed, covering all methods listed above. The remainder of this chapter (Sections 10.4 and following) focuses largely on the Physics of Failure as a basis for mechanical reliability prediction.

4.3.2.1 Statistical methods

Statistical methods for reliability prediction are in principle domain-independent, and do not require any specific considerations for mechanical parts. However, the following two important reliability characteristics of mechanical parts should be kept in mind when doing the analysis:

- Many mechanical items are prone to premature wear-out, requiring a thorough selection of the distributional model. Assuming constant failure rates as a default is not acceptable, especially when the dominating failure mechanisms are known to be time-dependent.

- Mechanical items reliability is strongly dependent on the specific design and use conditions. Data from different applications should thus be treated with care, possibly requiring a dedicated Physics of Failure modelling to understand the impact of the relevant variables.

The use of statistical methods for mechanical reliability prediction is not discussed any further in the present chapter; the reader is referred to Chapter 6 (Part II) for a discussion of these methods.

4.3.2.2 Physics of Failure methods

The use of structural reliability methods for mechanical reliability prediction is discussed in more detail in Sections 10.6 and 10.7. The use of other methods that are based on the Physics of Failure (not described in this chapter) is possible, provided that the failure mechanism modelling is accompanied by a suitable quantification of the relevant uncertainties.

4.3.2.3 Handbook methods

The use of existing reliability handbooks and data sources, especially as a stand-alone method, is generally not recommended for the modelling of mechanical items in space applications. Limitations of the NPRD [RD14] and NSWC  [RD15] handbooks are discussed in Section 10.3.1.5, including some guidance on potential uses of these methods, e.g. to derive ball-park estimates for preliminary design or as a prior for a combined approach using Bayesian updating.

4.3.2.4 Combined approach

Bayesian updating is the recommended method for combining different sources of information for mechanical reliability prediction with independent statistical data. The specific case of Bayesian updating for reliability estimates derived with structural reliability methods, using failure data collected e.g. in part level testing, is discussed in Section 10.6.5. However, Bayesian statistics as discussed in Chapter 7 (Part II) are applicable also more generally, e.g. to update preliminary estimates based on Handbook data or expert elicitation.

## Failure mechanisms identification and focus

A full mechanical reliability prediction looking at all potential failure mechanisms and modes can require substantial efforts as well as inputs that may not be available especially in early phases of a space project. It is therefore reasonable to focus on what is dominating the probability of failure, and to exclude failure mechanism that are very unlikely to occur (compared to the dominating ones). The present section describes a step-wise procedure supporting the decision what should be modelled (i.e. which items/parts and failure mechanisms), before discussing how to model each failure mechanism in Section 10.5 and after.

To implement this procedure in practice, early interaction between design and reliability engineers is required. In particular, the following information exchange is necessary:

- Heritage and test data on identical or similar components: cumulated hours, observed failure modes, operational conditions.

- Identification of the most critical failure modes based on the available data, design considerations and physics of failure.

- Insight into the failure mechanics of the critical failure modes: conducted tests, available models, governing variables and their distributions.

- To perform reliability calculations for a specific item and failure mechanism, additional design data such as geometry or loads can be required.

### Failure dominance and focus of the prediction

The first step before starting a reliability assessment is a clear definition of its objective and scope, i.e. the question and the associated boundary conditions shall be clearly stated and understood, see Chapter 5 (Part I) for details. Knowing the objective and scope is important also because it implicitly defines which events and consequently failure mechanisms need to be considered.

Before going into the detailed reliability assessment of a mechanical item, it is helpful to identify its main working principle using the classifications presented in Table 4-3 below.

Table 4-3: Classification of mechanical items based on their working principle.

| Category |  | Description | Examples |
| --- | --- | --- | --- |
| I | Active mechanisms | Mechanisms employing electric motors and actuators | Deployment and pointing mechanisms, scanner, inertial measurement units |
| II | Passive and pyro actuated mechanisms | Mechanisms actuated by pre-stressed springs or pyro charges | Release and separation mechanisms, clamp bands, hinges |
| III | Non-fluidic structural components | Components realizing the structural architecture | Primary and secondary structure, housings, harness |
| IV | Fluidic structural components | Components containing fluids or gases | Pressurized vessels, heat pipes, loop heat pipes, tubing |

Note that some of these items may be covered by “standard” Miscellaneous models (Chapter 11). However, the modelling approaches for Mechanical items provided in the present chapter may be useful in the case the “standard” Miscellaneous model based on IOR is not applicable and a “non-standard” model has to be developed by the user.

The classification helps to preliminary identify the failure mechanisms that are most relevant at assembly level. For example, as soon as motorization is present, increase of friction due to lubrication wear or distortion play a dominant role for the overall item’s reliability. For passive and pyro mechanisms, fatigue and aging are expected to define the reliability.

However, the final selection of the dominant failure mechanisms shall be performed based on the concrete design solution. The identification of the dominant and secondary failure mechanisms is important as it helps focusing on the key reliability aspects whilst effectively using the schedule. To give an example, it may be sufficient to focus on the dominant ones when making a prediction at an early stage of the development.

Typically, components falling into category III “Non-fluidic structural components” can be considered as “intrinsically reliable” within the design lifetime when the following requirements are satisfied:

- Qualification and design factors ≥ 1.25

- Material and loading variability within usual ranges

- The design shows positive margins

- Qualification is completed

- Quality Assurance program is established

Under these conditions, it may be assumed that the reliability figure for the considered structure is equal to unity for practical purposes (in reality, it is just very close to unity). Note that the listed requirements are generally fulfilled for standard structures manufactured with traditional (metallic) materials and manufacturing processes, and using structural factors of safety as defined in ECSS-E-ST-32-10C [RD6] . Reliability predictions using structural reliability methods under these conditions yield very small probabilities of failure (see e.g. the case study in [RD18] , which is confirmed also by the good performance of structure in orbit. It should be noted that the probability of systematic failures due to gross design, manufacturing or operations errors may not be negligible irrespective of these conclusions, especially in the case of new designs.

For structures that do not fulfil the abovementioned conditions, reliability figures need to be calculated considering the relevant structural failure mechanisms. Studies aiming at the reusability of the flight hardware for multiple launches will need additional considerations.

### Failure mechanisms completion

The final step involves an assessment of relevant qualitative RAMS data (e.g. FMEA reports, non-compliances reports, design review data packages, and history data) to identify critical failure modes and related failure mechanisms. The outcome shall be used to cross-check the conclusions from the previous steps and to consider additional failure mechanisms as relevant.

Similar to the considerations made for the structural category in Section 10.4.1, it can be justified to exclude certain parts and failure mechanisms from the analysis, provided that all or most of the following conditions are fulfilled:

- High factors of safety and/or low variability

- Space qualified components

- Quality Assurance program is established

- Space heritage without observed failures

- Successful qualification and acceptance testing

Assuming a reliability of unity for these items (apart systematic failures) results in a reduced list of the failure mechanisms to be considered in the quantitative reliability calculations. However, the exclusion of these items assumed to be “intrinsically reliable” from the quantitative analysis should be justified and documented clearly.

The role of testing for flight item reliability

The approach followed for verification testing plays an important role for the flight item’s reliability, and thus for the decision which items can be considered as “intrinsically reliable” with reliability equal to unity. The following questions are relevant in this context:

- Is the test performed on the flight item or on another - possibly several item(s)?

- How many items are tested in case it is not the flight item that is tested?

- What is the test level (e.g. load factors) applied?

The first question is clearly the most relevant from a reliability perspective. Testing the flight item – in the mission representative environment with test levels equal to or larger than, but still in a similar order of magnitude as the stresses expected during the mission – is an excellent means to detect flaws due to both gross error and “normal” process variability that may otherwise lead to failures in orbit. It may be assumed that the good performance of structural items can be explained – partly or largely – to this effect. It is not possible to make a general statement on the choice of the exact test level in this context. On the one hand, the screening for existing flaws will be the more effective, the higher the stresses applied during the test. On the other hand, new flaws may be introduced by the test, and in the case of cumulated loading (e.g. fatigue loading), the test loads can be an important contribution to the overall life cycle loading of the flight item.

For some items and/or failure mechanisms, e.g. one-shot or limited lifetime items, it is not possible to perform the test on the flight item. In this case it is critical to know how many (other) items are tested. Testing only one duplicate of the flight item is still very important for the purpose of quality assurance, but nearly useless from the perspective of reliability prediction for the (untested) flight item. Only certain systematic flaws, e.g. due to gross design or manufacturing mistakes affecting all identical items in the same way, can be detected with such a test. Even though this information is important and often decisive, e.g. during the qualification campaign followed by acceptance testing, it does not infer “intrinsic” reliability. Margins in the failure mechanisms variables and low system level complexity can indicate potentially high reliability, but in general case failures due to process variability can only be avoided (or predicted) with a larger number of tests, e.g. batch acceptance testing performed on the same lot as the flight item.

Regarding the choice of the test level or duration, it is of course more interesting to test at higher levels (ideally up to failure), and there is no disbenefit from a reliability perspective as long as it is not the flight item that is tested. However, this requires good understanding of the relevant failure mechanisms. For example, to screen for premature failure (infant mortality) of parts prone to fatigue or wear, it can be necessary to reach a certain number of cycles with substantial loading while keeping cumulated damage as low as possible; aging effects might affect explosive devices which might be critical for long duration missions. Finally, it should be controlled that the test environment is representative.

To summarize, thorough testing of the flight item is a good argument to neglect a certain item or failure mechanism during the reliability prediction, especially when all or most of the other conditions listed above are true. If the probability of failure shall anyway be quantified, it is possible to consider the fact that the flight item has survived the verification test by adapting the strength distribution when using structural reliability methods [RD18] . A possible application of this approach may be to trade-off between reliability and mass for one-cycle-to-failure items such as tanks.

If it is not possible to test the flight item, assuming a reliability of one needs to be further justified, ideally with the aid of quantitative reliability prediction. Using the structural reliability methods discussed in Section 10.6, it is straightforward to account for the factors of safety applied during design, and space heritage without failures and/or extensive testing can easily be considered with statistical methods, or with the aid of Bayesian updating of preliminary reliability estimates (Section 10.6.5 and Chapter 7 Part II) if the sample size is not large enough.

## Method selection for different items and failure mechanisms

Once it is clear what needs to be considered in the reliability prediction (see Section 10.4), the next step is to decide how each part and failure mechanism should be modelled. The present section gives guidance on this method selection task.

### Categories and combinations of mechanical failure mechanisms

The method selection for mechanical parts reliability modelling should always take basis in a good understanding of the failure mechanisms that are of relevance for the considered item, especially the predominant ones. To simplify the presentation in the following sections, mechanical failure mechanisms are grouped into five distinct categories listed in Table 4-4.

Table 4-4: Categories of mechanical failure mechanisms.

| Failure mechanism category | Examples |
| --- | --- |
| Distortion | Creep, elastic deformation, plastic deformation, displacement |
| Fracture / Fatigue | Ductile/brittle fracture, high-cycle fatigue, low-cycle fatigue |
| Mechanical wear | Adhesive wear, abrasive wear, lubricated wear |
| Corrosion | Stress-corrosion cracking, galvanic corrosion |
| Material degradation | Thermal degradation, radiation damage |

The probability of failure can for some items be driven by several different failure mechanisms. When using PoF methods, the results for the individual mechanisms have to be combined to estimate the overall part reliability. The correct way of doing this depends on whether the different failure mechanisms can be considered as independent processes or events.

For independent failure mechanisms, it is sufficient to consider one process at once, and to combine the results using normal probability calculus.

However, independence between different failure mechanisms may not always be a valid assumption, e.g. in case of physical interactions between different failure mechanisms processes. Table 10-5 gives an overview for which combinations of failure mechanisms such effects may have to be considered. The table distinguishes interactions between different failure mechanisms affecting the same part (left hand side) and between different failure mechanisms affecting different parts (right hand side).

Table 4-5: Possible interactions between different mechanical failure mechanisms.

A simple way to account for such interactions is to select the dominating failure mechanism (e.g. the one with the highest probability of failure) and to consider the effect of other failure mechanism(s) on the dominating failure mechanism. When using structural reliability methods, this can be achieved by adapting the basic variable distributions to account for physical effects related to other failure mechanism processes. The assessment then requires the following steps:

- Selection of the dominating failure mechanism – e.g. wear of a lubricated bearing.

- Identification of other failure mechanisms that may interact with the dominating failure mechanism using Table 4-5 – e.g. distortion of the membrane compressing the bearing.

- Identification of physical variables used by the model for the dominating failure mechanism that may be affected by the interaction – e.g. increase of the contact pressure in the bearing.

- Adjustment of the identified basic variables distributional models to account for the effect of the interaction – e.g. by considering the increased contact pressure.

Finally, it should be noted that besides physical interactions between failure mechanism, there are other possible reason for statistical dependence, such as common cause effects due to random physical variables (e.g. temperature, loads), having an effect on different failure mechanisms and/or parts. An example for explicit common cause modelling in a redundant bolted interface with load redistribution can be found in [RD18] .

### Selection of methods for part level mechanical reliability prediction

The approach for method selection presented in the following is centred around the identified failure mechanisms, see Table 10-6 giving an overview of different mechanical part types with associated failure mechanisms and modelling recommendations.

The third column of Table 10-6 gives an indication about the relevance of each part type for different classes of mechanical systems introduced in Table 10-3. As has been discussed in Section 10.4, it may be sufficient to focus on those parts that dominate system level reliability. In the same way, the reliability of a specific part is often dominated by a single (or very few) failure mechanism(s), allowing to focus the analysis especially with methods that are based on the PoF.

Which failure mechanism(s) dominates reliability depends not only on the part type, but also on the application; e.g. a spring is likely to fail differently when used with static, cyclic or dynamic loading. The “predominant” failure mechanisms listed in Table 10-6 should therefore be considered only as a first guidance.

The remaining columns of Table 10-6 provide recommendations on the modelling of the selected parts and failure mechanisms. The focus is on the methods making reference to the Physics of Failure, in particular Structural reliability methods as outlined in Section 10.6 and 10.7..

The application of the Handbook approaches is restricted to selected parts, allowing the possible use of the NSWC Handbook [RD15] as a scaling method (which is based the Physics of Failure) to adapt statistical estimates based on data samples that are more relevant for space applications than the NSWC data, see Section 10.3.1.5 for discussion. The NPRD Handbook [RD14] is not considered any further.

In Table 10-6, statistical methods are mentioned only where a prediction based on a pure Physics of Failure approach is unlikely to be successful. However, as has been discussed already in Section 10.3.2, statistical methods are generally preferred over Physics of Failure approaches provided that sufficient and relevant data is available to perform the prediction. The failure mechanisms listed in Table 10-6 can provide guidance for judging the relevance of a specific data set, always keeping in mind the strong variability in mechanical parts reliability when seemingly identical parts are applied under different conditions.

Also a combined approach with Bayesian updating of a prior derived from the Physics of Failure, using relevant data even with limited sample size, is generally preferred over a pure Physics of Failure approach, see Sections 10.3.2.4 and 10.6.5 for guidance.

Table 4-6: Mechanical part types, associated failure mechanisms and modelling recommendations.

### Selection of methods for assembly level mechanical reliability prediction

Reliability prediction for assemblies of mechanical parts can either be made bottom-up, combining part level inputs with system level reliability methods, or top-down, using inputs directly collected at the level of the considered assembly. In general, similar considerations apply as for part level mechanical reliability prediction, leading to the following modelling recommendations:

- Statistical (or combined) methods are suitable provided that relevant failure data is available directly at the level of the considered assembly.

- Physics of Failure methods based on Structural reliability theory are suitable if the assembly level probability of failure is dominated by one or very few failure mechanisms (see also the discussion in Section 10.4).

- Bottom-up modelling starting from part level analysis is recommended in case none of the abovementioned conditions is fulfilled.

It should be noted that the NSWC Handbook [RD15] contains models also for some items that are better considered as assemblies rather than parts, including e.g. electric motors, solenoids and transducers. For reasons already discussed in Section 10.3.1.5, the direct use of these models is not recommended for space applications.

## Structural reliability methods and inputs

In the following, the use of different methods based on structural reliability theory are briefly discussed in the context of mechanical reliability prediction (Section 10.6.1 to 10.6.3):

- General structural reliability methods, as introduced in Chapter 6 (Part II)

- Simplified structural reliability methods, with analytic solutions for the probability of failure

- Stress-Strength methods with or without analytic solutions, as a special case of simplified structural reliability methods

Finally, Section 10.6.4 discusses the probabilistic modelling of random variables required as an input for structural reliability methods (with given failure mechanism model, see e.g. the models presented in Section 10.7). Bayesian updating of structural reliability estimates with new (failure) data is discussed in Section 10.6.5.

### Structural reliability methods for mechanical reliability prediction

The general principles of structural reliability prediction methods are discussed in Chapter 6 (Part II) of this handbook. Applying these methods to mechanical reliability prediction requires going through the following steps:

- Selection of the failure mechanism(s) to be modelled (cf. Section 10.4 and 10.5 above)

- Formulation of a limit state function gX based on a mathematical model for the considered failure mechanism, defining the failure domain as gX≤0

- Probabilistic modelling (distribution type and parameters) of the physical variables X entering the limit state function (“basic variables”)

- Derivation of the failure probability P gX≤0 using suitable methods.

It is relevant to note that the limit state function gX must always be related to a certain time interval (e.g. the duration of the mission) or event (e.g. launch). To derive a time-to-failure distribution, the limit state function and/or the basic variables in X must be explicitly modelled as a function of time, requiring repeated evaluations of the probability of failure.

Limit state functions for various well-known mechanical failure mechanisms are presented in Section 10.7. The list is not complete and dedicated models may be required to account for other failure mechanisms in specific applications. The definition of other limit state functions can take basis in models published in the literature (e.g. [RD16] , [BR65]), information from the design process or experimental evidence.

The probabilistic modelling of basic variables needs to account for the most relevant uncertainties associated with the failure mechanism modelling and with the physical variables entering the limit state function. Possible inputs for this step are discussed in Section 10.6.4.

Aiming at realistic modelling, it will not generally be possible to derive analytic solutions for the probability of failure with a given limit state function and probabilistic modelling. Simplified models with analytic interference calculations are discussed in Section 10.6.2. Also, the well-known stress-strength methods for structural items may in certain cases be solved analytically, see Section 10.6.3 for details. More general models may be developed using the numerical methods listed in Chapter 6 (Part II).

### Simplified structural reliability modelling with analytic solutions

To allow for a quick reliability assessment without detailed knowledge of the underlying probabilistic methods, simplified part level methods based on structural reliability theory have been developed for several failure mechanisms, see Section 10.7 for details. The simplifications concern the formulation of the limit state function, the distributional assumptions for basic variables entering the model, and the derivation of analytic formulas for interference calculations.

The most important aspects of simplified structural reliability modelling from a user’s perspective are summarized in Table 4-7 below.

Table 4-7: Difference between the use of “full” and simplified structural reliability methods

|  | Full method | Simplified methods |
| --- | --- | --- |
| Limit state function | Any functional form (explicit or implicit, e.g. numerical code) | Simplified, brought into the general format of Equation 10-2 |
| Basic variables | Probability distributions for all variables are chosen by the user | Only mean values and coefficients of variation are required as user input |
| Interference method | FORM/SORM or Monte Carlo | Analytic solutions are given |

The resulting models allow users to perform a reliability assessment based on structural reliability theory without detailed knowledge of the underlying probabilistic methods; only very limited user input is required. In the following, the general approach is briefly explained; see [RD18] for more detailed explanations, also on the derivations for each failure mechanism presented in Section 10.7.

      - 4.6.2.1 Generic approach for the formulation of the limit state function

The simplified approach is based on a generic limit state function with the following simple format:

| gX=X1-Θ∙X2 | Equation 10-2 |
| --- | --- |

Here, X1 may be interpreted as the “resistance” (allowable stress, or strength) of the considered part and X2  as the “load” (or stress) acting on it. To capture the uncertainties inherent in the applied models, a model uncertainty variable Θ is introduced. Failure occurs if the load is larger than the resistance, or ΘX2>X1. Thus, the probability of failure is derived from the overlap of the statistical distributions of X1 and ΘX2, see Figure 10-3 for illustration.

Time dependence is considered by modelling either X1 or X2 (or both) as a function of time.

Figure 4-3: Illustration of structural reliability methods, with probability of failure resulting from the overlap of the statistical distribution for the “load” and the “resistance” (statistical interference).

Despite its simplicity, the limit state function in Equation 10-2 is fairly generic, considering that both X1 and X2 can be a function of several random variables specific to the considered failure mechanism. In the simplified method, the two variables are defined such that the mean and coefficients of variation of X1 and X2 as well as the probability of failure Pf can be estimated analytically with pre-defined distributional models for the underlying basic variables. The results may easily be updated if new test or operation data becomes available during the course of a space project, see Section 10.6.5 for details.

The distribution of the model uncertainty Θ is fixed for the simplified models, assuming a coefficient of variation of vΘ=0.2  for most of the models considered in Section 10.7, which are generally empirical models. Setting the mean value to μΘ=1 (assuming that the model or, more precisely, the limit state function is unbiased) is appropriate when realistic predictions are required.

When needed, a certain explicit conservatism can be introduced in the simplified method by letting μΘ>1 , which is equivalent to assuming a higher loading or “stress” than estimated by the model (i.e. a biased load and/or resistance model). In accordance with ECSS-E-ST-32-10C [RD6] where a “model factor” is multiplied with the design loads to account for uncertainties in mathematical models, it is recommended to set μΘ=1.2 (or higher), referring to the model factor typically used for spacecrafts at the beginning of new developments. The effect of this assumption on the estimated failure probability depends on the distributional assumptions and on the design margin of safety. Figure 4-4 shows a comparison between some results for μΘ=1.0 (unbiased) and μΘ=1.2 (conservative bias), assuming Lognormal distributions for all random variables.

Figure 4-4: Effect of assuming a conservative bias in the model uncertainty variable (μΘ>1) on the estimated probability of failure.

      - 4.6.2.2 General process for the application of the simplified structural reliability method

The application of the simplified failure mechanism models always follows the same basic steps:

- Estimate the mean and coefficient of variation for all basic variables required by the model
The tables with variable definitions contain information about which variables should be considered as probabilistic, and in some cases a rough estimate for the appropriate CoV range. The mean (or deterministic) values have to be estimated for all variables. Guidance on the selection of distribution parameters based on the available information (e.g. conservative design estimates interpreted as fractile values) can be found in Chapter 6 (Part II). The distribution type of each variable has been fixed during the development of the simplified models and must not be selected by the user.

- Determine the mean and coefficient of variation for the “resistance” and the “load”
The development of the simplified models is based on the definition of a “resistance” (strength) and a “load” (stress) variable, denoted X1 and X2, which can be functions of one or several variables included in the basic variables list. In Section 10.7, specific formulas to estimate the mean value and coefficient of variation of X1 and X2 are provided for each failure mechanism with simplified model.

- Estimate the probability of failure using the provided interference function
The mean and coefficient of variation of the “resistance” and the “load” variable X1 and X2  are used as an input to estimate the probability of failure, which is performed using a predefined analytic function for statistical interference between the “resistance” and “load” (distributions for X1 and X2). The formulas provided in Section 10.7 for each failure mechanism with simplified model.

### Stress strength methods for structural items

Another well-known application of the generic limit state function in Equation 10-2 is the stress-strength method commonly applied for structural components, but also for other applications in mechanical reliability prediction (e.g. to assess motorization performance). Typically, the “strength” or resistance X1  and the “stress” or load  X2  are the only two variables considered, which is equivalent to dropping the model uncertainty variable θ in Equation 10-2. Note that this simplification furthermore implies that only a single stress and resistance is considered, unless e.g. several loads can be combined in a single metric, allowing to reduce a multivariate load distribution to a single random “stress” X2 (see also Section 10.6.2). Problems that cannot be reduced to a two-variable problem can be handled using the general structural reliability methods discussed in Section 10.6.1.

The modelling of the two basic variables generally has to account for time dependency. In the simplest case without strength degradation, the resistance X1 can be considered as time invariant. A time variant load  X2(t) may then be transformed into a time invariant random variable by taking the maximum load X2max=maxX2(t) during a specified time period, e.g. the highest load experienced during launch. The probabilistic modelling for the maximum load should then be based on extreme value theory (e.g. by fitting a random variable distribution to the maxima of a random process). More complex cases with time variant strength (e.g. degradation effects) and/or cumulative loading (e.g. fatigue loads) require dedicated considerations using time variant structural reliability methods. In the remainder of this section, these applications will not be discussed any further, the focus will be on time invariant reliability calculations.

Time invariant reliability problems - i.e. problems without the need to consider time dependency of  X1 and X2 during the reliability calculations - can generally be solved using one of the methods discussed in Chapter 6 Part II (e.g. FORM/SORM, Monte Carlo simulations). Analytic solutions are available for the following simple distributional assumptions:

- Both strength X1  and stress  X2 are approximated by a Normal distribution

- Both strength X1  and stress  X2 are approximated a Lognormal distribution

- The variability of one of the two variables (typically the Strength X1) is negligible compared to the variability of the other variable, allowing for an approximate analytic solution

In the following, closed-form solutions are given for these three cases. The choice of distributional assumptions should, however, not be driven by mathematical convenience, especially considering the limited effort required to perform an analysis for this simple limit state function, using numerical methods like FORM or Monte Carlo Analysis, which offer full flexibility in terms of distributional assumptions.

The probability of failure for the Normal distribution assumption is derived as follows:

| Pf=PX1-X2≤0=Φ-μX1-μX2σX12+σX22=Φ-p-1p2vX12+vX22 | Equation 10-3 |
| --- | --- |

Where μX, σX and vX represent the mean value, standard deviation and coefficient of variation of the random variables X1 and X2, and Φ denotes the cumulative distribution function of the standard Normal distribution. On the right-hand side of Equation 10-3, the probability of failure is derived as a function of the so-called central safety factor,  p=μX1μX2.

For the Lognormal distribution case we get:

| Pf=PX1-X2≤0=Φ-lnp+0.5lnvX12+1-lnvX22+1lnvX12+1+lnvX22+1 | Equation 10-4 |
| --- | --- |

Finally, in the third case, the probability of failure is simply derived from the distribution function of the remaining random variable:

| Pf≅FX1X2 or Pf≅1-FX2X1 | Equation 10-5 |
| --- | --- |

Where X1 or X2 represents a point estimate of the variable whose dispersion is neglected. It is relevant to note that, even though the references values for the usual dispersion of strength variables provided in Table 10-8 are generally smaller than the values provided in Table 10-9 for the dispersion of loads, the strength dispersion is not negligible, and the closed-form solution in Equation 10-5 cannot be used as an approximation for these cases.

The closed-form solutions in Equation 10-3 and Equation 10-4 show that for given distributional models, reliability is fully determined by the coefficients of variation vX1 and vX2 and the central safety factor p=μX1μX2 representing the ratio between the mean values of strength and stress. Note that this conclusion holds also for other distributional assumptions without analytic solution. However, the quantitative relationship between the central safety factor and the probability of failure is rather sensitive to the distributional assumptions, as interference takes place only in the tails of the stress and strength distributions.

The effect of distributional assumptions is illustrated in Figure 10-5, comparing Equation 10-3 for Normal distributed stress and strength with Equation 10-4 for Lognormal distributed basic variables. The difference between the two models grows with the coefficients of variation (in Figure 10-5, only vX2 is varied) and with the central safety factor, and thus with the reliability of the considered part.

It should be noted that the design factors used in practice, defined as the ratio between the design allowable load and the design limit load, are generally much smaller than the central safety factors depicted on the horizontal axis in Figure 10-5. The reason is that designers usually make use of conservative estimates for stress and strength, which first have to be transformed to mean values before making use of Equation 10-3 or Equation 10-4.  To give an example, ECSS-E-ST-32C [RD5] defines the limit load statistically as the load level not being exceeded with a probability of 99% during the service life of a structure. Assuming a Normal distributed load with a coefficient of variation of 0.2, this fractile value is a factor of 1.5 higher than the corresponding mean value of the same distribution. Additional conservatism is introduced in the strength value (e.g. using A- or B-values for material strength, defined as lower 1% or 10% fractiles of the strength distribution), and with the aid of explicit margins and/or factors of safety introduced in the design.

It is interesting to note that the impact of different distributional assumptions gets smaller when referring to design values for stress and strength, compared to the results shown in Figure 10-5, which are directly based on the central safety factor and thus referring to mean values for both variables. An important prerequisite for robust results is that the transformation between fractiles and mean values is performed based on the same distributional assumptions as the ones used for the reliability calculations.

Figure 10-5: Effect of distributional assumptions on the relationship between the probability of failure and the central safety factor, defined as the ration between the mean values of strength and stress.

### Basic variable modelling for mechanical reliability applications

The probabilistic modelling for the physical variables entering the limit state function, also called “basic variables”, requires going through the following three steps (see also Chapter 6 in Part II):

- Decide which uncertainties need to be modelled
The uncertainties associated with some variables (e.g. geometrical properties) may be negligible compared to others (e.g. stress loading). It is thus justified to model some variables as deterministic, and to focus on the main uncertainty drivers for probabilistic modelling.

- Select a suitable distribution function
The distribution type can be selected based on the following information:

  - Available data, e.g. from material testing

  - Published information, e.g. from launcher handbooks

  - Information about natural bounds (e.g. strength > 0)

  - Engineering knowledge on the underlying processes
(e.g. extreme value loads, weakest link theory, sums or products of variables…)

- Determine the distribution parameters
The distribution parameters can be determined from available data sources and/or making reference to the design values with some estimate on the scatter of the distribution.

- Bayesian updating with additional data (if available)
Basic variable distributions should be updated whenever new data becomes available, see Chapter 7 (Part II) for details. The information derived from the previous steps is used to formulate the prior distribution of the model parameters.

For the simplified method, the selection of uncertainty drivers and distributional assumptions are already predefined in Section 10.7. Thus, only the third step needs to be accomplished by the user when using these models. Some practical guidance for load (stress) and resistance (strength) variable modelling is given in dedicated subsection below.

For all basic variables, it should be noted that it may be necessary to adapt the uncertainty quantification in order to include process contributions in the analysis for a specific failure mechanism (see Section 10.2 for the discussion of root causes coverage):

- Process contributions may increase the variability of the loads or stresses

  - E.g. uncertainty in the environmental conditions considered during design

  - E.g. assembly uncertainties having an impact on stresses acting at part level

- Process contributions may increase the variability of the resistance or strength

  - E.g. model uncertainties in the designer’s strength calculations

  - E.g. minor manufacturing defects affecting the strength at part or assembly level

- Manufacturing variability may introduce additional uncertainties to be considered

  - E.g. uncertainties in the geometrical properties due to manufacturing tolerances

Mathematically, these effects can be accounted for by increasing the coefficients of variation of the basic physical variables used in the model and/or with the aid of a “model uncertainty” variable, possibly including a bias in the mean value (see e.g. Section 10.6.2). It depends on the information used for the basic variable modelling how much the variability needs to be increased to account for process contributions. To give an example, strength modelling based on material data (e.g. for metallic materials) does not account for any process contributions apart from those introduced during material production. The situation is, however, different when considering assembly level data, e.g. from structural inserts testing, which typically considers the whole insert as an assembly. In the latter case, possible process contributions must only be considered on the loading side.

      - 4.6.4.1 Modelling of strength variables

Obtaining statistical information for the strength (or resistance) of materials is generally straight-forward. Vendor’s datasheets and manufacturer specifications can be considered, and data for standard materials can be found in the literature. Well-known data sources for structural materials are the Metallic Materials Properties Development and Standardization (MMPDS) Handbook [BR66] for metallic material properties (superseding MIL-HDBK-5J [BR67]) and the FRAMES-2 database for mechanical test data of metallic and composite materials maintained by ESA [BR68]. New materials or conventional materials produced with a new production process are generally tested to determine the material characteristics.

The available data can be used also to define or check the assumed distribution function, e.g. using quantile plotting techniques. Engineering knowledge and data from similar materials can support this task if the data is not sufficient to justify distributional assumptions, for example:

- The Lognormal distribution may be assumed appropriate for most metallic materials, with the advantage that the distribution supports only values larger than zero.

- The Weibull distribution is appropriate for materials with “weakest links” characteristics, e.g. as ceramic materials whose strength is governed by randomly distributed material flaws.

- The Normal distribution is relevant in most cases when material characteristics are derived from statistical regression of experimental data, e.g. S/N curve parameters for fatigue strength.

With a given distributional assumption, the final step involves the estimation of the distribution parameters (or the mean and coefficient of variation) from the data. It is important to note that the distribution fit should be optimized especially in the lower tail of the data, which is dominating failure. The design value, typically defined statistically as a lower fractile value of the strength distribution, is thus a highly relevant piece of information, which can be combined with a coefficient of variation (or – better – with a second fractile value, e.g. combining A- and B-value) to derive the parameters of any two-parameter distribution model.

Reference values for usual dispersions of material strength are provided in Table 10-8, which is based on PSS-01-306 Draft 1 [BR69]. The original source of the data dates back to 1987 [BR70], considering test results and publications available at that time. It should be noted that the coefficients of variation have been derived, and traditionally been used, assuming normal distributed stress and strength variables.

Table 4-8: Reference values for usual dispersions (Coefficients of Variation) of strength variables [BR69].

| Material or item | Strength characteristics | CoV |
| --- | --- | --- |
| Metallic materials | Fracture strength (ultimate stress) Yield strength (ultimate stress / yield stress < 1.2) Yield strength (ultimate stress / yield stress > 1.2) | 8% 8% 15% |
| Metallic shells | Buckling strength (under combined loading) | 14% |
| Compound fibre carbon | Fracture strength (no yield limit) | 10% |
| Screw, rivet, weld | Fracture strength | 8% |
| Bonding | Shear strength | 12% |
| Honeycomb | Strength w.r.t. tension loading Strength w.r.t shear or compression loading Strength w.r.t. face wrinkling | 16% 10% 8% |
| Structural inserts | Strength w.r.t. axial loading Strength w.r.t. in plane loading | 12% Skin CoV* |
| Equipment inserts | Strength w.r.t. axial loading Strength w.r.t. in plane loading | 16% Skin CoV* |
| *Remark: the skin CoV corresponds to the Coefficient of Variation for the strength of the flange material. |  |  |

      - 4.6.4.2 Modelling of stress variables

Determining the statistical distribution of load variables is more difficult than the assessment of material characteristics. Four major difficulties may arise during (probabilistic) load analysis

- Data availability
Flight heritage data can be used for some environments to determine the statistical distribution of load variables and statistically defined limit loads, e.g. the launch loads provided in the Ariane 5 User Manual [BR71]. Other types of loadings, such as operating loads for space mechanisms, can only be derived conservatively from analysis or test under worst-case conditions. The exceedance probability of these limit loads is generally unknown.

- System level loading
Loads are often imposed at system level and must be transformed to lower levels, following the process of load analysis (ECSS-E-HB-32-26A [BR72]). The part level loads thus depend on the stiffness properties of the spacecraft structure as well, leading not only to a transformation of system level loads, but also to the introduction of additional model uncertainties, which are not easily quantified.

- Time dependency
Loads typically vary as a function of time. The dependency on time may be either known (i.e. deterministic) or random. For some failure mechanisms it may be sufficient to regard the maximum loads during a certain time interval, reducing the reliability assessment to a simple time-invariant problem. Extreme value analysis is not applicable for failure mechanisms resulting from accumulated damage such as e.g. fatigue. In this case, the full load spectrum including all (possibly random) load cycles has to be reduced to a series of cyclic loadings using techniques such as e.g. rainflow counting [BR73].

- Load combination
Practical reliability problems generally involve several load and resistance variables that must be described by their joint probability distribution function (possibly including dependencies) if several loads are interacting for the same failure mechanism.

A pragmatic approach to handle the challenges listed above is to take the limit loads used in deterministic design as an upper fractile value with corresponding exceedance probability (e.g. 1% at a 90% confidence level for limit loads defined statistically in consistency with ECSS-E-ST-32C  [RD5] ). Together with a rough estimate for the coefficient of variation of the stress distribution, this information can be used to derive the distribution parameters of any two-parameter model.

Distributional assumptions should nevertheless be backed by an engineering understanding of the load analysis process. To give an example, random vibration loads may be assumed to follow a Normal distribution when looking at the instantaneous (point-in-time) distribution of the random process. Assuming a linear response of the spacecraft structure, this assumption is still valid for the point-in-time distribution of the response spectrum. However, the peak response during a certain load event follows a Rayleigh distribution, or an extreme value distribution with parameters depending on the duration of the random vibration [BR74]. It should be noted that also the exceedance probability of a “three sigma” peak response, often used as limit load for design purposes, depends on the duration of the loading, and cannot be derived directly from the Normal distribution.

Reference values for usual dispersions of applied stresses are provided in Table 10-9, which is based on PSS-01-306 Draft 1 [BR69]. The original source of the data dates back to 1987 [BR70], considering the knowledge and technology available at that time. With the development of new launchers ([BR70] refers to data collected for Ariane I to III, STS and Delta), much of this information must be considered as outdated, though probably conservative. Some of the highest values in Table 10-9 are a direct result of the limited sample size of flight measurements in the same configuration that were available for the analysis. Moreover, it should be noted that the coefficients of variation have been derived, and traditionally been used, assuming normal distributed stress and strength. Also, for some loads it is not clear how the values provided are related to the underlying random process, e.g. whether the coefficient of variation for acoustic vibrations relates to the point-in-time response distribution, to the peak response, or to the resulting fatigue loading.

Table 4-9: Reference values for usual dispersions (Coefficients of Variation) of loads [BR69].

| Type of loading | CoV |
| --- | --- |
| Launch vehicle thrust | 5% |
| Launch vehicle other static loads | 30% |
| Transient loads | 50% |
| Thermo-elastic loads: correlated temperatures Thermo-elastic loads: uncorrelated temperatures | 7.5% 20% |
| Deployment shocks | 10% |
| Thruster loads | 2% |
| Acoustic vibration loads | 40% |
| Sine vibration loads | 20% |

### Bayesian updating of structural reliability estimates

Apart from the use of Bayesian updating for basic variable distributions (Section 10.6.4), the Bayes approach can also be applied to improve prior reliability estimates derived with structural reliability methods, making use of data samples with individual time-to-failure records of several items. Also small data samples can be used for this purpose.

An example for Bayesian updating of a prior structural reliability estimate is given in Section 10.8.2 (analytic approach) Section 10.8.3 (numerical approach using MCMC).

The general principles of Bayesian updating in a reliability context are discussed in Chapter 7 (Part II). For Bayesian updating in the present context, the prior has to be defined based on the considered limit state function and the corresponding basic variable distributions. The limit state function can usually explicitly or implicitly be defined as a function of time t (or number of cycles / revolutions). Thus, the cumulative distribution function of the random time to failure can be derived by estimating the probability of failure as a function of t:

| FTt=PT≤t=Pft=PgXt≤0 | Equation 10-6 |
| --- | --- |

The resulting function may be interpreted as the prior predictive distribution for the sample variable, as all uncertainties (including model uncertainties and uncertainties associated with the basic variables) are already integrated in the assessment.

A straightforward approach for updating this prior is to treat the model uncertainty Θ, which is one of the basic variables in X, as the uncertain model parameter of the prior model. The prior distribution for Θ is then the distribution assumed during the structural reliability assessment and the sampling distribution (likelihood) can be derived by conditioning on Θ=θ:

| FTtθ=PT≤tθ=Pftθ=PgXt≤0 Θ=θ | Equation 10-7 |
| --- | --- |

The conditional distribution function FTtθ defined by Equation 10-7 can now be used as the sampling distribution for the formulation of the Likelihood, which is combined with a prior for the distribution of the random model uncertainty, f'Θθ, to apply Bayes rule for updating. In the general case, this will require numerical methods to perform the updating. However, an analytic solution can be derived for the special case of Bayesian updating for a prior derived from the simplified structural reliability method introduced in Section 10.6.2, as will be discussed in the following.

Using the generic simplified limit state function defined in Equation 10-2, Equation 10-7 can be reformulated as follows:

| FTtθ=PX1t-θ⋅X2t≤0 =Ppt⋅X'1-θ⋅X'2≤0 | Equation 10-8 |
| --- | --- |

Here, X1t denotes the random “resistance” (or strength) of the considered part against the random “load” (or stress) X2t. Both random variables are normalized (variables with prime) such that EX'1=EX'2=1. The mean value information is then summarized in the parameter p=EX1EX2. In Equation 10-8 it is assumed that only the expected values of X1 and X2 are a function of time, while the coefficients of variation vX1 and vX2 remain constant.

The analytic method for Bayesian updating is based on the assumption that all random variables in Equation 10-8 (i.e. X1 and X2 or X'1 and X'2) are Lognormal distributed. With this assumption, the sampling distribution FTtθ is derived as follows:

| FTtθ=Φ-lnpt+lnθ+0.5⋅lnvX12+1-lnvX22+1lnvX12+1+lnvX22+1 | Equation 10-9 |
| --- | --- |

To proceed, a suitable function pt must be defined. For the limit state functions provided with simplified model in Section 10.7, it may be assumed that the mean value parameter p is proportional to 1t, although for some failure mechanisms this may be an approximation only. An alternative approach would be to derive the sampling distribution for p instead of t. The data used for updating (observations for t) must then be transformed to “observed” mean value parameters pt.

The following derivation is based on the proportionality assumption mentioned above. In this case, pt may be replaced by p=k⋅1t, where k is a constant specific to the considered item and failure mechanism. This leads to the following formulation:

| FTtθ=Φlnt-lnk+lnθ+0.5⋅lnvX12+1-lnvX22+1lnvX12+1+lnvX22+1 | Equation 10-10 |
| --- | --- |

By comparing this result with the cumulative distribution function of the Lognormal distribution, it may be concluded that the random time to failure T follows a Lognormal distribution with the following distribution parameters:

| μT=lnk-lnθ+0.5⋅-lnvX12+1+lnvX22+1 | Equation 10-11 |
| --- | --- |
| σT=lnvX12+1+lnvX22+1 | Equation 10-12 |

For the Bayesian updating, the „deterministic“ model uncertainty θ in Equation 10-11 is now replaced by a random variable Θ. The random model uncertainty is assumed to follow a Lognormal distribution with mean value EΘ =1 (when assuming an unbiased failure mechanism model, see Section 10.6.2.1 for discussion) and coefficient of variation vΘ (with specific values proposed for each simplified model presented in Section 10.7). With this, the parameter μT is Normal distributed with the following prior (hyper-)parameters:

| μ'=lnk-lnEΘ+0.5⋅lnvΘ2+1-lnvX12+1+lnvX22+1 | Equation 10-13 |
| --- | --- |
| σ'=lnvΘ2+1 | Equation 10-14 |

The Normal distribution for the location parameter μT is a conjugate prior for the Lognormal sampling distribution. With a random sample of time-to-failure observations t=t1,…,tn , the posterior hyperparameters for the Lognormal sampling distribution can thus be derived analytically:

| μ''=σ''2⋅μ'σ'2+i=1nlntiσT2 | Equation 10-15 |
| --- | --- |
| σ''=1σ'2+nσT2-12 | Equation 10-16 |

To summarize, a simple Bayesian updating scheme for reliability estimates derived from the simplified method introduced in Section 10.6.2 may be defined as follows:

- First the prior reliability estimate is derived, including the estimation of vX1, vX2 and pt=EX1tEX2t. The simplified model equations provided in Section 10.7 can be used for this task, using suitable input for the basic variable modelling.

- The constant k for the function pt=k⋅1t is derived from the simplified model equations provided in Section 10.7 (time t may have to be replaced by e.g. number of revolutions rev for some failure mechanisms); the proportionality assumption for the function pt should be checked in this step.

- The standard deviation parameter σT of the sampling distribution is calculated from vX1 and vX2, using Equation 10-12. Note that this parameter is assumed to be fixed in the analytic updating approach and will not be affected by the updating.

- The prior (hyper-)parameters μ' and σ' of the conjugate Normal distribution for the location parameter μT are calculated from the input for the simplified method, using Equation 10-13 and Equation 10-14 above.

- With this, the prior is fully defined and the Bayesian updating can be performed using the formulas for the posterior hyperparameters μ'' and σ'' for the Lognormal sampling distribution.

The assumptions underlying this approach are summarized as follows:

- The limit state function for the prior reliability estimate can be brought into the generic format used in Equation 10-8.

- The variables X1, X2 and Θ follow a Lognormal distribution.

- Only the mean values of X1 and X2 are a function of time t.

- The mean value parameter pt=EX1tEX2t can be modelled as pt=k⋅1t.

- The data sample for updating is uncensored, i.e. all items have been observed until failure

As has been discussed below Equation 10-9, the approach may be adapted by considering p instead of t as the “observed” variable, allowing the use of more general functions for pt; An analytic solution is still possible in this case. More general distribution types for X1, X2 and/or Θ may possibly be approximated by Lognormal distributions. For more general applications, it is very unlikely that an analytic solution based on conjugate priors can be derived. Numerical methods are then required to solve the problem of updating a structural reliability estimates with observations for the random time to failure, see Chapter 7 (Part II) for details. The same is true if the data set is strongly censored, i.e. the items have not been observed until failure. It should be noted that Equation 10-6 and Equation 10-7 can still be used as the starting point for the formulation of the prior probabilistic model, even if the analytic solution derived above is not applicable.

## Structural reliability models and equations

The present section provides limit state functions and guidance for the modelling of several failure mechanisms that are of relevance for mechanical reliability prediction. The content of this Section is largely based on [RD18] , where some aspects are discussed in more detail.

An overview on the failure mechanisms covered by the different subsections is found in Table 10-10. The limit state functions provided in each dedicated subsection are generally applicable with the “full” structural reliability method described in Section 10.6.1 and in Chapter 6 (Part II). Simplified models in the sense of Section 10.6.2, with analytic solutions for the probability of failure calculations, are provided for some failure mechanisms. Failure mechanisms that can be modelled with the aid of simple stress strength methods (as discussed in Section 10.6.3) are also mentioned in Table 10-10, but will not discussed any further in the present section.

Table 4-10:  Overview on Failure mechanisms covered in Section 10.7.

| Failure mechanism category (cf. Table 10-4) | Limit state functions provided in Section 10.6.5 | Simplified model? | Section |
| --- | --- | --- | --- |
| Distortion | The modelling of distortion is very case specific, but can often be handled with generic stress strength methods |  | (10.6.1, 10.6.3) |
| Wear | General (adhesive) wear | Yes | 10.7.1.1 |
|  | Solid lubricant wear | Yes | 10.7.1.2 |
|  | Fluid lubricant wear | Yes | 10.7.1.3 |
| Fracture / Fatigue | High cycle fatigue | Yes | 10.7.2.1 |
|  | Fatigue crack growth | No | 10.7.2.2 |
|  | Fracture and low cycle fatigue: stress strength approach |  | (10.6.3) |
| Corrosion | Stress corrosion cracking | No | (10.7.3) |
| Material degradation | Radiation damage | Yes | 10.7.4.1 |

Failure mechanisms that are not listed in Table 10-10 need to be modelled by the handbook user, possibly making use of published information, e.g. in RiAC’s WARP [RD16] or Kowal [BR65]. The “full” structural reliability method may be applied with any limit state function or failure mechanism model, see Section 10.6.1 for details.

The following subsections dedicated to each failure mechanism model always follow the same logic: First the failure mechanism is introduced and a generic limit state function is derived that can be used with general structural reliability methods, possibly requiring numerical methods to derive the probability of failure. Closed form solutions (if available) are then discussed separately under the heading “Simplified failure mechanism model”.

The derivation of these closed-form solutions is based on several assumptions related to the limit state function and the basic variable modelling, see Section 10.6.3 for a general introduction. The detailed assumptions are specific to the considered failure mechanisms; a short summary is provided together with each simplified model. Note that it is not possible to make a general statement how “exact” the analytic solutions are, e.g. when compared to a “full” structural reliability assessment. However, the assumptions listed can and should be checked to decide whether the simplified model can be used for a given application or whether the “full” structural reliability method is more appropriate.

### Modelling of failures due to mechanical wear

Wear is a complex phenomenon related to the erosion or displacement of material during the contact of solid surfaces. Several types of wear can be distinguished:

- Adhesive wear – displacement and attachment of wear debris from one surface to another.

- Abrasive wear – cutting and deformation of material from a soft surface when a harder surface slides across it (2-body) or due to hard particles between two surfaces (3-body).

- Fretting wear – wear between two solid surfaces experiencing oscillatory relative motion of low amplitude, e.g. induced by vibration.

- Lubricated wear – wear in lubricated contacts.

- Fatigue wear – weakening of a surface by cyclic loading as wear particles are detached by cyclic crack growth of micro-cracks on the surface.

Adhesive, abrasive and fretting wear show common behaviour, and it is commonly agreed that for these types the adhesive wear mechanism plays a dominant role. A general wear model with a focus on adhesive wear is presented in Section 10.7.1.1.

Lubricated wear is related to the degradation of the lubricant or the lubricant reservoir in lubricated contacts. This regime is typical for bearings, with analytical models provided in Section 10.7.1.2 for solid lubricated bearings (particularly bearing cages), and in Section 10.7.1.3 for fluid lubricated bearings.

Finally, fatigue wear is caused by cyclic loading and corresponding crack formation. It is the dominant bearing failure mechanism in on-ground applications, where bearing life is traditionally modelled based on the Lundberg-Palmgren theory [BR75] with empirical formulas for the “basic life” of a bearing (corresponding to the number of revolutions at which 10% of the bearings have failed). However, in space applications, bearing lifetime is almost never limited by material fatigue, but instead by the degradation of the lubricant [BR76]. For this reason, fatigue wear is not discussed any further in the following.

      - 4.7.1.1 Adhesive wear

Adhesive wear is considered as governing for many wear failures, and is thus typically used for the analytical prediction of wear. The key parameter for the wear estimation is the volume worn away per unit of sliding distance, which can be estimated based on the classical wear model by Archard [BR77], here used in the form of Lancaster [BR78]. Failure due to adhesive wear is then modelled with the following limit state function:

| gXt=X1-Θ∙X2t=Vlimit-Θ∙0TKHt∙St∙vtdt | Equation 10-17 |
| --- | --- |

Vlimit is the limiting value for the volume worn away, Θ the model uncertainty for the Archard wear model, KH the specific wear rate, S the normal load, v the sliding velocity and T the considered time interval. As an alternative to the limiting value for the volume worn away, the model can also be reformulated in terms of the wear depth d:

| gXt=X1-Θ∙X2t=dlimit-Θ∙0TKHt∙StAapp∙vtdt | Equation 10-18 |
| --- | --- |

Where dlimit is the limiting value for the wear depth d and Aapp is the area of apparent contact. Both limit state functions are basically identical, and either one of them can be used for the wear modelling depending on the problem definition.

By adapting the wear rate, the approach can also be used for other wear mechanisms, such as fretting or abrasive wear. The nature of the wear coefficient is generally very complex and its analytical modelling is difficult. Hence, in practice the Archard model is combined with experimental data in order to perform lifetime assessments.

Simplified adhesive wear model

Assuming that the specific wear rate KH is constant over the whole mission (in practice it is typically derived from tests under “worst case” conditions) and that the remaining time dependent variables are constant during certain deterministic time intervals (e.g. different mission phases), the limit state function given in Equation 10-17  can be simplified as follows:

| gXt=X1-Θ∙X2t=Vlimit-Θ∙KH∙i=1nPPi∙ti | Equation 10-19 |
| --- | --- |

Where Pi=Si∙vi corresponds to the sliding work per time unit (the sliding “power”). Thus, X2 is defined as the volume worn away and X1 as the corresponding limiting value. Detailed variable definitions are given in Table 10-11.

Table 10-11: Assumed probabilistic models for the simplified adhesive wear model.

The expected values and coefficients of variation of X1 and X2 are determined as follows:

| μX1=E[Vlimit] | Equation 10-20 |
| --- | --- |
| μX2=EKH∙i=1nPEPi∙ti | Equation 10-21 |
| vX1=vVlimit=Var[Vlimit]E[Vlimit] | Equation 10-22 |
| vX2=vKH2+1+vKH2∙Vari=1nPPi∙tii=1nPEPi∙ti2 | Equation 10-23 |
| Vari=1nPPi∙ti=i=1nPVarPi∙ti2+2i=1nPj=i+1nPCovPi,Pj∙ti∙tj=i=1nPVarPi∙ti2+2i=1nPj=i+1nPρPiPj∙VarPi∙VarPj∙ti∙tj | Equation 10-24 |

Where ρPiPj denotes the correlation between the sliding work per time unit in different mission phases. The correlation structure may be defined in a simplified way, e.g. assuming ρPiPj=1 if the sources of uncertainty in P are the same in mission phase i and j and ρPiPj=0 if there are different uncertainty sources. Assuming full correlation increases the coefficient of variation of X2, which is conservative for small probabilities of failure.

The approach can be further simplified when considering only one mission phase (i.e. nP=1), leading to: Vari=1nPPi∙ti=VarP∙t2. In practice, this may also be used as a conservative simplification for nP>1, taking P from the most critical phase and t as the total length of the mission. Another strong simplification is possible if the uncertainty in the sliding work per time unit is negligibly small. In this case, the uncertainty in the sum may be neglected and the variance term in Equation 10-24 becomes zero.

With the distributional assumptions in Table 10-11, an analytic solution for the probability of failure calculations is derived based on a Lognormal approximation for X2:

| Pft=PX1-X2t∙Θ≤0 =ΦlnEΘ-lnμX1μX2+0.5lnvX12+1-lnvX22+1-lnvΘ2+1lnvX12+1+lnvX22+1+lnvΘ2+1 | Equation 10-25 |
| --- | --- |

The main assumptions underlying the simplified adhesive wear model are summarized as follows:

- The mission - or the considered time interval t - is divided into nP intervals with known (deterministic) duration ti.

- The wear rate KH is constant over the whole mission (all time intervals i).

- The normal load Si and sliding velocity vi (or the sliding work per time unit Pi=Si∙vi) are constant during each time interval ti.

- The distributions of all basic variables can be represented by the models listed in Table 10-11. Other models for KH and Pi are possible as long as the distribution of X2 may still be approximated by a Lognormal distribution.

- The effect of the Lognormal assumption for X1=Vlimit has been investigated in [RD18] . It may lead to an underestimation of the probability of failure, see Figure 10-6 below.

Due to the effect of the last assumption, the analytic method should not be used without justification if the coefficient of variation of X1=Vlimit is larger than 0.3.

Figure 10-6: Comparison of the analytic solution in Equation 10-25 (adhesive wear model with Lognormal distributed Vlimit) with Monte Carlo results for Normal distributed Vlimit.

      - 4.7.1.2 Solid lubricant wear

Due to the fact that solid lubricants do not evaporate, they are suitable for extreme temperatures (both cold and hot) and for applications where contamination could be an issue, e.g. in optical systems. In order to enable on-ground testing and to support the lubrication regime during operations, the lubricant (e.g. sputtered MoS2) is stored in special reservoirs such as cages in case of bearings, which are at the same time used to avoid ball collisions in ball bearings. When the lubricant is degraded, new lubricant is provided via the wear of the reservoir.

In the following, solid lubricant wear modelling is described taking example in a ball bearing. The modelling is applicable to other cases of solid lubricant wear; however, the number of revolutions has to be substituted with another suitable measure of sliding distance.

The modelling easily can be adapted from the adhesive wear model given in Section 10.7.1. Wear is realized during cage/ball contact; hence the relation 0tS(t)∙vtdt in Equation 10-17 or Equation 10-18 represents the work of the ball/cage interaction forces. For the bearing example, time is replaced by the number of revolutions rev. The equality 0tSt∙vtdt=0revαrevdrev can be used for experimental evaluation of the average ball/cage interaction forces, defining a new interaction parameter α.

Simplified solid lubricant wear model

Using the simplified form of the adhesive wear model in Equation 10-19 (assuming constant conditions during each mission phase), a limit state function for solid lubricant wear can be derived as follows:

| gXt=X1-Θ∙X2t=Vlimit-Θ∙i=1nPKH,i∙αi∙revi | Equation 10-26 |
| --- | --- |

X2 is defined as the volume worn away and X1 as the corresponding limiting value. Detailed variable definitions are given in Table 10-12. It should be noted that the parameter αi , defined as the average work of ball/cage interaction forces per bearing revolution, will typically be estimated from tests and is thus dependent on the wear rate KH,i . This is the reason why the wear rate cannot easily be taken out of the sum, as in Equation 10-19 for the general adhesive wear model.

Table 4-12: Assumed probabilistic models for the simplified solid lubricant wear model.

The expected values and coefficients of variation of X1 and X2 are determined as follows:

| μX1=E[Vlimit] | Equation 4-27 |
| --- | --- |
| μX2=i=1nPEKH,i∙αi∙revi=i=1nPEKH,i∙Eαi+CovKH,i,αi∙revi | Equation 4-28 |
| vX1=vVlimit=Var[Vlimit]E[Vlimit] | Equation 4-29 |
| vX2=1μX2i=1nPVarKH,iαi∙revi2+2i=1nPj=i+1nPCovKH,iαi,KH,jαj∙revi∙revj | Equation 4-30 |

Assuming full correlation to derive the covariance between KH,i and αi (for Equation 10-28) and for the product of the two variables between different time intervals (for Equation 10-30) leads to conservative results. As in the general adhesive wear model, the problem can be simplified by considering only one mission phase (nP=1), requiring assumptions only on the covariance in Equation 10-28; the covariance term in Equation 10-30 can be dropped if nP=1.

With the distributional assumptions in Table 10-12, an analytic solution for the probability of failure is derived based on a Lognormal approximation for X2:

| Pft=PX1-X2t∙Θ≤0 =ΦlnEΘ-lnμX1μX2+0.5lnvX12+1-lnvX22+1-lnvΘ2+1lnvX12+1+lnvX22+1+lnvΘ2+1 | Equation 10-31 |
| --- | --- |

The main assumptions underlying the simplified solid lubricant wear model are the following:

- The mission is divided into nP intervals with known number of revolutions revi.

- The wear rate KH,i and ball-cage interaction work αi are constant during each interval.

- The distributions of all basic variables can be represented by the models listed in Table 10-12. Other models for KH,i and αi are possible as long as the distribution of X2 may still be approximated by a Lognormal distribution.

- The effect of the Lognormal assumption for X1=Vlimit may lead to an underestimation of the probability of failure, see [RD18] and Figure 10-6.

Due to the effect of the last assumption, the analytic method should not be used without justification if the coefficient of variation of X1=Vlimit  is larger than 0.3.

      - 4.7.1.3 Fluid lubricant wear

In this section a fluid lubricant wear modelling is described taking example in a ball bearing again. The modelling is applicable to other cases of fluid lubricant wear, but the number of revolutions has to be substituted with another measure of sliding distance in this case.

The starting point for the modelling of fluid lubricant wear is the following limit state function, see [BR79] for details:

| gXt=X1-Θ∙X2t=M0-Minactive-Θ∙Nb2∙1-bdpd∙cosϑ2∙K∙exp-3.35pm∙revt | Equation 10-32 |
| --- | --- |

Here, M0 denotes the initial mass of the lubricant, Minactive the mass of lubricant not participating in the contact (e.g. due to evaporation), and Θ the model uncertainty for the associated term, which represents the model to estimate lubricant life described in [BR79]. In this term, Nb denotes the number of balls in the bearing, pd the pitch diameter, bd the ball diameter, ϑ the contact angle, K is an empirical constant, pm the contact pressure and rev the number of revoluations during the considered time interval t. The variables related to bearing geometry are illustrated in the figure given below.

Figure 10-7: Bearing geometry.

Simplified fluid lubricant wear model

For the simplified modelling, it is considered sufficient to model only Θ and K as random variables; all other variables in Equation 10-32 are assumed to be deterministic. Since the relation for the bearing life from [BR79],  K∙exp-3.35∙pm, is purely empirical, curve fitting residuals or the distribution of the coefficient K may be used for the probabilistic assessments.

Note that in principle, Minactive may also be treated as stochastic, but its value will generally be only a small fraction of M0, typically not exceeding 1-2% when considering only evaporation losses for a bearing with a labyrinth seal under optimal temperature conditions [BR79].

It is furthermore assumed that all variables (except for the number of revolutions rev) are constant over the whole length of the mission. Minactive should be estimated considering the amount of lubricant that has evaporated (or is not participating in the contact due to other reasons) at the end of the mission. In reality the amount of evaporated lubricant depends on the number of revolutions, thus the proposed approach leads to a slightly conservative estimation.

These assumptions allow the application of Equation 10-32 without any further simplifications in the limit state function. Thus, X1=M0-Minactive is defined as the amout of lubricant available and X2 as the amount of lubricant required for the mission, estimated based on the lubricant life model presented in [BR79]. Detailed variable definitions are reminded in Table 10-13 below.

Table 10-13: Assumed probabilistic models for the simplified fluid lubricant wear model.

The mean value of X1 is determined as μX1=M0-Minactive with coefficient of variation vX1=0. For X2, the following equations can be used:

| μX2=vK2+1EK∙Nb2∙1-bdpd∙cosϑ2∙exp-3.35pm∙rev | Equation 10-33 |
| --- | --- |
| vX2=vK=VarKEK | Equation 10-34 |

The probability of failure is estimated as follows:

| Pft=PX1-X2t∙Θ≤0 =ΦlnEΘ-lnμX1μX2-0.5lnvX22+1+lnvΘ2+1lnvX22+1+lnvΘ2+1 | Equation 10-35 |
| --- | --- |

The main assumptions underlying the simplified fluid lubricant wear model are the following:

- Failures in fluid lubricant wear are dominated by the life of the lubricant.

- The amount of lubricant not participating in the contact (e.g. due to evaporation), Minactive, is a small fraction of the initial amount of lubricant, M0 (1-2% for bearings with a labyrinth seal under optimal temperature condition).

- The estimate for the (deterministic) variable Minactive is chosen conservatively, e.g. by determining the amount of lubricant evaporated at the end of the mission based on the model provided in [BR79].

- The uncertainty in most of the variables listed in Table 10-13 (except for the empirical constant of the lubricant life model K and the model uncertainty Θ) can be neglected.

- The distributions of K and Θ can be represented by the models listed in Table 10-13.

### Modelling of failures due to fatigue

Fatigue is a failure mechanism incurred by cyclic loading, leading to the initiation and extension of cracks, which degrade the strength of materials and structures. Failures due to fatigue can be considered as the consequence of several steps:

- Formation of the crack (crack initiation)

- Small-crack growth

- Large-crack growth

- Failure by fracture

Two loading conditions are distinguished:

- High-Cycle Fatigue: A high-frequency, low-amplitude loading condition created by structural, acoustic, or aerodynamic vibrations that can propagate flaws to failure.

- Low-Cycle Fatigue: A low-frequency, high-amplitude loading condition created by thermal, pressure, or structural loads that can propagate flaws to failure.

Failures due to low-cycle fatigue are driven by accumulated plastic deformations, which are typically not allowed (i.e. the design is restricted to elastic regime). If necessary, low-cycle fatigue can be assessed by the ultimate failure criterion, e.g. by means of non-linear finite element analysis with dedicated failure models or dedicated criteria like Manson-Coffin failure criterion. The probability of failure can be assessed using general structural reliability or stress strength methods.

In the remainder of this section, only high-cycle fatigue is considered.

The standard approach to the fracture control for space projects is presented in ECSS-E-ST-32-01C [BR80]. Depending on the criticality, different approaches can be used:

- Fail-safe – damage-tolerance design principle, where a structure has redundancy to ensure that failure of one structural element does not cause general failure of the entire structure during the remaining lifetime.

- Safe life – fracture-control design principle, for which the largest undetected defect that can exist in the part does not grow to failure when subjected to the cyclic and sustained loads and environments encountered during the lifetime.

- Low risk of fracture – e.g. when the stress level is below the endurance limit C, then assessment can be performed approximately based on stress and endurance limit only.

Crack formation analysis is typically performed for the fail-safe approach in order to ensure that failure of one redundant structural element does not cause general failure of the entire structure during the remaining lifetime. This analysis is performed based on the S/N curve approach, whose application for reliability modelling is outlined in Section 10.7.2.1.

The safe life principle implies an assessment of the crack growth using fatigue crack growth models, as discussed in Section 10.7.2.2. Environmental properties can also be considered in these models, allowing the consideration of failure mechanisms due to stress corrosion cracking (SCC).

      - High cycle fatigue modelling with the S/N curve approach

Crack generation due to high cycle fatigue is most commonly modelled using so-called S/N curves relating a stress level or equivalent stress Seq to the number of cycles to failure, Nf:

| log10Nf=A-B∙log10Seq-C | Equation 10-36 |
| --- | --- |

Where the equivalent stress is determined as follows:

| Seq=Smax1-RP=Smax1-SminSmaxP | Equation 10-37 |
| --- | --- |

Here, Smin and Smax denoting the minimum and maximum stress during one load cycle and A, B, C and P are model parameters of the S/N curve model that can be derived from test data, e.g. those presented in MMPDS-12 [BR66] or its predecessor, MIL-HDBK-5 [BR67]. Of special interest is the parameter C, corresponding to the fatigue limit stress, or endurance limit, under which it is assumed that no fatigue failure occurs, thus NfS≤C→∞.

The S/N curve is valid for cyclic loading with constant amplitude or equivalent stress. For cyclic loading with varying stress levels, the S/N curve is combined with the Palmgren-Miner accumulation law to estimate the accumulated damage Dtot after N stress cycles:

| Dtot=i=1N1Nf,i=i=1N10-A∙maxSeq,i-C;0B = j=1kNj∙10-A∙maxSeq,j-C;0B | Equation 10-38 |
| --- | --- |

Where each Seq,i is drawn from the distribution of equivalent stresses derived from the loading process. The second line in Equation 10-38 is derived by dividing the continuous distribution of Seq,i into a discrete number of bins. In this final expression, k denotes the number of bins, Nj the number of cycles in each bin and Seq,j the corresponding equivalent stress representative for each bin.

As a failure criterion, the accumulated damage Dtot is compared with a fixed threshold Dcr that is typically assumed to be equal to one. However, due to the empirical nature of Miner’s law, failures may also occur at values for Dtot that are below or above unity and  Dcr must be considered as a random variable.

A limit state function for high-cycle fatigue is now derived as follows:

| gXt=X1-Θ∙X2=Dcr-Θ∙Dtot =Dcr-Θ∙i=1Nt10-A∙maxSeq,i-C;0B =Dcr-Θ∙j=1kNjt∙10-A∙maxSeq,j-C;0B | Equation 10-39 |
| --- | --- |

Note that the time dependent formulation in Equation 10-39 neglects load history effects, which is consistent with the accumulative nature of the Palmgren-Miner model. However, when the goal is to evaluate the probability of failure in different mission phases, the accumulated damage contribution of each load event (e.g. testing, launch, in-orbit) should be evaluated separately.

Simplified S/N curve modelling

The starting point for the simplified model is the limit state function in Equation 10-39, with load cycles summarized in bins (last row). For the simplified approach, the variables Nj, B and C are assumed to be deterministic. The uncertainty associated with the loading Seq,j is then the only uncertainty that cannot be taken out of the sum. A simple approach to model this uncertainty is to multiply deterministic values for  Seq,j (e.g. taken from design calculations) with a random stress scaling factor SSF that is applied globally, i.e. taking on the same value for all bins [BR81]. If it is furthermore assumed that the endurance limit C equals zero (which is conservative for all applications), the limit state function defined in Equation 10-39 can be brought into the following simple expression:

| gXt=X1-Θ∙X2=Dcr-Θ∙Dtot=Dcr-Θ∙10-A∙SSFB∙j=1kNj∙Seq,jB | Equation 10-40 |
| --- | --- |

Thus, X2 is defined as accumulated damage, which is calculated based on the S/N curve approach with the Palmgren-Miner accumulation law, and X1 as the corresponding limiting value. Detailed variable definitions are reminded in Table 10-14.

Table 10-14: Assumed probabilistic models for the simplified high-cycle fatigue model.

Without additional information, the expected value of Dcr may be assumed to be equal to unity and its coefficient of variation equal to 0.3 (based on JCSS [BR82] and Wirsching [BR83]). The mean value and coefficient of variation of the stress scaling factor SSF can be determined as follows: First an appropriate coefficient of variation vSSF reflecting all uncertainties associated with the load analysis is chosen, considering the most uncertain load event to be conservative. The mean value of the stress scaling factor then depends on the definition of the Seq,j values. Design values for fatigue stresses are typically defined as upper fractile values, leading to the following relationship:

| ESSF=vSSF2+1expΦ-1α∙lnvSSF2+1 | Equation 10-41 |
| --- | --- |

Where Φ-1∙ denotes the inverse of the Standard Normal distribution function and α is the probability that the stresses will be smaller than Seq,j.

Finally, the expected values and coefficients of variation of X1 and X2 are determined as follows:

| μX1=E[Dcr] | Equation 10-42 |
| --- | --- |
| μX2=E10-A∙SSFB∙j=1kNj∙Seq,jB | Equation 10-43 |
| E10-A∙SSFB=10-EA+12∙ln10∙VarA∙ESSFB∙vSSF2+1B2-B | Equation 10-44 |
| vX1=vDcr=Var[Dcr]E[Dcr] | Equation 10-45 |
| vX2=10VarA∙ln10∙vSSF2+1B2-1 | Equation 10-46 |

With the distributional assumptions in Table 10-14, an analytic solution for the probability of failure calculations is derived as follows:

| Pft=PX1-X2t∙Θ≤0 =ΦlnEΘ-lnμX1μX2+0.5lnvX12+1-lnvX22+1-lnvΘ2+1lnvX12+1+lnvX22+1+lnvΘ2+1 | Equation 10-47 |
| --- | --- |

With large coefficients of variation for the S/N curve parameter A and/or the stress scaling factor SSF, both vX2 and μX2  can become very large, leading to numerical issues during the estimation of the input required for Equation 10-47. A more robust solution for the probability of failure is achieved with the aid of a small reformulation:

| Pf=ΦlnEΘ-lnμX1+lnμX2+0.5lnvX12+1-VarlnX2-lnvΘ2+1lnvX12+1+VarlnX2+lnvΘ2+1 | Equation 10-48 |
| --- | --- |

Where VarlnX2 and lnμX2 can be estimated as follows:

| VarlnX2=vA2∙EA2∙ln102+B2∙lnvSSF2+1 | Equation 10-49 |
| --- | --- |
| lnμX2=0.5∙EA2∙vA2∙ln102+B2-B∙lnvSSF2+1 -EA∙ln10+B∙lnESSF+lnj=1kNj∙Seq,jB | Equation 10-50 |

The main assumptions underlying the simplified high cycle fatigue model are the following:

- The distributions of all basic variables can be represented by the models listed in Table 10-14.

- The failure mechanism is described by the S/N curve approach combined with the Palmgren-Miner accumulation law. Load history effects are neglected.

- Uncertainties associated with the (equivalent) number of load cycles Nj in each bin / loading block are neglected.

- Uncertainty in the fatigue loading is modelled with the aid of a “global” stress scaling factor, assuming the same uncertainty (and full correlation) for the load cycles in different bins.

- The uncertainty associated with the loading is independent of the uncertainty associated with the S/N curve model.

- Any uncertainty in the S/N curve slope B is neglected, or must be included in the probabilistic model for the intercept A.

- The endurance limit is set to C=0. The conservatism introduced by this assumption can be reduced by setting the number of cycles Nj for all bins with design values Seq,j below the endurance limit to zero. More realistic estimates can only be derived by modelling C as a random variable in Equation 10-39, requiring numerical methods to estimate the probability of failure.

Application of the simplified model to low risk items

The probability of failure for parts with low risk of fracture, e.g. with design stresses below the endurance limit , can in principle also be modelled with the simplified method introduced above. However, in deterministic analysis for low risk items, it is generally not required to determine the (equivalent) number of cycles Nj for each bin, making it difficult to derive the required input even for the simplified method.

To derive a conservative ball park estimate, the sum j=1kNj∙Seq,jB may be replaced by considering only a single load event, i.e. . The stresses Seq should then be taken from the most serious bin (in terms of loads). Also, the number of cycles N should be estimated conservatively, e.g. assuming N=108 or using a rough estimate for the total number of cycles in all load events (including those with lower stresses). If this turns out to be too conservative, the sum must be calculated as in the general S/N curve approach, considering all bins with their respective stresses and (equivalent) number of cycles. A dedicated modelling of the uncertainties associated with C may be required to further improve the analysis

To allow for a first quick assessment, some reference values for the coefficients of variation and some mean values are provided in Table 10-15. The values may be assumed to be applicable or conservative for most practical purposes, especially when considering the additional conservatism that has been introduced by assuming C=0.

Table 10-15: Assumptions for the application of the simplified fatigue model to low risk items.

      - 4.7.2.2 Fatigue crack growth modelling

Crack growth calculations are performed to show that the growth of minor cracks under fatigue loading does not lead to structural failure. The required deterministic methods are available and implemented in specialized Software, such as ESACRACK [BR84] / NASGRO [BR85]. A probabilistic approach based these models is possible and has been described by [BR86] and [BR81], discussing the development and use of a probabilistic version of ESACRACK. Developments for NASA in the field of probabilistic fracture mechanics are discussed in [BR87]. In the present section, only the generic approach to probabilistic fracture analysis is presented together with a short discussion of the required uncertainty quantification. More detailed information can be found in Mattrand [BR81].

In the space industry fatigue crack growth calculations are typically performed using a relationship called the NASGRO equation, which was initially documented by Forman and Mettu [BR88].  It is given by [BR85]:

| dadN=C1-f1-R∆Kn1-ΔKth∆Kp1-KmaxKcq | Equation 10-51 |
| --- | --- |

With the following variable definitions:

- N number of applied fatigue cycles

- a crack length

- K=K(Si,Fi,a) stress intensity factor, derived from the components of stress Si with geometrical correction factors Fi  that are derived specifically for each crack case.

- R=KminKmax stress ratio

- f=f( R,α,Smaxσ0) crack opening function (α denotes a plane stress/strain constraint factor and Smaxσ0 the ratio between the maximum applied stress and the flow stress)

- ∆K stress intensity factor range

- ΔKth threshold stress intensity factor (empirical relation, depending on the crack geometry)

- Kc critical stress intensity factor (empirical relation, depending on the crack geometry)

- C,n,p,q empirical constants of the crack growth model

Typically fatigue crack growth data has three regions, as depicted in Figure 10-8:

- Near-threshold region, characterized by very low crack growth (no growth below the threshold ∆Kth). This region is modelled with the aid of the factor 1-ΔKth∆Kp

- Steady-state where the relation between ∆K and dadN is linear (in log-log scale), which is modelled with the aid of the well-known Paris law dadN=C∆Kn. The factor 1-f1-R has been introduced to account for crack opening effects

- Near instability region, characterized by rapid, unstable crack growth, ultimately leading to fracture at  Kmax=Kc. This region is modelled by the factor  1-KmaxKcq

Figure 10-8: Typical fatigue crack growth curve [BR89].

The crack length a(t) as a function of time (or number of cycles) can be calculated iteratively based on Equation 10-51. There are three possible failure criteria associated with crack growth:

- Crack instability: It occurs when Kmaxat exceeds the relevant fracture toughness Kinstability, which depends on the crack geometry, stress/strain state and the environment.

- Yielding: It occurs if the net section stress σnat exceeds the flow stress σflow, defined as the average between yield and ultimate stress.

- Crack size: As an optional criterion the maximum allowable crack size amax  may be checked.

To perform a reliability analysis, each failure criterion must be evaluated individually. In principle, this evaluation has to be made at every iteration to obtain correct reliability estimates as a function of time. However, in practice it may be sufficient to estimate the probability of failure at discrete time steps, e.g. at the end of each load event or mission phase.

When considering each failure criterion individually, the formulation of limit state functions for a given crack length a(t)  is straight-forward (variable definitions have been given in the definition of the three failure criteria above:

| g1X,at=Kinstability-Kmaxat | Equation 10-52 |
| --- | --- |
| g2X,at=σflow-σnat | Equation 10-53 |
| g3X,at=amax-at | Equation 10-54 |

Different cases must be distinguished to determine the fracture toughness Kinstability:

- For through cracks: Kinstability=Kc

- For part-through cracks: Kinstability=KIe/(a-tip) or Kinstability=KIe/(c-tip)

- For part-through crack tips that have free surfaces: Kinstability=1.1KIe

- For corrosive environments (e.g. Stress Corrosion Cracking): Kinstability=Keac

Where Kc denotes the low constraint toughness value, KIe the apparent surface flaw toughness and Keac corresponds to environmentally assisted crack toughness.

Finally, the probability of failure is estimated by combining all required criteria, e.g. (if only the first two criteria, crack stability and yielding, apply):

| Pft=Prg1at≤0∪g2at≤0 | Equation 10-55 |
| --- | --- |

As an alternative, g1 and g2 may also be combined into a single limit state function by making use of an interaction model, as described e.g. by [BR81]. Note, however, that these models are generally developed for a specific crack geometry, e.g. assuming a through crack in an infinite plate. Hence, the applicability to the crack under consideration shall be assessed when using an interaction model instead of Equation 10-55.

Independent of the approach chosen to combine several failure criteria, reliability is best assessed using Monte Carlo simulations with variance reduction techniques to reduce the number of required simulations. FORM/SORM techniques are not recommended due to the iterative nature and nonlinear behaviour of the crack growth model.

To perform a reliability assessment, the following uncertainties have to be considered and propagated through the model:

- Loading uncertainty

- Initial crack size uncertainty

- Uncertainties associated with material properties

The probabilistic modelling should explicitly account for the main uncertainties related to each of these three modelling areas.

Uncertainties associated with the loading

For a given crack size and geometry, the stress intensity factor  is proportional to the stress. Therefore, the uncertainties in the loading may in principle be modelled using the same approach as for the S/N-Curve approach (Section 10.7.2.1), i.e. based on a global stress scaling factor  applied to each stress intensity factor during the calculation. However, a more rigorous treatment of the loading sequence is required for crack growth analysis than for the S/N curve approach.

One possible reason for load sequence effects is that the crack growth model predicts larger final crack sizes for a load sequence starting with the largest load amplitudes than for another load sequence with the same load spectrum, but starting with the smallest amplitudes. These effects are best considered in a simulation framework, where the whole load process must be simulated for each calculation.

The simulated load sequence may also be used to assess the failure criteria related to crack instability and yielding. Note that the maximum stress intensity factor Kmaxat and the net section stress  σnat in these criteria depends not only on the crack length, but also on the applied maximum stress level , leading to a time variant reliability problem.

A simplified, conservative approach to determine the probability of failure during a certain load event or mission phase is to combine the simulated crack length at the end of the considered time interval with the simulated maximum stress level during the same interval when assessing the limit state functions g1 and  g2.

Uncertainties associated with the initial crack size

Mechanical parts of space products are usually inspected for defects, using one of the available non-destructive evaluation (NDE) techniques. Nevertheless, even if no defect was detected during the inspection, it cannot be assumed that the inspected part is free of cracks. A probabilistic model for the crack size distribution after inspection can be derived based on the following assumptions:

- A crack is always present, even though it might be very small.

- The crack aspect ratio is known, allowing to focus on a single variable a to describe the crack.

- The probability distribution of the crack size before inspection is

- The uncertainty associated with the chosen NDE technique can be described with the aid of a probability of detection curve , see e.g. [BR90].

- Only parts without detected defects are used.

Bayes’ theorem (Chapter 7 Part II) can now be used to determine the probability distribution of the crack size after inspection [BR91]:

| f''Aa=1-PODa∙f'Aa0∞1-PODa∙f'Aa da | Equation 10-56 |
| --- | --- |

In practice, the prior  will often have to be determined based on expert judgment, as data on the crack size distribution before inspection is generally limited. In the case of a part that is inspected (again) after testing, a probabilistic crack growth analysis for the applied test loads can be used to derive f'Aa.

In [BR81],[BR87] it is stated that the POD curve may be interpreted directly as the cumulative distribution function of the crack size after inspection. This approach leads to the same result as Equation 10-56 when assuming a noninformative (positive) uniform prior distribution for the initial crack size.

Further simplifications are possible if only the NDE limit value is used, which is usually associated with a POD of 90% at a 95% confidence interval. For a given distribution type and coefficient of variation, a probabilistic model for the crack size  may then be derived by requiring that the exceedance probability of the limit value must be equal to 10%.

Uncertainties associated with material properties

The NASGRO equation parameters  and are empirical constants whose distribution can in principle be derived from data analysis using experimental data. In practice the problem may be simplified by focusing on those parameters that are relevant for the considered loading and initial crack size, keeping the remaining parameters constant. To give an example, focussing on the Paris law region of the NASGRO equation (region II in Figure 10-8) is acceptable if it is expected that most of the crack growth will occur in this region (i.e. for intermediate initial stress intensity factor ranges and small enough initial crack size). The strongest possible simplification in this case is to model only the multiplicative factor C as a random variable, summarizing all uncertainties associated with the NASGRO equation.

Apart from the NASGRO equation parameters, also the material properties entering the calculations may be treated as random variables which can be assessed based on experimental data. The sensitivity of these variables on the overall result depends on the loading and the initial crack size. To give an example, the uncertainty associated with the threshold stress intensity factor range ∆Kth (e.g. derived from the distribution of ∆K0 in Table 10-16) may be neglected if the loading does not contain a large number of cycles with stress intensity factor ranges below this threshold. At the upper limit, uncertainty in the fracture toughness (KIc or Kinstability) and flow stress σflow (or yield stress  σy) may be expected to play a major role only if the initial crack size is close to the critical size [BR81].

A literature survey presented by Mattrand [BR81] reports coefficients of variation between 5% and 25% for the fracture toughness and yield strength of metallic materials (note that some material specific information may be found e.g. in MMPDS-12 [BR66] or its predecessor, MIL-HDBK-5 [BR67]).  Typical distributional assumptions for the two variables are Weibull or Lognormal distributions. A proposal for the probabilistic modelling of Al 7075 T7351 and Ti 6Al 4V material properties can be found in Table 10-16 (taken from [BR81]).

Table 10-16: Probabilistic modelling of material parameters for Al 7075 T7351 and Ti 6Al 4V [BR81].

### Modelling of failures due to corrosion

Compared to terrestrial applications, the relevance of mechanical failures due to corrosion is generally small, as space systems operate in vacuum and a short exposure to corrosive environments on ground is ensured by specific storage conditions. Special types of corrosion that that are of relevance only for specific parts, e.g. in the propulsion subsystem, are not discussed in the present handbook. The reader is referred to the compendium of limit states for space applications that has been prepared by Kowal [BR65], which includes also a discussion of several types of corrosion.

A corrosion mechanism that is closely related to the crack growth phenomenon is Stress Corrosion Cracking (SCC), which is defined as an environmentally induced formation and growth of cracks in materials exposed to a corrosive environment. The standard strategy to avoid failures due to this phenomenon is based on the selection of suitable materials, making use of a test-based classification by susceptibility to SCC. Quantitative reliability prediction for this failure mechanism is difficult, even though the effect of the environment may in principle be accounted for in crack growth modelling, see Section 10.7.2.2 for details.

### Modelling of failures due to material degradation

Material degradation is a time-dependent process leading to a reduced performance of materials (deterioration of the physical properties) under the influence of environmental actions such as temperature or radiation. For normal mission durations, these effects can usually be controlled during the design, e.g. by appropriate material selection. However, a dedicated modelling may be required for specific applications, or more broadly for wear-out modelling in the context of life time extensions beyond the nominal mission.

Thermal degradation is of relevance mainly for non-metallic materials such as polymers or insulation material. Some basic models have been reviewed in [BR65]. Thermal degradation as an individual failure mechanism is not discussed any further in the present chapter. Note, however, that temperature may have to be considered as a contributor or influencing variable during several other failure mechanisms, e.g. affecting strength and elasticity parameters for common materials.

The following subsection provides a brief discussion of radiation degradation and proposes a generic modelling approach for its modelling in mechanical reliability predictions.

      - 4.7.4.1 Radiation degradation modelling

The spacecraft radiation environment is characterized by the type, fluence, dose rate, energy spectrum and spatial distribution of nuclear radiation at the considered point of interest inside and outside the spacecraft for the duration of the mission. However, the response can be related directly to the absorbed dose. The dose shall be calculated taking both external and internal radiation sources into account.

Radiation affects a variety of material parameters including yield strength and ultimate stress, creep, fatigue, thermal conductivity, hardness, optical properties etc. Metallic materials are rather unaffected by radiation, but degradation needs to be considered for optical materials (e.g. darkening of glass) and for organic materials such as plastics and polymers [BR92]. Due to the complexity of the phenomena, a quantitative estimation of the radiation effects is difficult. Therefore, a simplified assessment of the radiation hardness is performed during the design, taking its basis in an allowable radiation dose, above which the material exhibits significant change in properties.

A similar philosophy can be employed for the reliability assessment. In this case the difference between the radiation dose D and allowable dose DA is used as a limit state function:

| gX=X1-Θ∙X2t=DA-Θ∙Dt | Equation 10-57 |
| --- | --- |

Thus, X1 is defined as the allowable radiation dose DA and X2 as the cumulated dose D determined as a function of time t.

Simplified radiation degradation model

A strongly simplified assessment can be achieved by summarizing all uncertainties associated with the radiation modelling in the model uncertainty variable θ, allowing to consider both D and DA in Equation 10-57 as deterministic variables. The resulting simplified model should be seen mainly as a starting point or placeholder for a more detailed modelling.

The variable definitions are given in Table 10-17 below.

Table 10-17: Assumed probabilistic models for the simplified radiation degradation model.

To account for the uncertainties in choosing the deterministic values for D and DA, the coefficient of variation for the model uncertainty is larger than for the other simplified models presented in Section 10.7.1 to 10.7.2. The model uncertainty CoV can be reduced if the model is improved by considering D and/or DA as random (instead of deterministic) variables with appropriate uncertainty quantification.

With deterministic dose variables, the expected values of X1 and X2 are simply derived as μX1=DA and  μX2=D; the corresponding coefficients of variation are assumed equal to zero. With these assumptions, the analytic solution for the probability of failure is derived as follows:

| Pft=PX1-X2t∙Θ≤0=ΦlnEΘ-lnμX1μX2-0.5∙lnvΘ2+1lnvΘ2+1 | Equation 10-58 |
| --- | --- |

The main assumptions underlying the simplified radiation degradation model are the following:

- Failure occurs if the cumulated radiation dose exceeds the allowable dose limit.

- All uncertainties associated with the cumulated and allowable dose and their determination are summarized in the lognormal distributed model uncertainty variable.

## Examples

The following subsections provide some examples for the application of the methods introduced in Section 10.6 and 10.7.

### Simplified wear modelling: Solid lubricant wear

In this example the simplified method presented in Section 10.7.1.2 is utilized to model the reliability of a bearing. The considered failure mechanism is solid lubricant wear.

Given

A solid lubricated ball bearing is considered. The basic variables, necessary to model the failure mechanism, are modelled as follows (please note that the values indicated in the following table serve as an illustrative example).

| List of variables |  | Unit | Distribution | Expected value | CoV |
| --- | --- | --- | --- | --- | --- |
| Vlimit | Limiting value (worn volume) | [m3] | Lognormal | 6⋅10-8 | 0.2 |
| nP | Number of phases / time intervals | [–] | Deterministic | 1 | – |
| KH,i | Specific wear rate in interval i | [Pa-1=m2/N] | Lognormal |  | 0.6 |
| αi | Ball-cage interaction in interval i | [Nm] | Lognormal | 0.033 | 0.1 |
| revi | Number of revolutions in interval i | [–] | Deterministic | 106 | – |
| Θ | Model uncertainty | [–] | Lognormal | 1.2 | 0.2 |

In this example, it is assumed that only one mission phase is relevant, i.e. nP=1.

Reliability model

To estimate the probability of failure with the simplified methods, the following needs to be known:

- The expected value of the limiting volume,

- the coefficient of variation of the limiting volume,

- the expected value of the volume worn away

- the coefficient of variation of the volume worn away

According to Equation 10-27 the expected value of the limiting volume is:

- EX1=EVlimit=6⋅10-8

According to Equation 10-29 the coefficient of variation of the limiting value is:

vX1=vVlimit=VarVlimitEVlimit=EVlimit⋅0.2EVlimit=0.2

To calculate the coefficient of variation of the volume worn away, the covariance of KH and α is calculated under the assumption of full correlation between KH and α:

CovKH,α=σKH⋅σα=vKH⋅EKH⋅vα⋅Eα==0.6⋅4⋅10-15⋅0.1⋅0.033==7.92⋅10-18

The variance of the product KH⋅α is calculated as:

VarKH⋅α=VarKH⋅Varα+VarKHEα2+VarαEKH2==5.76⋅10-30⋅1.1⋅10-5+5.76⋅10-30⋅0.0332+1.1⋅10-5⋅4⋅10-152=6.51⋅10-33

According to Equation 10-29 the expected value of the volume worn away is:

EX2=EKH⋅α⋅rev=EKH⋅Eα+CovKH,α⋅rev==4⋅10-15⋅0.033+7.92⋅10-18106==7⋅10-8

And the coefficient of variation of the volume worn away is according to Equation 10-30:

vX2=1EX2⋅rev⋅VarKH⋅α=11.4⋅10-10⋅106⋅6.51⋅10-33=0.577

Finally, the probability of failure is calculated according to Equation 10-31:

Pf=PX1-X2⋅Θ≤0=ΦlnEΘ-lnEX1EX2+0.5⋅lnvX12+1-lnvX22+1-lnvΘ2+1lnvX12+1+lnvX22+1+lnvΘ2+1==Φln1.2-ln6⋅10-81.4⋅10-10+0.5⋅ln0.22+1-ln0.5772+1-ln0.22+1ln0.22+1+ln0.5772+1+ln0.22+1==Φ-2.709=0.0034

Where Φ⋅ denotes the cumulative Standard Normal distribution.

### Updating of reliability estimates derived from structural reliability methods

This example is based on the example provided in section 10.8.1, i.e. it considers a bearing that fails from solid lubricant wear. The structural reliability model is established with the simplified methods in Section 10.7.1.2. Upon availability of new data on the reliability of the bearing in question, the model is updated making use of the approach described in Section 10.6.5.

Given

A solid lubricated ball bearing is considered. The basic variables, necessary to establish a reliability model in accordance with the simplified method described in Section 10.7.1.2, are modelled as follows:

| List of variables |  | Unit | Distribution | Expected value | CoV |
| --- | --- | --- | --- | --- | --- |
| Vlimit | Limiting value (worn volume) | [m3] | (Lognormal) | 6⋅10-8 | 0.2 |
| nP | Number of phases / time intervals | [–] | Deterministic | 1 | – |
| KH,i | Specific wear rate in interval i | [Pa-1=m2/N] | Lognormal | 4⋅10-15 | 0.6 |
| αi | Ball-cage interaction in interval i | [Nm] | Lognormal | 0.033 | 0.1 |
| revi | Number of revolutions in interval i | [–] | Deterministic | 106 | – |
| Θ | Model uncertainty | [–] | Lognormal | 1.2 | 0.2 |

From Section 10.7.1.2:

EX1=EVlimit

And, under the assumption of a single mission phase:

EX2=EKH⋅α⋅rev=EKH⋅Eα+CovKH,α⋅rev

Prior model

The probability of failure in function of the number of revolutions rev is modelled. According to section 10.6.5, it can be approximated with the Lognormal distribution.

rev~Lognormalμrev,σrev

The coefficients of variation vX1 and vX2 have already been determined in the example in Section 10.8.1.

σrev=lnvX12+1+lnvX22+1=ln0.22+1+ln0.5772+1=0.571

The location parameter μrev is considered uncertain and is modelled with a Normal distribution (conjugate prior, see Chapter 7 Part II).

μrev~Normalμ',σ'

According to Section 10.6.5, p=EX1EX2 has to be brought to the form p=k⋅1rev. Using the relations in Equation 10-24 and Equation 10-28:

p=EX1EX2=EVlimitEKH⋅α⋅rev.

k=EVlimitEKH⋅α=6⋅10-81.40⋅10-16=4.29⋅108.

The prior hyperparameters for the distribution of μrev are estimated according to Equation 10-13 and Equation 10-14.

μ'=lnk-lnEΘ+0.5⋅lnvΘ2+1-lnvX12+1+lnvX22+1=ln4.29⋅108-ln1.2+0.5⋅ln0.22+1-ln0.22+1+ln0.5772+1=19.84

σ'=lnvΘ2+1=ln0.22+1=0.198

Additional data

Additional data on the reliability of the bearing is given in the table below.

| Specimen i | Revolutions to failurerevi |
| --- | --- |
| 1 | 2.5∙108 |
| 2 | 2.4∙108 |
| 3 | 2.7∙108 |
| 4 | 3.1∙108 |
| 5 | 3.4∙108 |
| 6 | 3.8∙108 |
| 7 | 5.1∙108 |

Updating

With the additional data, the reliability model for the bearing can be updated. The updating is done using the equations for the analytic approach using conjugate priors, given in Chapter 7 (Part II):

σ''=11σ'2+ndataσrev2=110.1982+70.5712=0.145

μ''=σ''2⋅μ'σ'2+i=1ndatalnreviσrev2=0.14619.8401982+1370.5712=19.72

The posterior predictive of rev is also a Lognormal distribution. Its distribution function can be calculated with the help of the analytic formulas provided in Chapter 7.

Results

In Figure 10-9 the prior and posterior distribution of parameter μrev is represented. In Figure 10-10, the prior and posterior probability of failure are shown in function of the number of revolutions rev. As can be seen in the figure, the probability of failure increases with the updating.

Figure 10-9: Prior and posterior distribution for parameter μrev  .

Figure 10-10: Prior and posterior predictive distribution of revolutions to failure.

### Updating of structural reliability methods using right censored data

In contrast to the example in Section 10.8.2, censored data is often available in practice. In this case, the simplified analytic approach for updating is no longer applicable and numerical methods must be used. In this example, MCMC will be used to do the updating for the location parameter. To keep the example simple, the same assumption as in Section 10.8.2 is made that the scale parameter is known and will not be updated. This assumption is not necessary using MCMC for the updating and might be relaxed for a more advanced modelling.

Additional data

Additional, censored, data on the reliability of the bearing is given in the table below.

| Specimen i | Revolutions to failure revi |
| --- | --- |
| 1 | 2.5∙108 |
| 2 | 2.4∙108 |
| 3 | 2.7∙108 |
| 4-20 | ≥5.1∙108 |

Updating

With the additional data, the reliability model for the bearing can be updated. The updating is done using the MCMC method described in Chapter 7 (Part II). The data set is right censored and the likelihood L for a right censored data set is:

| L∝ i=1nfxi¦θ⋅j=1m1-Fxup¦θ | Equation 10-59 |
| --- | --- |

Using the data provided above, the likelihood is:

| L∝f2.5 108¦θ⋅f2.4 108¦θ⋅ f2.7 108¦θ⋅1-F3.4 108¦θ | Equation 10-60 |
| --- | --- |

Using the additional data, the reliability model for the bearing can be updated. The result of the updating results in:

σ''=0.1292

μ''=20.23

Figure 10-11: Markov Chain and posterior density of the parameter.

The posterior predictive of rev is shown in Figure 10-12. It is seen that the consideration of censored data might have a large impact on the updating and has a large potential especial for end of life decision-making.

Figure 10-12: Prior and posterior distribution for parameter μrev  .

Figure 10-13: Prior and posterior predictive distribution of revolutions to failure rev.

## MEC Annex A: Bibliography for the Mechanical chapter

Accounting for proof test data in a reliability-based design optimization framework. 
Venter G and Scotti SJ, AIAA journal 50, Nr. 10 (2012): 2159–2167.

Mechanical System Reliability for Long Life Space Systems
NASA-CR-197215 Final Report, Kowal, M. T. (1994)

Metallic Materials Properties Development and Standardization Handbook
MMPDS-12, Batelle Memorial Institute (2017)

Metallic Materials and Elements for Aerospace Vehicle Structures
MIL-HDBK-5J, US Department of Defence, 2003

FRAMES-2 Data base and Software
http://www.frames-2.com/, accessed 27.04.2020

Reliability analyses and verification of nonelectronic systems
ESA PSS-01-306 Draft 1, 1993

Evaluation of design of tests safety factors – Final report
1710 CA/CC/S, Aerospatiale Cannes, 1987

Ariane 5 User’s Manual, Issue 5 Rev. 2
Arianespace, October 2016

Space engineering - Spacecraft mechanical loads analysis handbook
ECSS-E-HB-32-26A, 2013.

Standard practices for cycle counting in fatigue analysis. 
Annual book of ASTM standards, p.710-718, ASTM 1999

Peak Response to Random Vibration (Rev F)
Irvine, T., 2017, https://vibrationdata.wordpress.com/2016/04/17/peak-response-to-random-with-probabilityof-exceedance/

Die Lebensdauer von Kugellagern. 
Palmgren, A., Zeitschrift des Vereins Deutscher Ingenieure 68 (1924), No. 14, pp. 339–341

Guidelines for space mechanism ball-bearing design assembly and preloading operations
ESTK, TEC-MSM/2013/147/ln/LG

Wear Theory and Mechanisms. Wear Control Handbook
Archard, J. F., American Society of Mechanical Engineers, New York, 1980, pp. 35-80

Dry bearings: a survey of materials and factors affecting their performance. 
Lancaster, J.K., Tribology International 6 (1973), No. 6, pp. 219–251

Life Prediction of Fluid Lubricated Space Bearings: A Case Study
Palladino M., Murer J., Didierjean S. and Gaillard L., in 14th European Space Mechanisms and Tribology Symposium, Constance, Germany; ESMATS, 2011.

Space engineering – Fracture control
ECSS-E-ST-32-01C Rev. 1, 2009

Probabilistic methods applied to fracture control of spaceflight structures. 
Mattrand, C., Technical Report ESTEC TEC-MCS, 2007

JCSS Probabilistic Model Code
Joint Committee on Structural Safety, 2011 
www.jcss-lc.org/jcss-probabilistic-model-code; accessed 27.04.2020

Fatigue Reliability for Offshore Structures. 
Wirsching, P. H., Journal of Structural Engineering 110 (1984), No. 10, pp. 2340–57

ESACRACK Version 4.3.1a, 2012
www.esa.int/TEC/Structures/SEMENHK22EH_0.html; accessed 27.04.2020

NASGRO Version 7.1 Reference Manual, 2014
www.swri.org/consortia/nasgro; accessed 27.04.2020

Extending ESAFATIGUE and ESACRACK for Rational Handling of Uncertainties
Macke M., Henriksen T., Schuëller, G.I. and Klein, M. 1994

Initial Assessment for “Level 1” Probabilistic Fracture Mechanics Analysis Approach. 
Project No. 18.18074.08.808, Southwestern Research Institute (SwRI)

Behavior of surface and corner cracks subjected to tensile and bending loads in Ti-6Al-4V alloy
Forman, R.G. and Mettu, S.R.; Fracture Mechanics: Twenty-second Symposium, Vol. 1, ASTM STP 1131, 1992, pp. 519-546

A Brief Introduction to Fatigue Crack Growth
Lecture Notes, Southwestern Research Institute (SwRI)

Nondestructive Evaluation (NDE) Capabilities Data Book. 
Rummel, W. D.; Matzkanin, G.A.: DTIC Document, 1997

Role of Non-Destructive Evaluation in Time-Dependent Reliability Analysis. 
Zheng, R.; Ellingwood, B.R.: Structural Safety 20 (1998), No.4, pp. 325–339

Designer's guide to radiation effects on materials for use on Jupiter fly-bys and orbiters.
Bouquet, F.L.; Price, W.E.; Newell, D.M.: IEEE Transactions on Nuclear Science 26 (1979), No. 4, pp.4660-4669.

Space engineering – Space environment
ECSS-E-ST-10-04C, 2008

ESA Space Debris User Portal
https://sdup.esoc.esa.int/web/csdtf/home; accessed 27.04.2020